3GPP TSG-RAN WG2 Meeting #121		R2-2301422

27th February – 3rd March 2023

Source:	Qualcomm Incorporated

Title:	Discussion on NR-U Related Enhancements

Agenda Item:	8.13.3

Document for:	Discussion and Decision

# Introduction & Background

Rel-18, SON/MDT WID [1] has the following as one of the objectives,

Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PSCell change report

- Successful Handover Report (e.g. inter-RAT)

- NPN

- RACH report

- fast MCG recovery

- NR-U (MRO and UL MLB)

This paper discusses SON/MDT enhancements for the NR-U (MRO and UL MLB).

# Discussion

2.1 NR-U MRO

- On logging multiple RA Procedures related to consistent LBT failures

In RAN2#119bis-emeeting [2], RAN2 made the following agreements,

Agreements:

1	The UE will log information of multiple RA procedures related to consistent LBT failures. FFS details.

Agreements:

- Introduce a new raPurpose in the RA-Report to indicate that the RA was initiated following “consistent LBT failures” in the SpCell.

Note that there are two scenarios associated with the consistent LBT failures as follows:

- Consistent LBT failures are observed at the UE but do not result in RLF subsequently

- Consistent LBT failures are observed at the UE and RLF happens

Observation 1: There may exist two scenarios with consistent LBT failures as follows:

- Consistent LBT failures are observed at the UE but do not result in RLF subsequently

- Consistent LBT failures are observed at the UE and RLF happens

After consistent LBT failures are observed at a BWP at the UE, the BWP selection for performing the next RACH procedure is selected by the UE. To cover both scenarios of multiple RA procedures related to consistent LBT failures, the simplest solution would be to log the RACH information in the RA report.

Proposal 1: To cover both scenarios of multiple RA procedures related to consistent LBT failures discussed in observation 1, create separate RA entries for each RA procedure related to consistent LBT failures.

For logging of RA-InformationCommon in the RLF report due to the LBT issue, RAN2 can follow the existing UE behavior, i.e., UE logs the RACH-InformationCommon in the RLF-Report only when HoF is observed at the UE.

1>	if connectionFailureType is hof and if the failed handover is an intra-RAT handover:

2>	set the ra-InformationCommon to include the random-access related information as described in clause 5.7.10.5;

Proposal 2: Similar to Rel-16/Rel-17 UE behavior, the RA-InformationCommon is logged in the RLF only for HoF, and not when connectionFailureType is set as RLF and RLF-cause as lbtFailure-r16.

- On logging LBT-related information

In RAN2#119bis-emeeting [2], RAN2 further made the following agreements,

Agreements:

2	RAN2 agree to log kind of “the number of LBT failures” in the RA report.

LBT failure is the failure to access the channel before transmission.

The definition of “the number of LBT failures” should be clarified.

FFS how to log the number of LBT failures in the RA report.

According to TS 38.321 [3], upon the reception of LBT failure indication from the lower layer, UE takes the following action,

- If lbt-FailureRecoveryConfig is configured at the UE, UE performs the Random Access Resource Selection procedure

  - After the selection perform the Random Access Preamble transmission procedure

    - If UE reaches the configured lbt-FailureInstanceMaxCount before reaching preambleTransMax, UE declares the consistent LBT failure, and the RA procedure fails

    - If UE reaches the preambleTransMax before reaching lbt-FailureInstanceMaxCount, UE declares RA procedure failure

- If lbt-FailureRecoveryConfig is not configured at the UE

  - UE increases the PREAMBLE_TRANSMISSION_COUNTER

  - If UE reaches the preambleTransMax, UE declares RA procedure failure

Note that LBT failures do not impact the RACH resources, beam, or CSI-RS selection process during the ongoing RACH resources. Therefore, for the optimization of the RACH procedure, the required information may be if the RA procedure is stopped because of the consistent LBT failure.

Observation 2: There is no correlation between LBT failures and RACH resources, beams, or CSI-RS selected for performing the RACH procedure.

Proposal 3: It is sufficient for UE to indicate that there was consistent LBT failure per RA procedure in RA Report.

Note that channel access procedure, UE senses the channel and determines if the channel is free. The channel access procedure depends on sensed energy at the channel. While the measured signal energy is quantized to form the received signal strength indicator (RSSI), the channel access procedure is not directly correlated to the RSSI value. Furthermore, note that to optimize the transmission opportunity UE may configure the max configured value for the energy detection threshold (EDT).

Observation 3: Actual LBT evaluation is performed based on sensed energy on the channel, i.e., the channel access procedure is not directly correlated to the RSSI value.

Observation 4: In a practical implementation UE may use the max EDT configured at the UE, which is a cell-specific configuration and should be known to the network.

Proposal 4: The RSSI and EDT are not reported in the RA report.

# Conclusion

Observation 1: There may exist two scenarios with consistent LBT failures as follows:

- Consistent LBT failures are observed at the UE but do not result in RLF subsequently

- Consistent LBT failures are observed at the UE and RLF happens

Proposal 1: To cover both scenarios of multiple RA procedures related to consistent LBT failures discussed in observation 1, create separate RA entries for each RA procedure related to consistent LBT failures.

Proposal 2: Similar to Rel-16/Rel-17 UE behavior, the RA-InformationCommon is logged in the RLF only for HoF, and not when connectionFailureType is set as RLF and RLF-cause as lbtFailure-r16.

Observation 2: There is no correlation between LBT failures and RACH resources, beams, or CSI-RS selected for performing the RACH procedure.

Proposal 3: It is sufficient for UE to indicate that there was consistent LBT failure per RA procedure in RA Report.

Observation 3: Actual LBT evaluation is performed based on sensed energy on the channel, i.e., the channel access procedure is not directly correlated to the RSSI value.

Observation 4: In a practical implementation UE may use the max EDT configured at the UE, which is a cell-specific configuration and should be known to the network.

Proposal 4: The RSSI and EDT are not reported in the RA report.

# Reference

- RP-221825, “Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity)”

- RAN2#119bis-emeeting report

- TS 38.321, “Medium Access Control (MAC) protocol specification”