3GPP TSG-RAN WG2#121	R2-2301423

Athens, Greece, 27th February - 3rd March, 2023

Agenda Item:	8.5.4

Source: 	MediaTek Inc.

Title:  	Capacity enhancement for XR

Document for:	Discussion, decision

- Introduction

RP#98-e had agreed new WID [1] on XR enhancement for NR related to capacity as below:

- Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

- Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1);

- BSR enhancements including at least new BS Table(s); (RAN2);

- Delay reporting of buffered data in uplink; (RAN2);

- Provision of XR traffic assistance information for DL and UL (e.g. periodicity); (RAN2);

- Discard operation of PDU Sets (RAN2);

This paper will discuss how to enhance XR capacity from RAN2 point of view.

# 2	Discussion

2.1	CG enhancement

For objective 1 and 2, the CG enhancemet is led by RAN1. The general procedure is that gNB configures UE with multiple CG PUSCH transmission occasions via RRC message, when UE has UL data, UE calulate how much occasions is needed, and indicate the unused occasions to gNB via UCI. The impact to RAN2 would be TS 38.321 and TS 38.331. For example: how to define HARQ process ID to support CG with multiple occasions in 38.321 is an issue. How to signal the new RRC message for multiple CG PUSCH occasions in 38.331 is an issue as well, those impacts to RAN2 can wait for more progresses from RAN1.

Proposal 1: RAN2 to agree general procedure for CG enhancement:

- gNB configures UE with multiple CG PUSCH transmission occasions via RRC message.

- when UE has UL data, UE calulate how much occasions is needed, and indicate the unused occasions to gNB via UCI.

- Other details leave RAN1 to decide.

2.2	BSR enhancement

For objective 3 and 4, it is purely RAN2 scope, we first provide current XR traffic model from TR 38.838 [2], then based on the XR traffic model, we share our views about BSR and reporting enhancement in this section.

2.2.1	XR Traffic model and profile

In Rel-17 Study Item “Study on XR Evaluations for NR”, many of the XR and CG use cases are studied and characterised by quasi-periodic traffic (with possible jitter) with high data rate in DL (i.e., video stream) combined with the frequent UL (i.e., pose/control update) and/or UL video stream. Both DL and UL traffic are also characterized by relatively strict packet delay budget (PDB), the results are captured in TR 38.838 [2] and summarized in Table 1.

Table 1: XR Traffic model [2]

| Traffic Direction | DL |  |  |  | UL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Application | Cloud Gaming |  | AR/VR |  | AR | Pose/Control |
| Data rate (R) (Mbps) | 8 | 30 | 30 | 45 | 10 | 0.2 |
| Packet size distribution | Truncated Gaussian |  |  |  |  | Fixed |  |
| Packet size (Min, Ave, Max) (Bytes) | (8334, 16667, 25001) | (31250, 62500, 93750) |  | (46875, 93750, 140625) | (10417, 20834, 31251) | 100 |
| FPS (1/Periodicty) | 60 |  |  |  |  | 250 |  |
| Jitter value distribution | Truncated Gaussian |  |  |  |  | Fixed |  |
| Jitter value (Min, Ave, Max) (ms) | (-4, 0, 4) |  |  |  |  | 0 |  |
| Packet delay budget (PDB) (ms) | 15 |  | 10 |  | 30 | 10 |
| Min. required data rate (Mbps) | 8.89 | 33.33 | 50 | 75 | 5.56 | 0.08 |
| Reliability requirement (%) | 99 |  |  |  |  |  |  |

Observation 1: In XR Traffic model, different applications have different minimal packet size and max packet size.

2.2.2	BSR enhancement

During SI phase, three enhanced directions for BSR are identified 1) Enhanced BSR table specific for XR packet, 2) Enhanced BSR format with delay information 3) New trigger condition for BSR associated to XR packet.

2.2.2.1	BSR Table

In the current spec, buffer sizes reported in BSR table are coded by an exponential function, i.e.

Bk = Bmin (1+p) k, where p = (Bmax / Bmin)1 / (N-1) – 1,

where Bmin is the minimum buffer size and Bmax is the maximum buffer size that can be reported by UE. With the knowledge of XR traffic model, we know that the minimum packet size is not small as other traffic. Therefore, a different minimum packet size can be configured for XR-specfic traffic, which belongs to certain logical channel group (LCG). In order to differentiate the new BSR table and legacy BSR table, the configuration of minimum packet size could be optional. If it is configured, that means new BSR table is used. If not, that means legacy BSR table is used.

Proposal 2: NW uses RRC message to configure minimum (Bmin) XR packet size for the LCG.

- If Bmin is not configured, use legacy BSR table.

- If configured, use new BSR table.

In order to reduce the quantization error, we suggest to use liner BSR table for XR services, not exponential BSR table. The step size configuration is the same as configuration of minimum packet size of the LCG.

Bk = Bmin + (k-1) * (step size),

Proposal 3: The new BSR table is linear, i.e., equally step size for each index k.

Proposal 4: NW uses RRC message to configure step size of linear BSR table for the LCG.

- If step size is not configured, use legacy BSR table.

- If configured, use new linear BSR table.

2.2.2.2	Delay reporting

The packet delay budget is associated with buffer data, however, the buffer data might have different remaining packet delay budget, we suggest to choose the minimum value as the reported information. The reported information is different among different LCGs. We also suggest that the reported information should associated with BSR. The meaning of each reporting index could follow similar mechanism as BSR, i.e., the delay information could be a range.

Proposal 5: UE chooses minimum delay value as reported information.

Proposal 6: UE reported delay information with BSR.

Proposal 7: The reported delay information could be a range.

2.2.2.3	BSR trigger condition

A BSR can be triggered by different events, it can be triggered when new higher priority data and expiry of retxBSR-Timer, which is called “Regular BSR”. It can also be triggered by expiry of periodicBSR-Timer, which is called Periodic BSR. The last one is called “Padding BSR”, which can be piggyback when UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC CE plus its subheader.

When XR packet comes to buffer, we assume it has higher priority than other traffic, then a “Regular BSR” can be be triggered. However, as discussed in 2.2.1, for XR packet with tight latency requirement, it might not have too much chance to send another BSR. To increase the chance to send BSR, new trigger condtion for BSR is needed. We assume the XR packet will be segmented into different TBs for transmission. The new trigger condition could allow that for each TB transmission with XR packet, BSR is always included. The concept is more like padding BSR, which is piggyback at each TB transmission with XR packet.

Proposal 8: RAN2 to study new trigger condition for BSR for XR services.

# 3	Conclusion

We have the following observations and proposals:

Observation 1: In XR Traffic model, different applications have different minimal packet size and max packet size.

Proposal 1: RAN2 to agree general procedure for CG enhancement:

- gNB configures UE with multiple CG PUSCH transmission occasions via RRC message.

- when UE has UL data, UE calulate how much occasions is needed, and indicate the unused occasions to gNB via UCI.

- Other details leave RAN1 to decide.

Proposal 2: NW uses RRC message to configure minimum (Bmin) XR packet size for the LCG.

- If Bmin is not configured, use legacy BSR table.

- If configured, use new BSR table.

Proposal 3: The new BSR table is linear, i.e., equally step size for each index k.

Proposal 4: NW uses RRC message to configure step size of linear BSR table for the LCG.

- If step size is not configured, use legacy BSR table.

- If configured, use new linear BSR table.

Proposal 5: UE chooses minimum delay value as reported information.

Proposal 6: UE reported delay information with BSR.

Proposal 7: The reported delay information could be a range.

Proposal 8: RAN2 to study new trigger condition for BSR for XR services.

# 4	References

[1]	RP-223502, New WID on XR Enhancements for NR, Nokia, Qualcomm.

[2]	TR 38.838