3GPP TSG-RAN WG2 Meeting #121		R2-2301424

27th February – 3rd March 2023

Source:	Qualcomm Incorporated

Title:	On SgNB RACH report for MR-DC scenario

Agenda Item:	8.13.6

Document for:	Discussion and Decision

# Introduction & Background

Rel-18, SON/MDT WID [1] has the following as one of the objectives,

Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PSCell change report

- Successful Handover Report (e.g., inter-RAT)

- NPN

- RACH report

- fast MCG recovery

- NR-U (MRO and UL MLB)

This paper discusses SON/MDT enhancements for the SON/MDT enhancements for RACH report, where we will discuss the SgNB RACH report for the MR-DC scenario.

# Discussion

In RAN3 LS [2], RAN3 mentions the following (while other content is about RACH report retrieval):

- RAN3 has supported SN RA Report for EN-DC, (NG)EN-DC, and NR-DC scenarios in Rel-17. No further work will be triggered in RAN3.

- RAN3 believes that if RAN2 decides to support SN RA Report for EN-DC and (NG)EN-DC, the UE should report the PSCell identity outside the RACH report to help an eNB forward the report to the correct node without the need to decode the RACH report.

In our understanding, some part of RAN3 LS, “UE should report the PSCell identity outside the RACH report to help an eNB forward the report to the correct node without the need to decode the RACH report.”, is not in line with RAN2 Rel-16 agreement for NR RACH report. In the RAN2#108 meeting [3], RAN2 agreed that the network cannot retrieve only parts of the stored list of the RACH report. This implies that during the reporting, the UE sends the entire list to the network and no additional processing for individual entry is required at the UE. Therefore, the UE should report NR RA-ReportList in the container to the LTE MN.

Note that UE is not required to process each entry individually. Therefore, UE is not required to create separate containers for each entry in the RA-ReportList. In such a case, in the EN-DC, the MN cannot open the container and send individual entries to cells. Therefore, there is no benefit of indicating cell ID outside the container.

Observation 1: In the RAN2#108 meeting [3], RAN2 agreed that the network cannot retrieve only parts of the stored list of the RACH report, i.e. no processing for individual entry is required at the UE.

From section 9.1.3.25 in TS 38.423 [4], we can observe that the entire RA-ReportList is included in the RACH report container. Therefore, RAN3 does not require UE to process each entry of the RA-ReportList and put them in separate containers.

#### 9.1.3.25	ACCESS AND MOBILITY INDICATION

This message is sent by NG-RAN node1 to transfer access and mobility related information to NG-RAN node2.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| RACH Report List |  | 0..1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>RACH Report Container | O |  | OCTET STRING | RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| >>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |
| Successful HO Report List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | – |  |
| >>Successful HO Report Container | O |  | OCTET STRING | SuccessHO-Report-r17 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | – |  |

Observation 2: From section 9.1.3.25 in TS 38.423 [4], we can observe that the entire RA-ReportList is included in the RACH report container. Therefore, RAN3 does not require UE to process each entry of the RA-ReportList and put them in separate containers.

Proposal 1: For SgNB RACH reporting for EN-DC and (NG)EN-DC, UE is not required to create separate containers for each entry. UE reports NR RA-ReportList in a container to the LTE MN.

In our understanding, the LTE MN cannot open the RA-ReportList and obtain the individual entries, therefore, we believe there is no need for UE to include the cell ID list outside the container.

Observation 3: LTE MN cannot open the RA-ReportList and obtain the individual entries, as the container contains the RACH report list instead of individual RACH entries.

Proposal 2: For SgNB RACH reporting for EN-DC and (NG)EN-DC, the UE is not required to put cell IDs of each RACH entry outside the container, as LTE MN cannot forward the individual entries to the corresponding NR cell.

Whether UE needs to include any cell ID outside the container depends on whether cross-RAT reporting in the standalone scenario is allowed or not. For example, whether an LTE cell can request NR RACH report when UE is in standalone. Similarly, whether NR cell request LTE RACH report, when UE is in standalone. Therefore, we believe that RAN2 needs to discuss when can sgNB RACH report be sent to the network. In our understanding there exist mainly two scenarios,

  - NR RA-ReportList is sent to the LTE cell only when UE is in EN-DC or (NG)EN-DC.

  - NR RA-ReportList is sent to the LTE cell together with the LTE RACH report (if available), whenever the LTE cell requests the RACH report.

Observation 4: Whether UE needs to include any cell ID outside the container depends on whether cross-RAT reporting of the RACH report in the standalone scenario is allowed.

Proposal 3: RAN2 should discuss which scenario for the sgNB RACH report is valid

In our understanding there exist mainly two scenarios,

- NR RA-ReportList is sent to the LTE cell only when UE is in EN-DC or (NG)EN-DC.

- NR RA-ReportList is sent to the LTE cell together with the LTE RACH report (if available), whenever the LTE cell requests the RACH report.

# Conclusion

Observation 1: In the RAN2#108 meeting [3], RAN2 agreed that the network cannot retrieve only parts of the stored list of the RACH report, i.e. no processing for individual entry is required at the UE.

Observation 2: From section 9.1.3.25 in TS 38.423 [4], we can observe that the entire RA-ReportList is included in the RACH report container. Therefore, RAN3 does not require UE to process each entry of the RA-ReportList and put them in separate containers.

Proposal 1: For SgNB RACH reporting for EN-DC and (NG)EN-DC, UE is not required to create separate containers for each entry. UE reports NR RA-ReportList in a container to the LTE MN.

Observation 3: LTE MN cannot open the RA-ReportList and obtain the individual entries, as the container contains the RACH report list instead of individual RACH entries.

Proposal 2: For SgNB RACH reporting for EN-DC and (NG)EN-DC, the UE is not required to put cell IDs of each RACH entry outside the container, as LTE MN cannot forward the individual entries to the corresponding NR cell.

Observation 4: Whether UE needs to include any cell ID outside the container depends on whether cross-RAT reporting of the RACH report in the standalone scenario is allowed.

Proposal 3: RAN2 should discuss which scenario for the sgNB RACH report is valid

In our understanding there exist mainly two scenarios,

    - NR RA-ReportList is sent to the LTE cell only when UE is in EN-DC or (NG)EN-DC.

    - NR RA-ReportList is sent to the LTE cell together with the LTE RACH report (if available), whenever the LTE cell requests the RACH report.

# Reference

[1] RP-221825, “Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity)”

[2] R3-226053, Reply LS on SN RACH report status in R17, Source: RAN3, To: RAN2

[3] RAN2#108 meeting

[4] TS 38.423