3GPP TSG-RAN WG2 #121	R2-2301438

Athens, Greece, 27 February – March 3, 2023

Agenda Item:	8.1.3

Source:	Ericsson

Title:	Discussion on capabilities and remaining issues for NCR

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we focus on providing our view regarding the features that an NRC should support.

In addition, we address the following open issue:

- NCR-MT support of SRB2 configuration without DRB,

- support of RLM, BFD, BFR.

# 2	Discussion

## 2.1	Open issues

As seen in TS 38.331, for NR UEs a configuration with SRB2 without DRB is not supported. However, this is not the case for an IAB-MT. 

On this matter, during RAN2#119bis-e, it was agreed that DRBs are optional, while in RAN2#120, RAN2 agreed that SRB2 is a mandatory feature for NCR-MT. Hence, in order not to limit possible uses of SRB2 for NAS messages, a similar approach to that in IAB-MT can also be adopted for the NCR-MT.

NCR-MT can optionally indicate its capability to support SRB2 configuration without DRB.

Regarding the support of RLM, BFD and BFR. We would like to start by highlighting that since FR2 appears to be key for NCRs, the support of BFD and BFR appear as beneficial for the management of NCR. For which we propose the following.

For FR2, NCR-MT supports BFD and BFR.

As for RLM, we acknowledge that if there is support for RLF, then it is required to support RLM as well.

Support for RLF comes together with support for RLM.

## 2.2	What capabilities the NCR-MT should support

One of the objectives of the work item is also to study which capabilities and RRM functions need to be supported by the NCR-MT. This is because the NCR should be a relatively inexpensive network node and thus the NCR-MT may not need to support the whole set of features that are typical for a normal UE. On top of this, one needs to keep in mind that the NCR-MT is just a function entity that communicates with a gNB via a Control link (C-link) to enable exchange of control information (e.g., side control information at least for the control of NCR-Fwd).

The NCR-MT is defined as a function entity to communicate with a gNB via a Control link (C-link) to enable exchange of control information (e.g., side control information at least for the control of NCR-Fwd).

Another aspect that needs to be clarified and kept in mind when studying which capabilities an NCR node should support, is that the NCR has to be a network node fundamentally cheaper and simpler than an IAB node and thus what applies to an IAB (or an IAB-MT in particular) does not need to necessarily apply also for NCR-MT.

The NCR has to be a network node fundamentally cheaper and simpler than an IAB node and thus what applies to an IAB (or an IAB-MT in particular) does not need to necessarily apply also for NCR-MT

If we take inspiration by the work that has been done during the Rel-16 IAB work item, one option would be to check in the TR 38.822 [3] which features may be needed to be supported by the NCR. A possible example of what feature may be needed for the NCR is shown in Table 1 where we have the following:

- Highlighted in green are the features that shall be supported by an NCR (without capability signalling)

- Highlighted in yellow are the features that could be supported by an NCR (optional with or without capability signalling) but that RAN2 needs to discuss.

- Highlighted in grey are the features that shall not be supported by an NCR.

Table 1. List of features that an NCR shall, may, and not support.

| Features | Index | Feature group | Components | Mandatory/Optional | Ericsson comment |
| --- | --- | --- | --- | --- | --- |
| 0. General (including supported bearer types) | 0-1 | Access stratum release | Access stratum release | Optional with capability signalling and candidate value set is {Rel-15, spare7, … , spare1} | Can support. Not really a “feature” |
|  | 0-3 | DRB | 1) Maximum number of DRBs | 1) Agreed to be optional, values {1, 16}. If absent, the NCR-MT does not support DRB. |  |
|  | 0-7 | PCell operation | 1) PCell operation on FR2 | Mandatory with capability signalling | Primary focus for the NCR should be FR2 and thus it makes sense to have it |
| 1. PDCP | 1-0 | Basic PDCP procedures | 1) (de)Ciphering on DRB/SRB 2) Integrity protection on SRB 3) Timer based SDU discard 4) Re-ordering and in-order delivery 5) Status reporting 6) Duplicate discarding 7) 18bits SN | Mandatory without capability signalling | The yellow highlighted feature are not critical. |
|  | 1-5 | Short SN | Short SN | Mandatory with capability signalling | There is some value in having the short SN, even if maybe this is not a critical feature. |
|  | 1-7 | DRB IP data rate | 1) DRB IP data rate in DL 2) DRB IP data rate in UL | Optional capability is signalled by NAS signalling defined in 24.501 | 1 DRB may be needed for OAM. |
| 2. RLC | 2-0 | Basic RLC procedures | 1) RLC TM 2) RLC AM with 18bits SN* 3) SDU discard | Mandatory without capability signalling | At least RLC AM and RLC TM may be needed for some RRC procedure. About the discarding of SDUs, this maybe can be discussed. |
|  | 2-1 | RLC AM with short SN | RLC AM with short SN | Mandatory with capability signalling |  |
|  | 2-2 | RLC UM with short SN | RLC UM with short SN | Mandatory with capability signalling |  |
|  | 2-4 | NR RLC SN size for SRB | NR RLC SN size for SRB | RAN2 decided only short RLC SN is used for SRB. | RAN2 needs to discuss what value are suitable for NCR. |
| 3. MAC | 3-0 | Basic MAC procedures | 1) RA procedure on PCell or PSCell (in case of EN-DC) 2) UE initiated RA procedure (including for beam recovery purpose) 3) NW initiated RA procedure (i.e. based on PDCCH) 4) Support of ssb-Threshold and association between preamble/PRACH occasion and SSB 5) Preamble grouping 6) UL single TA maintenance 7) HARQ operation for DL and UL 8) LCH prioritization 9) Prioritized bit rate 10) Multiplexing 11) SR with single SR configuration 12) BSR 13) PHR 14) 8bits and 16bits L field | Mandatory without capability signallling |  |
|  | 3-3 | DRX | 1) DRX with long DRX cycle 2) DRX with short DRX cycle | Mandatory with capability signalling | There is some value in have a DRX configured at the NCR-MT, also since there cases where the NCR-Fwd part is OFF. |
| 5. SDAP | 5-1 | QoS | 1) Flow-based QoS 2) Multiple flows to 1 DRB mapping 3) AS reflective QoS | 1), 2) Mandatory without capability signalling 3) Optional with capability signalling | RAN2 agreed to optionally support DRBs. However, these features may not been needed. |
|  | 5-2 | HD format | 1) DL SDAP HD 2) UL SDAP HD 3) SDAP End-marker | 1) Conditional mandatory if either NAS reflective QoS or AS reflective QoS is supported. No capability signalling is needed. 2), 3) Mandatory without capability signalling | RAN2 agreed to optionally support DRBs. However, these features may not been needed |
| 9. RRC | 9-1 | RRC buffer size | Maximum overall RRC configuration size | 45 Kbytes | One approach is to reuse the existing value. |
|  | 9-2 | RRC processing time | 1) RRC connection establishment 2) RRC connection resume without SCell addition/release and SCG establishment/modification/release 3) RRC connection reconfiguration without SCell addition/release and SCG establishment/modification/release 4) RRC connection re-establishment. 5) RRC connection reconfiguration with sync procedure 6) RRC connection reconfiguration with SCell addition/release or SCG establishment/modification/release 7) RRC connection resume 8) Initial security activation 9) Counter check 10) UE capability transfer | 1) to 3) 10ms 4) 10ms 5): 10ms + additional delay (cell search time and synchronization) defined in TS 38.133 6) and 7) 16ms 7) 10 or 6ms (See details in 12, TS 38.331) 8) and 9) 5ms 10) 80ms | One approach is to use the same numbers as for normal UEs. But some of these features are of course not supported (e.g. CA), but will be discussed in other rows. About 5) the only case where this may be needed without mobility could be due to a security key change. |

RAN2 to discuss which features are supported by the NCR-MT using the list shown in Table 1 as baseline.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Support for RLF comes together with support for RLM.

Observation 2	The NCR-MT is defined as a function entity to communicate with a gNB via a Control link (C-link) to enable exchange of control information (e.g., side control information at least for the control of NCR-Fwd).

Observation 3	The NCR has to be a network node fundamentally cheaper and simpler than an IAB node and thus what applies to an IAB (or an IAB-MT in particular) does not need to necessarily apply also for NCR-MT

Based on the discussion in the previous sections we propose the following:

Proposal 1	NCR-MT can optionally indicate its capability to support SRB2 configuration without DRB.

Proposal 2	For FR2, NCR-MT supports BFD and BFR.

Proposal 3	RAN2 to discuss which features are supported by the NCR-MT using the list shown in Table 1 as baseline.

# 4	References

RP-222673, “New WID on NR network-controlled repeaters”, ZTE, Sanechips, RAN#97-e, September 2022

3GPP TR 38.867, “Study on NR Network-controlled Repeaters”, RAN1 (RAN2, RAN3), v18.0.0, September 2022

3GPP TR 38.822, “NR; User Equipment (UE) feature list”, RAN2, v16.3.0, April 2022

R2-2213009, “Report from NC Repeater breakout session”, Session Chair (Apple), RAN2#120, November 2022, Toulouse, France

RP-223505, “Revised WID on NR network-controlled repeaters”, RAN#98-e, December 2022