3GPP TSG-RAN WG2 #121	R2-2301439

Athens, Greece, 27th February – 3rd March 2023

Agenda Item:	8.1.3

Source:	Ericsson

Title:	Support of IDLE with RRCRelease for NCR

Document for:	Discussion, Decision

# 1	Introduction

In the last RAN2#120 meeting, the following agreements were taken about the relationship of the RRC state of an NCR-MT and the state (ON/OFF) of the NCR-Fwd:

On NCR-Fwd ON/OFF:
- When NCR-MT is in RRC_CONNECTED mode, the NCR-Fwd can be ON or OFF following the side control information received from the gNB. 
- After NCR-MT enters RRC_INACTIVE mode, the NCR-Fwd can be ON or OFF following the last configuration received from the gNB.
- Release to RRC-IDLE is FFS.

This contribution is to address the remaining FFS and to clarify how an NCR-MT that is sent to RRC_IDLE can be paged to transition into RRC_CONNECTED.

# 2	Discussion

In legacy UE-Network operation, when the network decides to send a UE in RRC_IDLE, this UE will transition back to RRC_CONNECTED mainly for two reasons: i) the UE has UL data to send to the network and thus will initiate an RRC setup procedure, or ii) the UE is paged by the network (e.g., because the network has DL data to send to the UE) and this will trigger in turn the UE to initiate an RRC setup procedure.

Nevertheless, the NCR-MT has no traffic itself (since it only supports control plane traffic and it simply relays data from/to the gNB to/from the UE). Since for a UE in RRC_IDLE only CN-initiated paging is supported, this means that the AMF will never initiate a paging procedure for NCR-MT in RRC_IDLE.

Since the NCR does not have its own traffic, a CN-initiate paging for NCR-MT in RRC_IDLE will never happen.

Given this, there are different solutions to overcome this issue. In particular the following solutions can be applied:

- When sending the NCR-MT in RRC_IDLE, a new timer is included in the RRCRelease message so that the NCR-MT is aware of when it should initiate the RRC setup procedure. Eventually, the timer may be also broadcasted in system information for those cases in which the NCR-MT will go to RRC_IDLE without any explicit signalling from the network.

- The NG-RAN node that has sent the NCR-MT in RRC_IDLE sends a request to the core network (e.g., AMF) to initiate a CN-initiate paging procedure with the NCR-MT.

- The NCR-MT is pre-configured (e.g., by OAM) to know when it needs to be in RRC_CONNECTED.

According to the listed options, Option 2 requires a new signalling over the NG interface (which is not in the scope of the WID). Also, the NG-RAN that requests the paging to the core network is the last node to have the NCR-MT in RRC_CONNECTED, but it may not be the same node later. This brings further complexity to this solution.

Introducing a new signalling over the NG interface to request the core network to page the NCR-MT is not desirable, as it is complex and needs the involvement of SA and CT (which are not part of the WID).

Regarding Option 3, instead, this solution provides the less flexibility since OAM is not done to handle dynamic changes to the configuration. For this release this solution may be fine since the NCR is considered anyway to be a static node, but it would be difficult to scale this solution in scenarios where the NCR(-MT) is moving. Also, at the moment RAN2 agreed that OAM traffic it goes via DRB(s) to the NCR-MT and when the NCR-MT is in RRC_IDLE there is no DRB active. Therefore, this solution may require further standardization efforts to be adopted. Further, the feasibility of this solution would require input from RAN3 and SA5.

The NCR-MT may be pre-configured to know when it needs to be in RRC_CONNECTED (e.g., via OAM) but this solution implies no flexibility and is also difficult to scale in case the NCR-MT is mobile.

Given this, Option 1 seems to be the most suitable for the scope of this WI. Indeed, it just requires introducing a timer that, upon its expiry, indicates to the NCR-MT that it should initiate the RRC setup procedure. However, the same timer may also need to be introduced in system information, as the NCR-MT may go to RRC_IDLE for other reason. However, this can be left FFS as it is not a critical aspect for the time being.

We want to highlight though, that if RAN2 would decide not to support the release of NCRs to RRC IDLE (and instead, only rely on RRC INACTIVE) the problem goes away altogether, which we believe would be the most suitable approach.

Release of NCRs to RRC IDLE is not supported.

If Proposal 1 is not agreed, we instead propose the following, which seems most straightforward and feasible to do given the short time remaining for this work item:

A new timer is introduced in the RRCRelease message to indicate to the NCR-MT when it should transition to RRC_CONNECTED. FFS on whether the same timer needs to be introduced in system information.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Since the NCR does not have its own traffic, a CN-initiate paging for NCR-MT in RRC_IDLE will never happen.

Observation 2	Introducing a new signalling over the NG interface to request the core network to page the NCR-MT is not desirable, as it is complex and needs the involvement of SA and CT (which are not part of the WID).

Observation 3	The NCR-MT may be pre-configured to know when it needs to be in RRC_CONNECTED (e.g., via OAM) but this solution implies no flexibility and is also difficult to scale in case the NCR-MT is mobile.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Release of NCRs to RRC IDLE is not supported.

Proposal 2	A new timer is introduced in the RRCRelease message to indicate to the NCR-MT when it should transition to RRC_CONNECTED. FFS on whether the same timer needs to be introduced in system information.

# 4	References

RP-222673, “New WID on NR network-controlled repeaters”, ZTE, Sanechips, RAN#97-e, September 2022

R2-2213009, “Report from NC Repeater breakout session”, Session Chair (Apple), RAN2#120, November 2022, Toulouse, France

RP-223505, “Revised WID on NR network-controlled repeaters”, RAN#98-e, December 2022