3GPP TSG-RAN WG2 #121	R2-2301442

Athens, Greece, 27 February – March 3, 2023

Agenda Item:	8.16.3

Source:	Ericsson

Title:	Use cases aspect for AIML for NR air interface

Document for:	Discussion, Decision

# 1	Introduction

This is the third meeting on which RAN2 discusses the “AI/ML for NR Air Interface” SI (see the approved SID in RP-213599, and the revised version in RP-221348). Previous RAN2 meetings have focused on general and organizational aspects.

And while we believe that this meeting should also prioritize the general aspects related discussions, in this document we provide some observations regarding the use of RRC to collect data for model training for the Beam Management and CSI use cases.

# 2	Discussion

## 2.1	Observation regarding RAN1 agreements

RAN1 agreements concerning RAN2 have been included in Annex B of this document. From these, one could argue for, is that the exercise of taking current RAN1 agreements and transforming them into RAN2-related proposals is not straightforward. Especially given the fact that we see no point in studying the same things in RAN1 and RAN2 simultaneously.

Hence, input, or requirements from RAN1 will aid RAN2 discussions on specific solutions for the different topics within the scope of this SI. We take this into consideration for the following section.

## 2.2	Data collection aspects

Data collection may be needed for model training, model inference, model monitoring, model selection, model update, etc. However, it is important to highlight that latency requirements for NWs to obtain data can vary depending on the use that will be made of this information.

When focusing on model training, the latency requirement could potentially be relaxed.

The above could, for example, affect when having to decide whether a solution is suitable or no. As a straightforward option, RAN2 could analyse an RRC-based alternative for this purpose.

To proceed and based on the long email discussion on data collection (see outcome in R2-2301440) RAN2 could consider the following metrics when analysing different data collection frameworks: a) the content of the data, b) the data size, c) latency and periodicity, d) signalling, entities involved, and configuration aspects.

While waiting for RAN1’s requirements and understanding that right now it is not possible to reach any type of agreement on this matter, we have provided an “a priori” list of metrics for CSI and BM use cases in Annex A (note: these are the same as the ones discussed during the email discussion).

In this regard, for these use cases, it could be beneficial to have UEs logging their measurements together with the meta information for several measurement occasions. Especially when the latency requirements are low.

From the lists, one can then observe the following:

For the CSI use case, when collecting data from the UE to the NW for model training a measurement occasion could consists of a single CSI-RS resource.

For the CSI use case, when collecting data from the UE to the NW for model training, a UE could log all measurements performed on CSI-RS in addition to “assistance information” (e.g., time stamps, cell ID, etc.). The UE can then trigger a report of these accumulated data to the NW.

For NW-sided beam prediction, model training requires UE downlink L1 measurements.

For NW-sided beam prediction, a UE could log its radio/non-radio measurements together with the meta information (e.g., time stamps, cell ID, and/or UE location) for multiple measurement occasions. The UE can then trigger a report of these accumulated data to the NW.

On the above, RRC messages can also effectively be used to report large amounts of measurement data to the NW with reduced signalling overhead and radio resource consumption.

Additionally, and when considering a general data collection framework for the different use cases. We consider that an RRC-based approach could eventually fulfil many of the criteria.

RRC allows the NW to configure a UE to log and report data.

An RRC-based data collection solution enables the NW to efficiently collect large amounts of measurement data.

For the beam management and CSI use cases, RAN2 to consider this document’s observations when analysing the feasibility of having an RRC-based data collection solution for model training.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	For the CSI use case, when collecting data from the UE to the NW for model training a measurement occasion could consists of a single CSI-RS resource.

Observation 2	For the CSI use case, when collecting data from the UE to the NW for model training, a UE could log all measurements performed on CSI-RS in addition to “assistance information” (e.g., time stamps, cell ID, etc.). The UE can then trigger a report of these accumulated data to the NW.

Observation 3	For NW-sided beam prediction, model training requires UE downlink L1 measurements.

Observation 4	For NW-sided beam prediction, a UE could log its radio/non-radio measurements together with the meta information (e.g., time stamps, cell ID, and/or UE location) for multiple measurement occasions. The UE can then trigger a report of these accumulated data to the NW.

Observation 5	RRC allows the NW to configure a UE to log and report data.

Observation 6	An RRC-based data collection solution enables the NW to efficiently collect large amounts of measurement data.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the beam management and CSI use cases, RAN2 to consider this document’s observations when analysing the feasibility of having an RRC-based data collection solution for model training.

# 4	References

RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec 2021

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#96, Budapest, Hungary, June 2022

R1-2205695, “Session notes for 9.2 (Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface)	“, Ad-hoc Chair (CMCC), RAN1#109-e, May 2022

R1-2208145, “Session notes for 9.2 (Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface)”, Ad-hoc Chair (CMCC), RAN1#110, Toulouse, August 2022

R2-2210677, “RAN2 Work Plan for Rel-18 SI on AI/ML for NR air interface”, Ericsson, Qualcomm Inc., RAN2#119bis-e, Oct. 2022

R1-2210690, “Session notes for 9.2 (Study on AI/ ML for NR air interface)”, CMCC, RAN1#110bis-e, Oct. 2022

R1-2212845, “Session notes for 9.2 (Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface)”, Ad-hoc Chair (CMCC), RAN1#111, Toulouse, November 2022

R2-2301440, “Outcome of [Post120][054][AIML18] Data Collection” Ericsson, vivo, RAN2#121, Athens, Greece, February 2023

# Annex A: Evaluation metrics

## A.1	CSI feedback enhancement

- The content of the data

  - Radio measurements:

    - CSI-RS measurements (target H or target precoder for model training)

  - Non-radio measurements:

    - Cell ID, area ID, carrier frequency, UE/NW antenna beam configuration/ID, time stamp, UE location, measurement accuracy, measurement resolution, etc. …

  - Monitoring metrics:

    - Intermediate KPIs, e.g., SGCS

    - Legacy CSI based monitoring, additional legacy CSI reporting

- The data size

  - Data should be collected from UE(s), at one- or multiple-time instances

  - The number of bits needed for reporting a single channel measurement (a target H) depends on how the target channel is represented, i.e., the data format and preprocessing/quantization.

- Configuration-related requirements

  - It should be possible to configure a UE to store measurements on multiple occasions and then report the accumulated data to the NW

  - Periodic and event-triggered data collection approaches could be considered for further study

  - Pre-processing may need configuration, depending on how the target channel is represented

## A.2	Beam management

- The content of the data

  - Radio measurements:

    - L1-RSRP measurements and/or SSBRI/CRI (beam ID) of a set of beams at one- or multiple-time instances

  - Non-radio measurements:

    - Cell ID, area ID, carrier frequency, UE/NW antenna beam configuration/ID, time stamp, UE location/mobility/rotation, measurement accuracy, or measurement resolution, etc…

  - Monitoring metrics:

    - Prediction accuracy

- The data size

  - Data should be collected from multiple UEs, at one- or multiple-time instances

  - The number of bits needed for reporting measurements per time instance depends on the number of beams a UE is configured to measured and report

- Latency and periodicity

  - As per RAN1#111’s agreement:

    - For NW-sided AIML model monitoring, study the necessity and potential spec. impact from the following aspects:

      - UE reporting of beam measurement(s) based on a set of beams indicated by gNB,

      - Signaling, e.g., RRC-based, L1-based

- Configuration-related requirements

  - It should be possible to configure a UE to store measurements on multiple occasions and then report the accumulated data to the NW

  - Periodic and event-triggered data collection approaches could be considered for further study

# Annex B: RAN1 agreements concerning RAN2

## B.1	For CSI feedback enhancement

During RAN1#109-e the following was agreed:

Agreement Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case. - Note: Study of other sub use cases is not precluded. - Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

During RAN1#110-e the following was agreed:

## Agreement In CSI compression using two-sided model use case, further discuss at least the following aspects, including their necessity/feasibility/potential specification impact, for data collection for AI/ML model training/inference/update/monitoring: - Assistance signaling for UE’s data collection - Assistance signaling for gNB’s data collection - Delivery of the datasets.

During RAN1#110bis-e the following was agreed:

Agreement In CSI compression using two-sided model use case, study potential specification impact for performance monitoring including: - NW-side performance monitoring: NW monitors the performance and make decisions of model activation/ deactivation/updating/switching - UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model activation/ deactivation/updating/switching Agreement In CSI compression using two-sided model use case, further study potential specification impact related to assistance signaling and procedure for model performance monitoring. Agreement In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode. Agreement In CSI compression using two-sided model use case, further study at least the following options for performance monitoring metrics/methods: - Intermediate KPIs as monitoring metrics (e.g., SGCS) - Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK). - Legacy CSI based monitoring: schemes using additional legacy CSI reporting - Other monitoring solutions, at least including the following option: - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection

During RAN1#111 the following was agreed:

Agreement Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Note: Continue evaluation discussion in 9.2.2.1. Note: RAN1 Defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer futher till the end of R18 AI/ML SI. Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases. Conclusion In CSI compression using two-sided model use case, training collaboration type 2 over the air interface for model training (not including model update) is deprioritized in R18 SI. Note: - To align terminology, output CSI assumed at UE in previous agreement will be referred as output-CSI-UE. - To align terminology, input-CSI-NW is the input CSI assumed at NW

## B.2	For Beam Management enhancement

During RAN1#109-e the following was agreed:

Agreement For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations - BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams - BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams - FFS: details of BM-Case1 and BM-Case2 - FFS: other sub use cases Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

On the above, the following was also agreed for spatial-domain DL beam prediction (BM-Case 1):

Agreement For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study: - Alt.1: AI/ML inference at NW side - Alt.2: AI/ML inference at UE side

While for temporal-domain DL beam prediction (BM-Case 1) it was also agreed to:

Agreement Regarding the sub use case BM-Case2, the measurement results of K (K>=1) latest measurement instances are used for AI/ML model input: - The value of K is up to companies Agreement Regarding the sub use case BM-Case2, AI/ML model output should be F predictions for F future time instances, where each prediction is for each time instance. - At least F = 1 - The other value(s) of F is up to companies Agreement For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study: - Alt.1: AI/ML inference at NW side - Alt.2: AI/ML inference at UE side

During RAN1#110 the following was agreed:

Agreement For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact: - Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals - Content/type of the collected data - Other aspect(s) is not precluded Agreement Regarding the model monitoring for BM-Case1 and BM-Case2, to investigate specification impacts from the following aspects - Performance metric(s) - Benchmark/reference for the performance comparison - Signaling/configuration/measurement/report for model monitoring, e.g., signaling aspects related to assistance information (if supported), Reference signals - Other aspect(s) is not precluded Agreement In order to facilitate the AI/ML model inference, study the following aspects as a starting point: - Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting - Enhanced or new signaling for measurement configuration/triggering - Signaling of assistance information (if applicable) - Other aspect(s) is not precluded

During RAN1#110bis-e the following was agreed:

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation Working Assumption For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the following L1 beam reporting enhancement for AI/ML model inference - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered Agreement For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the NW-side model monitoring: - NW monitors the performance metric(s) and makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation Agreement Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the potential specification impacts from the following aspects - Beam measurement and report for model monitoring - Note: This may or may not have specification impact.

During RAN1#111 the following was agreed:

Agreement For the sub use case BM-Case1 and BM-Case2, at least support Alt.1 and Alt.2 for AI/ML model training and inference for further study: - Alt.1. AI/ML model training and inference at NW side - Alt.2. AI/ML model training and inference at UE side - The discussion on Alt.3 for BM-Case1 and BM-Case2 is dependent on the conclusion/agreement of Agenda item 9.2.1 of RAN1 and/or RAN2 on whether to support model transfer for UE-side AI/ML model or not - Alt.3. AI/ML model training at NW side, AI/ML model inference at UE sideR1-2212718 Agreement For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on the following L1 reporting enhancement for AI/ML model inference - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered Agreement Regarding the data collection for AI/ML model training at UE side, study the potential specification impact considering the following additional aspects. - Whether and how to initiate data collection - Configurations, e.g., configuration related to set A and/or Set B, information on association/mapping of Set A and Set B - Assistance information from Network to UE (If supported) - Other aspect(s) is not precluded Agreement Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the necessity and the potential specification impacts from the following aspects: - UE reporting of beam measurement(s) based on a set of beams indicated by gNB - Signaling, e.g., RRC-based, L1-based - Note: Performance and UE complexity, power consumption should be considered

## B.3	For Positioning accuracy enhancement

During RAN1#109-e the following was agreed:

Agreement Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement. - AI/ML model training - training data type/size - training data source determination (e.g., UE/PRU/TRP) - assistance signalling and procedure for training data collection - AI/ML model indication/configuration - assistance signalling and procedure (e.g., for model configuration, model activation/deactivation, model recovery/termination, model selection) - AI/ML model monitoring and update - assistance signalling and procedure (e.g., for model performance monitoring, model update/tuning) - AI/ML model inference input - report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) - model input acquisition and pre-processing - type/definition of model input - AI/ML model inference output - report/feedback of model inference output - post-processing of model inference output - UE capability for AI/ML model(s) (e.g., for model training, model inference and model monitoring) - Other aspects are not precluded - Note: not all aspects may apply to an AI/ML approach in a sub use case - Note2: the definitions of common AI/ML model terminologies are to be discussed in agenda 9.2.1

During RAN1#110-e the following was agreed:

Agreement Regarding data collection for AI/ML model training, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - Ground truth label determination (e.g., based on UE/PRU/TRP measurement/report) - Partial and/or noisy ground truth label - Signaling for data collection - Other aspects are not precluded ##### Agreement Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - AI/ML model monitoring performance metrics - Condition of AI/ML model update - Reference signals and measurement feedback/report - Other aspects are not precluded

During RAN1#110bis-e the following was agreed:

Agreement - Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning Agreement Regarding AI/ML model indication[/configuration], to study and provide inputs on potential specification impact at least for the following aspects on conditions/criteria of AI/ML model for AI/ML based positioning accuracy enhancement - Validity conditions, e.g., applicable area/[zone/]scenario/environment and time interval, etc. - Model capability, e.g., positioning accuracy quality and model inference latency - Conditions and requirements, e.g., required assistance signalling and/or reference signals configurations, dataset information - Note: other aspects are not precluded Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on potential specification impact for the following aspects - Assistance signaling and procedure at least for UE-side model - Report/feedback and procedure at least for Network-side model - Note1: study is applicable to both of the following cases - Model inference and model monitoring at the same entity - Entity to perform the model monitoring is not the same entity for model inference - Note2: other aspects are not precluded Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, at least for each of the agreed cases (Case 1 to Case 3b) - Study whether (and if so how) an entity can be used to obtain ground truth label and/or other training data - Companies are requested to report their assumption of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) - Companies are requested to report their assumption of applicable ground truth label (e.g., location or other information) and/or other training data (e.g., measurement) for each case (Case 1 to Case 3b) - Feasibility study on the entity to obtain ground truth label and/or other training data takes into account at least - availability of the entity to obtain label and/or other training data - Note: further discussion and decision of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) is not precluded based on companies’ input - Study potential signalling and procedure to enable data collection - Potential specification impact on the details of request/report of label and/or other training data, and to enable delivering the collected label and/or other training data to the training entity when the training entity is not the same entity to obtain label and/or other training data - Potential specification impact on assistance signaling indicating reference signal configuration(s) to derive label and/or other training data

During RAN1#111 the following was agreed:

Agreement For the study of benefit(s) and potential specification impact for AI/ML based positioning accuracy enhancement, one-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI. Agreement Regarding AI/ML model inference, to study and provide inputs on potential specification impact (including necessity and applicability of specifying AI/ML model input and/or output) at least for the following aspects for each of the agreed cases (Case 1 to Case 3b) in AI/ML based positioning accuracy enhancement - Types of measurement as model inference input - new measurement - existing measurement - UE is assumed to perform measurement as model inference input for Case 1, Case 2a and Case 2b; TRP is assumed to perform measurement as model inference input for Case 3a and Case 3b - Report of measurements as model inference input to LMF for LMF-side model (Case 2b and Case 3b) - For AI/ML assisted positioning, new measurement report and/or potential enhancement of existing measurement report as model output to LMF for UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a) - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - New and/or enhancement to existing assistance signaling - Note: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified for further study - For direct AI/ML positioning, ground truth label is UE location - PRU with known location - UE generates location based on non-NR and/or NR RAT-dependent positioning methods - LMF generates UE location based on positioning methods - LMF with known PRU location - Note: user data privacy needs to be preserved - For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameter(s) corresponding to AI/ML model output - PRU generates label directly or calculates based on measurement/location - UE generates label directly or calculates based on measurement/location - Network entity generates label directly or calculates based on measurement/location - The following options of entity to generate other training data at least measurement corresponding to model input are identified for further study - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: other options of entity to generate other training data are not precluded - Note: Existing PRU definition is in 38.305 Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, study benefits, feasibility and potential specification impact (including necessity) for the following aspects - Request/report of training data - Ground truth label - Measurement corresponding to model input - Associated information of ground truth label and/or measurement corresponding to model input - Assistance signaling and procedure to facilitate generating training data - Reference signal (e.g., PRS/SRS) configuration(s) and configuration identifier - Assistance information, e.g., between LMF and UE/PRU, for label calculation/generation, and label validity/quality condition, etc. - Note1: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed - Note2: Study may consider different entity to generate training data as well as different types of training data when applicable - Note3: study considers both of the following cases when applicable - when the training entity is the same entity to generate training data - when the training entity is not the same entity to generate training data Agreement - Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on feasibility, potential benefits (if any) and potential specification impact at least for the following aspects - At least the following are identified for further study as potential data for calculating monitoring metric - If monitoring based on model output - E.g. , estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning - If monitoring based on model input - E.g., measurement corresponding to model inference input - Note1: other type of potential data for model monitoring is not precluded - Note2: combination of one or more type of potential data for monitoring is not precluded - If a given type of data is necessary for calculating monitoring metric, study whether and if so - How an entity can be used to provide the given type of data for calculating monitoring metric - Companies are requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric - Potential assistance signaling and procedure to facilitate an entity providing data for calculating monitoring metric - Potential UE-network interaction - E.g., model monitoring decision indication between UE and network Agreement For AI/ML based positioning accuracy enhancement, direct AI/ML positioning and AI/ML assisted positioning are selected as representative sub-use cases.