3GPP TSG-RAN WG2 Meeting #121	R2-2301446

Athens, Greece, 27 February – 3 March 2023

Agenda item:	8.17.3

Source: 	Qualcomm Incorporated

Title:  	Coordination of MUSIM gaps for NR-DC

Document for:	Discussion and decision

# Introduction

MUSIM gaps were introduced in Rel-17. The UE can request and be configured with such gaps on NW A when it needs to perform other tasks (e.g. paging or SI reception, performing measurements) on the other network (NW B).

Unlike measurement gaps, MUSIM gaps can only be per UE in Dual Connectivity (DC). In addition, they can only be configured by MN without any SN involvement and knowledge.

Several papers were submitted to RAN2#119bis-e which brought up the above problems and proposed methods to coordinate MUSIM gaps in NR-DC. The following conclusion was captured in the Chair Notes ([2]):

1: RAN2 can discuss NW A MN-SN coordination of Rel-18 MUSIM temporary capability restrictions due to UE being configured with NR-DC in NW A.

RAN2 thinks MN-SN coordination for Rel-17 MUSIM gaps requires WI clarification in RAN

Due to the second agreement, a discussion happened at the RAN plenary and the WID was updated with the addtition of the following new objective ([1]):

Specify MN-SN coordination of R17 MUSIM gaps when network A is NR-DC in Rel-18 [RAN2]

In this contribution, we discuss the possible options for coordination of MUSIM gaps in NR-DC.

# Discussion

As mentioned above, the first issue with Rel-17 MUSIM gaps is that it can only be configured by MN without informing SN. In addition, the gap is per UE and thus the UE may or may not be able to receive/transmit with SN. Since SN is unaware of the MUSIM gaps, it can still continue to schedule the UE during the gaps, which may be missed or ignored by the UE.

Observation 1: SN is not aware of MUSIM gaps configured by the MN, which can result in unpredictable behavior or reduced performance on SCG during the gap occasions.

Therefore, the first step should be to introduce signaling whereby MN informs SN about the configuration of MUSIM gaps. For legacy measurement gaps, such coordination is done via the inter-node messages and the same framework can be used.

Proposal 1: Introduce signaling for MN to inform SN about the configuration of MUSIM gaps. This can be done via inter-node messages.

In some cases, the UE may not need the gap for both the MN and SN. This is especially true when MN operates in FR1 and SN in FR2, which is currently the prevalent deployment option for NR-DC. For legacy measurement gaps, per-FR configuration has been supported from Rel-15. It is natural to have the same flexibility for MUSIM gaps.

Observation 2: The UE does not need MUSIM gaps for both MN and SN, especially for FR1+FR2 DC.

Observation 3: Per-FR measurement gaps have been supported since Rel-15.

A simple optimization could be for the UE to inform MN that it does not need the MUSIM gaps for SN. In this case, MN does not need to inform SN and Rel-17 behavior applies.

Proposal 2: The UE can signal that the MUSIM gaps are needed only for MN.

If MUSIM gaps are only needed for SN, then it should be possible to configure it only for SN so that transmission and reception are not impacted on MCG. The existing signaling can be enhanced for the UE to enable this.

Proposal 3: The UE can request SCG-only MUSIM gaps which can be configured by the MN.

If MUSIM gaps are only needed for SCG, then no MN involvement is really necessary. Thus, we can allow SN to configure the gaps. This can be done without any stage-3 changes when SRB3 is used since UAI can be sent on SRB3.

Observation 4: SCG-only MUSIM gaps can be configured by SN without any new changes to stage-3.

Proposal 4: MUSIM gaps can be configured only for SCG by SN over SRB3.

# Conclusion

In this document, we discussed the coordination of MUSIM gaps between MN and SN in NR-DC and propose the following:

Observation 1: SN is not aware of MUSIM gaps configured by the MN, which can result in unpredictable behavior or reduced performance on SCG during the gap occasions.

Proposal 1: Introduce signaling for MN to inform SN about the configuration of MUSIM gaps. This can be done via inter-node messages.

Observation 2: The UE does not need MUSIM gaps for both MN and SN, especially for FR1+FR2 DC.

Observation 3: Per-FR measurement gaps have been supported since Rel-15.

Proposal 2: The UE can signal that the MUSIM gaps are needed only for MN.

Proposal 3: The UE can request SCG-only MUSIM gaps which can be configured by the MN.

Observation 4: SCG-only MUSIM gaps can be configured by SN without any new changes to stage-3.

Proposal 4: MUSIM gaps can be configured only for SCG by SN over SRB3.

# References

- RP-223492, Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, vivo

- Report on LTE legacy, DCCA, MUSIM, Slicing, 71 GHz, XR and QoE