3GPP TSG-RAN WG2 #121	Tdoc R2-2301448

Athens, Greece, 27th February – 3rd March 2023

Agenda Item:	8.17.2

Source:	Ericsson

Title:	Overall Dual-RX/TX MUSIM Solution

Document for:	Discussion

# 1	Introduction

RAN2 WID RP-223492 [1] defines three objectives for the support of Dual Tx/Dual Rx Multi-USIM devices in Rel-18, quoted here:

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4]. - Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose - RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR. - Applicable UE architecture: Dual-RX/Dual-TX UE 2.Specify MN-SN coordination of R17 MUSIM gaps when network A is NR-DC in Rel-18 [RAN2] 3. Define RRM requirements for Rel-17 MUSIM gaps [RAN4, RAN2] - Define RRM requirements for Rel-17 MUSIM gaps [RAN4, RAN2] - The following MUSIM gap requirements are considered - Measurements in Network A - Measurements in Network B in RRC idle/inactive - Note: it is up to RAN4 decision whether to define requirements for Network B. - Identify and specify, if needed, solutions for MUSIM gap collision handling for the following cases [RAN4, RAN2] - Case 1: Collisions between MUSIM gap and legacy measurement gap (i.e., Rel-15 to Rel-17 measurement gaps) - Case 2: Collisions between MUSIM gap and SMTC - Case 3: Collisions between different MUSIM gaps - Note: RAN2 work can be triggered by RAN4 LS only, if needed - Identify impacts on L1 measurements, RLM/BFD and L3 measurements and specify corresponding UE requirements, if necessary, when MUSIM gap(s) are configured, for the following scenarios [RAN4] - Only MUSIM gap(s) are configured MUSIM gap(s) and legacy measurement gap are configured

The focus of this contribution is to the objective 1. A proposed mechanism is described for the scenario when Network A (a.k.a. NW-1) is NR with DC and Network B (a.k.a. NW-2) is NR. However, the solution can be also extended to the case when Network A is NR SA with CA or when Network B is LTE.

# 2	Discussion

The proposed procedure, which allows a Dual Tx/Dual Rx Multi-USIM device to operate in RRC_CONNECTED state simultaneously in NW-1 and NW-2, is shown in the figure below.

The signalling flow shows the entire procedure: registration phase, connection to NW-1 using full capability, simultaneous connection to both NW-1 and NM-2 using restricted capability, release of the connection with NW-2 and re-enabling of the full capabilities with NW-1.

Note that the MUSIM gap handling is discussed in [2].

The UE performs the registration procedure to both the networks as per legacy (e.g., the UE provides the full capabilities to both the networks). Then the UE sets up the connection to NW-1 using full capabilities (e.g., dual connectivity is used).

The UE performs the registration procedure to both the networks as per legacy (e.g., providing the full capabilities to both the networks) and sets up the first connection to NW-1 using its full capabilities.

When the UE wants to connect to NW-2 (e.g., because it has been paged), the UE has to request a temporary restriction of its capabilities to NW-1, for the following reasons:

- Ensure UE hardware (RX/TX) resources can be used for other NW.

- Ensure UE (baseband) processing capability is sufficient for connection with both NW-1 and NW-2.

- Avoid UE In-Device Co-existence (IDC) issues caused by NR communication in the other NW.

The UE requests the restriction of its capabilities to:
1) Ensure UE hardware (RX/TX) resources can be used for other NW.
2) Ensure UE (baseband) processing is sufficient for connection to both NWs.
3) Avoid UE IDC issues caused by NR communication in the other NW.

The UE can use the UEAssistanceInformation message to indicate the network its preference to temporary use restricted capabilities.

The UE uses the UEAssistanceInformation message to indicate the network (NW-1) its preference to temporary use restricted capabilities for Multi-USIM purposes.

In order to ensure that the UE hardware (i.e. one transceiver) is available for NW-2, the UE can indicate to NW-1 to release all SCells and/or the SCG, and simply continue the connection with NW-1 using the Pcell only. Similarly, the connection to be established with NW-2 should, for a simple and robust solution, not use CA/DC. This can e.g. be solved by an explicit indication to release CA/DC, or by reusing the IDC affectedCarrierFreqCombList.

In the Rel-18 MUSIM solution, CA/DC is not used when the UE has simultaneous connections to the two networks.

Furthermore, the mechanism must ensure that NW-1 does not utilize the fact that UE will operate in non-CA/non-DC to e.g. configure more MIMO layers for the Pcell. This can be implicit, by the fact that UE requests SCells/SCG release caused by MUSIM operation and NW-1 knows the PCell configuration (e.g. number of MIMO layers) should be kept. This can also be requested explicitly by UE, indicating max number of MIMO layers in the UEAssistanceInformation message.

In the Rel-18 MUSIM solution, UE (baseband) processing capability, e.g. the number of MIMO layers, should be limited when the UE has simultaneous connections to the two networks.

It is important to note that the SCells/SCG should be released and not de-activated. The reason is that just de-activating the SCells/SCG will not really work, since the UE is still expected to continue RRM measurements and even RLM/BFD on the deactivated SCG. So, the transceiver is not full available to be used to connect to the second network.

The deactivation of the SCells/SCG does not free up the transceiver, since the UE is still expected to continue RRM measurements and even RLM/BFD on the deactivated SCG. So, the transceiver is not full available to be used to connect to the second network.

What we have discussed so far in terms of UE capability restrictions would likely be sufficient in most MUSIM scenarios. In case UE would face IDC issues from the communication with both NW-1 and NW-2, it could simply request release with one of the networks.

If this WI should also consider MUSIM related IDC issues, a mechanism based on the existing IDC functionality (NW configures candidateServingFreqListNR, and UE indicates IDC problems using UAI message, including the field affectedCarrierFreqList) can be re-used for the MUSIM purpose.

A mechanism based on the existing IDC functionality can be re-used for MUSIM related IDC problems.

In a solution based on UEAssistanceInformation message, a “prohibit timer” is needed. However, its main purpose is not to prohibit frequent UAI signalling, but rather to allow the network to react to the UE indication of restricted capabilities and respond to UE (with RRCReconfiguration). At timer expiration in UE, the UE can take some actions, e.g., either leave NW-1 (using Rel-17 MUSIM leaving solution), or not respond to the Paging message in NW-2.

A prohibit timer is introduced to allow the network to react to the UE indication of restricted capabilities and respond to UE. At timer expiration, the UE can e.g., either leave NW-1 (using Rel-17 MUSIM leave indication), or not respond to the Paging message in NW-2.

After the reconfiguration from NW-1, the UE can use the available transceiver to setup/resume the connection to NW-2. When the UE connects to NW-2, it notifies the RAN about its temporary restricted capabilities, since full capabilities were provided at Registration phase. This can be done by indicating in RRCSetupComplete or RRCResumeComplete the information about the actual restricted capabilities. Alternatively, a flag can be used to indicate that the actual restricted capabilities will be sent in a following UEAssistanceInformation message.

When the UE connects to NW-2, it notifies the RAN about its restricted capabilities, since full capabilities were provided at Registration phase.

The UE notifies NW-2 about its restricted capabilities, by indicating in RRCSetupComplete or RRCResumeComplete the actual restricted capabilities or by including a flag indicating that the actual restricted capabilities are sent in a following UEAssistanceInformation message.

The connection is setup/resumed in NW-2. The UE is simultaneously in RRC_CONNECTED state in the two networks, but the ongoing services use restricted capabilities and without using CA/DC. When one of the ongoing services is terminated (e.g., in NW-2), the connection is released/suspended, and the UE enters IDLE/INACTIVE state. The UE can use the available transceiver to use the full capabilities in NW-1. The UE sends an UEAssistanceInformation message without the new IE including the restriction. The network interprets that as a request from the UE to use full capability (i.e., removal of the temporary restrictions). Even though the UE can operate with full capabilities, the SCG/SCells are not automatically setup since the network conditions may have been changed. The SCG/SCells are setup as needed and e.g., triggered by the UE measurement events/reports.

The removal of the temporary restrictions is indicated by sending the UEAssistanceInformation message where the new IE with the capabilities restriction is not present.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The UE performs the registration procedure to both the networks as per legacy (e.g., providing the full capabilities to both the networks) and sets up the first connection to NW-1 using its full capabilities.

Observation 2	The UE requests the restriction of its capabilities to:
1) Ensure UE hardware (RX/TX) resources can be used for other NW.
2) Ensure UE (baseband) processing is sufficient for connection to both NWs.
3) Avoid UE IDC issues caused by NR communication in the other NW.

Observation 3	The deactivation of the SCells/SCG does not free up the transceiver, since the UE is still expected to continue RRM measurements and even RLM/BFD on the deactivated SCG. So, the transceiver is not full available to be used to connect to the second network.

Observation 4	When the UE connects to NW-2, it notifies the RAN about its restricted capabilities, since full capabilities were provided at Registration phase.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The UE uses the UEAssistanceInformation message to indicate the network (NW-1) its preference to temporary use restricted capabilities for Multi-USIM purposes.

Proposal 2	In the Rel-18 MUSIM solution, CA/DC is not used when the UE has simultaneous connections to the two networks.

Proposal 3	In the Rel-18 MUSIM solution, UE (baseband) processing capability, e.g. the number of MIMO layers, should be limited when the UE has simultaneous connections to the two networks.

Proposal 4	A mechanism based on the existing IDC functionality can be re-used for MUSIM related IDC problems.

Proposal 5	A prohibit timer is introduced to allow the network to react to the UE indication of restricted capabilities and respond to UE. At timer expiration, the UE can e.g., either leave NW-1 (using Rel-17 MUSIM leave indication), or not respond to the Paging message in NW-2.

Proposal 6	The UE notifies NW-2 about its restricted capabilities, by indicating in RRCSetupComplete or RRCResumeComplete the actual restricted capabilities or by including a flag indicating that the actual restricted capabilities are sent in a following UEAssistanceInformation message.

Proposal 7	The removal of the temporary restrictions is indicated by sending the UEAssistanceInformation message where the new IE with the capabilities restriction is not present.

# References

RP-223492, WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

R2-2301449, Tdoc: Discussion on MUSIM gaps for a Dual-RX/Dual-TX UE, Ericsson