3GPP TSG RAN WG2 Meeting #121                        R2-2301460

February 27th – March 3rd, 2023

Source:	Sharp

Title:	NTN-TN Mobility Cell Reselection

Agenda Item:	8.7.4.1.1

Document for:	Discussion and Decision

# Introduction

In RAN2#120 meeting, an agreement was made regarding identification of TN and NTN cells for cell reselection enhancements:

Agreements:

- RAN2 will first continue the investigation on the details of the TN coverage data (e.g. accuracy requirements for describing where TN network(s) is/are available) and UE storage overhead before deciding how to send the information to the UE.

- Continue the discussion on whether to introduce explicit indication to identify TN cells from inter-frequency list and inter-RAT frequency list (FFS on the granularity) or whether we rely on implicit information.

This paper discusses further considerations on the highlighted objective shown above.

# Discussion

## 2.1    NTN and TN cell differentiation

The satellite coverage footprint of NTN has two major differences compared to TN coverage area:

- The NTN coverage area size can be much larger than TN coverage area, especially if GSO satellite is used

- If satellite type is non-GSO, the coverage area of NTN moves

In previous RAN2 meetings, reselection priority of NTN and TN cells were discussed.  The general consensus is that if UE is camped on NTN cell, then the TN cell frequency will be assigned with a higher priority.  However, due to the larger coverage area, camping on an NTN cell reduces cell reselections resulting in lower UE power consumption. While the mechanism for assignment of neighbour cell priority (TN vs NTN) is still being discussed, for cell reselection, always choosing TN neighbour cell may not be ideal.  Hence the UE requires categorization of TN and NTN cells is necessary.

Observation: If UE is in an area with both, TN and NTN coverages, the UE must differentiate between TN and NTN neighbour cell frequencies/ID

NTN satellites uses different frequency bands that include C (4-8GHz), X (8-12GHz), Ku (12-18GHz), and Ka (26-40GHz).  As a result, the ARFCN-ValueNR will be defined by the specified satellite frequencies.  If neighbour ARFCN-ValueNR is not within the range of the satellite frequency ARFCN-ValueNR(s), the cell is a TN cell. NTN cells are also identified in broadcasted SIB messages (i.e. SIB19) and other messages that contains NTN-Config IE [1].

Proposal 1: RAN2 consider that UE can implicitly determine whether the cell is TN or NTN

However, if TN and NTN frequencies will co-exist and utilize the same range of channels, then RAN2 may consider discussing alternatives for identifying TN from NTN cells. While it may be possible that TN may use the same band as NTN, we foresee that the allocation of ARFCN-ValueNR will be separated between TN and NTN

3GPP TR38.331 v17.3.0