3GPP TSG-RAN WG2 Meeting #121											R2-2301461

Athens, Greece, 27th Feb.– 3rd Mar., 2023

Agenda item:       5.2.2

Source: 	Huawei, HiSilicon

Title: 	Summary on RRC CRs

Document for:	Discussion and Decision

# Introduction

This document summarizes the contributions submitted to AI 5.2.2. Rapporteur’s recommendations on how to treat them are provided.

# Correction CRs to be discussed together with Rapporteur’s Misc CR

The following correction CRs could be discussed together. The agreed changes for RRC spec could be merged into a Rapporteur’s miscellaneous correction CR and its Rel-17 mirror CR.

Table 1: Correction CRs

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2300485 | Miscellaneous corrections on 38.331 | Huawei, HiSilicon | 1. In 6.3.5 Sidelink information elements, in the current field description of sl-TxPoolExceptional, it is stated that the resource in the pool is used for NR sidelink communication. Actually, the resource in the pool can also be applied to transmit NR sidelink discovery, which is not included in the current description. 2. All IEs refer to SL-PSSCH-TxConfigList for the configuration of “retransmission number”. However, the field description of sl-MaxTxTransNumPSSCH in SL-PSSCH-TxConfigList is that “Indicates the maximum transmission number (including new transmission and retransmission) for PSSCH”. So it is unclear on what sl-MaxTxTransNumPSSCH is really used for, the maximum transmission number or the maximum retransmission number. | The first change is to cover the missing case of "sidelink discovery transmission" which should be reasonable. The second change is to solve the same issue as vivo CR in R2-2301021, without causing any impacts on already implemented products, according to discussion of last meeting. |
| R2-2300836 | Miscellaneous corrections on TS 38.331 for NR sidelink | Xiaomi | - In clause 6.3.5, some parameters of SL-ConfiguredGrantConfig only apply to sidelink configured grant type 1, some paraemters of SL-ConfiguredGrantConfig only apply to sidelink configured grant type 2 and the other paraemeters apply to both sidenlink configured grant type 1 and type 2. However this is not clearly stated in the field descriptions of these parameters of of SL-ConfiguredGrantConfig. - Some editorials still exist. | Editorial changes (configured sidelink grant->sidelink configured grant, Type->type) can be accepted. Changing SL to sidelink may be unnecessary as SL is used "pervasively" in RRC spec without ambiguity. On adding "it applies for both type 1 and type 2", it can be discussed whether this addition is critically needed, as for those attributes of SL CG (ID, period, max number of transmission), the implication is that they are applicable for both CG types. |
| R2-2301021 | Clarification on retransmission number in SL-PSSCH-TxConfigList | vivo | According to current MAC specification in clause 5.22.1.1 as below, the UE will select a “retransmission number” from the “the allowed numbers”, where the “the allowed numbers” is defined by overlapped values within sl-MaxTxTransNumPSSCH included in sl-PSSCH-TxConfigList and sl-MaxTxTransNumPSSCH indicated in sl-CBR-PriorityTxConfigList. - select the number of HARQ retransmissions from the allowed numbers, if configured by RRC, in sl-MaxTxTransNumPSSCH included in sl-PSSCH-TxConfigList and, if configured by RRC, overlapped in sl-MaxTxTransNumPSSCH indicated in sl-CBR-PriorityTxConfigList for the highest priority of the logical channel(s) allowed on the carrier and the CBR measured by lower layers according to clause 5.1.27 of TS 38.215 [24] if CBR measurement results are available or the corresponding sl-defaultTxConfigIndex configured by RRC if CBR measurement results are not available; On the other hand, in current RRC, the field of sl-MaxTxTransNumPSSCH is described as “the maximum transmission number (including new transmission and retransmission)”. Based on above observations, a potential issue is how UE understands the “the allowed numbers” in MAC, given that they are implicitly derived by the value of sl-MaxTxTransNumPSSCH in RRC, other than directly apply the value of sl-MaxTxTransNumPSSCH. Therefore, some clarifications are made on the description of sl-MaxTxTransNumPSSCH. | Technically this change would solve the issue as in Rapp's Misc CR. However, there might be "circular reference" that the new FD points to MAC spec clause 5.22.1.1 which depends on RRC configuration. Strictly speaking, "value of this field minus 1" is not specified in MAC spec. Essentially, new transmission (one time) should be always allowed, i.e. this field sl-MaxTxTransNumPSSCH is only to control the maxi number of retransmissions. |
| R2-2301377 | Clarification on sl-MaxTransPower | vivo | In the RAN4 LS, RAN4 agreed not to define PEMAX,c for S-SSB configured Tx Power in TS38.101-1. The related parameter sl-MaxTransPower which is used for configuring PEMAX,c should be updated to reflect RAN4’s agreement. And in TS 38.101-1, it is clear the this parameters can be used for PSSCH, PSCCH and PSFCH respectively | This change can be agreeable, based on the related RAN4 agreement on a resource pool. |

Proposal: Discuss the CRs listed in Table 1 offline, the agreed changes for RRC spec could be merged into Rapporteur’s miscellaneous correction CR.

# Conclusions

The contributions submitted to AI 5.2.2 are summarized, Rapporteur’s recommendations are as follows.

Proposal: Discuss the CRs listed in Table 1 offline, the agreed changes for RRC spec could be merged into Rapporteur’s miscellaneous correction CR.