3GPP TSG-RAN WG2 Meeting #121	R2-2301473

Athens, Greece, Feb 27 – Mar 3, 2023

Agenda item:	6.10.2

Source:	Samsung

Title:	BC/GC for IUC transmission

Document for:	Discussion and Decision

# 1	Introduction

In RAN1#111, RAN1 made an additional agreement on cast type(s) for inter-UE coordination information transmission triggered by a condition other than explicit request reception in scheme 1. In this document, we would like to discuss what L2 id is used for this case.

# 2	Discussion

RAN1 made the following agreement in RAN1#111 and sent the LS to RAN2 [1].

Agreement:

- The following working assumption is confirmed as follows:

- Working Assumption (RAN1#107bis-e meeting):

- For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception

- Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set

- FFS: Under which conditions groupcast/broadcast can be supported

- Unicast for preferred resource set and non-preferred resource set

- FFS: Under which conditions unicast can be supported

RAN2 had also discussion on the same issue in RAN2#120 based on [2]. Discussion and conclusion were captured as follow [3].

From R2-2211238:

Observation 3	RAN1 didn’t make a final conclusion to support groupcast/broadcast in IUC.

Observation 4	There are technical open issues to support GC/BC in IUC from RAN2 perspective, e.g., the source and destination L2 ID of the IUC MAC CE.

Proposal 4	RAN2 confirm that GC/BC in IUC is not supported in this release from RAN2 perspective

- We can wait for RAN1, but RAN2 may need further discussion in RAN2 point of view even if RAN1 decides to support it.

[LG]: RAN2 discussed this issue last meeting and it was concluded RAN1 directly can discuss it. So we should wait for RAN1. [Vivo]: LG’s right, but at the same time RAN1 does not plan to continue this discussion. [LG]: LG RAN1 prepares RAN1 discussion. [Xiaomi, Intel, Lenovo, Ericsson]: Support the proposal. [Nokia, IDC]: Agree with LG.

With the latest RAN1 decision above, RAN2 needs to discuss the following technical open issues.

- Cast type selection

- L2 source and destination id selection

Some may propose a specified criterion for the UE to select a cast type when IUC is transmitted triggered by a condition in scheme 1. Similarly, some may propose a specified rule for the UE to select L2 source and destination id when GC/BC is used for IUC transmission triggered by a condition in scheme 1. In our view, since Rel-17 SL enhancement was completed long ago, it should not be desirable to spend much specification efforts to support very late introduced additional cast types in IUC even if RAN1 decided to support them. Note this discussion may also need to involve other WG (e.g. SA2) since L2 source and destination ids are provide by the upper layer. One simple solution is to leave cast type selection and L2 source and destination id selection to the UE implementation. Otherwise it is also ok not to support GC/BC for IUC transmission triggered by a condition in scheme 1 from RAN2 point of view.

[Proposal 1]: Cast type selection and L2 source/destination id selection for IUC transmission triggered by a condition in scheme 1 are up to UE implementation.

# 3	Conclusion

We have discussed GC/BC for the condition triggered IUC transmission and made the following proposals.

[Proposal 1]: Cast type selection and L2 source/destination id selection for IUC transmission triggered by a condition in scheme 1 are up to UE implementation.

# 4	Reference

[1] R1-2212822	LS on cast types for IUC scheme 1	RAN1

[2] R2-2211238	Discussion on left issues on user plane procedure	OPPO

[3] R2-2213005	Report from session on NR SL