3GPP TSG-RAN WG2 Meeting #121	R2-2301496

Athens, Greece, 27th February – 3rd March, 2023

Agenda item:	8.1.3

Source:	Samsung

Title:	On NCR RRC states and procedures related to idle mode

WID/SID:	NR_NetCon_Repeater

Document for:	Discussion and Decision

# Introduction

In RAN#94-e the SID for Network-Controlled Repeaters was agreed [1] and the latest WID updates were approved in RAN#98-e [2]. The updated objectives of the WID are the following:

The objectives of NR NCR WI follow the recommendations defined in TR 38.867 and will focus on scenarios and assumption listed below: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands based on the NCR model in TR38.867 - For only single hop stationary network-controlled repeaters - The NCR is transparent to the UE. - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously With these considerations, NR NCR supports the following features: Specify the signalling and behavior of the following side control information for controlling the NCR-Fwd [RAN1, RAN2] - Beamforming - UL-DL TDD operation - ON-OFF information Specify control plane signalling and procedures [RAN2, RAN1] - The configuration of signalling for side control information indication - NOTE: Down-selection of solutions in section 7.2 of TR 38.867 is needed Specify the solution of network-controlled repeater management (i.e., the identification and authorization/validation of NCR) [RAN3, RAN2] - NOTE: Down-selection of solutions in section 8 of TR 38.867 is needed taking into account the feedback of other working groups (i.e., SA3 and SA5). From a security point of view, the feasibility of NCR validation procedure in solution 1 and the feasibility of solution 2 will be decided by SA3.The selected solution shall provide inter-vendor interoperability. Study the RRM functions to be supported and specify the RRM requirements of NCR-MT if necessary [RAN2, RAN4] Study and specify the RF and EMC requirements of NCR if necessary [RAN4] Note: The existing requirements defined in RAN4 can be reused if applicable. Note: The work in RAN4 for beam related is expected to start on FR2 first.

In this contribution we discuss some NCR RRC states and procedures related to idle mode.

# Discussion

- In the last meeting, there were agreements and discussions on how an NCR node operates and is managed by a Donor gNB or network. The discussions span several topics such as mandatory features and how an NCR is supposed to operate in a network, i.e in an independent UE-like fashion allowing for more self-managing or having operation being more up to network implementation.

- In this contribution we discuss how to enable a NCR to have consistent behaviour for the UE to operate in RRC idle mode.

## RRC inactive state

- In RAN2#119bis-e [3] the following agreement was taken:

Agreement: - NCR-MT supports RRC_CONNECTED and RRC_IDLE states, FFS on RRC_INACTIVE state (e.g. optional support or not support).

- The main reason for making RRC_INACTIVE state optional is that the feature may not be a crucial mode and that the power savings related to RRC_INACTIVE are likely marginal as the main source of power consumption is repeating RF signals. This is different from a UE, where the establishment procedures themselves are power consuming, especially in relation to the power consumed by the data transmissions if the amount of data to transmit is very low.

- Then in RAN2#120 [4] the following agreements were taken in relation to NCR operation in idle and inactive mode:

- WA: RRC_INACTIVE is optionally supported without any specific enhancements

- In order to allow for different implementations in how an NCR-node may be managed, we think that RRC_INACTIVE should be supported. We also cannot see anything specific to RRC_INACTIVE that blocks the NCR being in RRC_INACTIVE.

Proposal 1: Confirm the working assumption on RRC_INACTIVE being supported.

- In the last meeting there were also discussions whether there needs to be a separate capability for an NCR-MT to support RRC_INACTIVE would be required. In 38.822 [5] we can see that RRC_INACTIVE is mandatory with capability signalling:

| Features | Feature group | Components | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 6. Inactive | RRC inactive | RRC inactive | inactiveState | UE-NR-Capability-v1530 | SA only | Mandatory with capability signalling |

- For IAB-MT RRC inactive is an optional feature without any specific capability bit as seen in 38.306 (RRC inactive is not part of the table of mandatory features of an IAB-MT, invalidating RRC inactive being mandatory for a Rel-15 UE):

Table 4.2.15.1-1, Table 4.2.15.1-2 and Table 4.2.15.1-3 capture feature groups, which are mandatory for an IAB-MT. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification are optional for an IAB-MT, unless indicated otherwise.

- Thus we believe that a separate capability bit for RRC inactive is not required for NCR-MT. We propose:

Proposal 2: Do not introduce separate capability bit for NCR-MT support of RRC_INACTIVE and define RRC_INACTIVE to be optional as for IAB-MT in 38.306.

## NCR-MT RRC idle/inactive mode operation

- In the last meeting it has been agreed that NCR-Fwd may operate, i.e NCR-Fwd is ON, while NCR-MT is in RRC_IDLE. The purpose is to allow the case where a Donor gNB does not need to control the NCR operation under a long time. However, to ensure proper idle mode operation, the NCR-MT will perform cell reselection and RRM measurements in RRC_IDLE and RRC_INACTIVE.

- When a UE performs cell reselection and selects another cell to camp on, the UE will in general not need to report this to the cell that it is camping on. This is because if there are DL data for the UE, the UE will be paged using paging procedures and if there are UL data, the UE will simply connect and send the data.

- The potential problem with this is that if an NCR-MT is in RRC_IDLE, NCR-Fwd is ON and then the NCR-MT selects another cell due to cell reselection, the Donor gNB will not be made aware of the change. Furthermore, if the cell reselection resulting in camping on another cell is desired, for instance to ensure a self-optimizing network, then the NCR-Fwd is ON and configured by a Donor gNB that it is no longer camping on. These are two problems that need to be solved to ensure that an NCR device operates as expected.

- To resolve both of these problems we propose that an NCR-MT shall notify the cell that it is camping on (as long as it is NCR-capable Donor gNB) and that an NCR-MT that reselects to camp on another cell shall turn NCR-Fwd OFF.

Proposal 3: After an NCR-MT camps on a cell following cell reselection, the NCR-Fwd is turned OFF and NCR-MT notifies the cell of its presence.

- Furthermore, in the case where due to network error the NCR-MT is unable to find a suitable cell, the NCR shall also turn its NCR-Fwd OFF.

Proposal 4: If an NCR-MT finds no suitable cell during cell reselection, the NCR-Fwd is turned OFF if ON.

## Reachability in RRC_IDLE

In the last meeting there were discussions on whether a Donor gNB should be able to release an NCR-MT. The main issue that was mentioned is how an NCR-MT would be brought to RRC_CONNECTED once released to RRC_IDLE. This is because paging a UE in RRC_IDLE is only initiated by the Core Network, and in this case it would likely only be the Donor gNB that would page a UE. The use case of this would for instance be to be able to keep an NCR in RRC_IDLE while NCR-Fwd is ON, and then page the UE to bring it back to RRC_CONNECTED and then perform some type of reconfiguration of the NCR such as turning the NCR-Fwd OFF. Thus for the purpose of resolving these questions, there are two things that need to be clarified: 1) how to page an NCR_MT and 2) how an NCR-MT is released.

Observation 1: Paging a UE in RRC_IDLE is only initiated by the Core Network.

Observation 2: How to page an NCR-MT and how an NCR-MT is released needs to be solved or clarified to have an NCR-MT operating in RRC_IDLE.

### Paging an NCR-MT

As mentioned above, to reach a UE in RRC_IDLE it is normally initiated by the Core Network, thus to enable we need to consider the paging procedure. The full paging procedure involves PHY (RAN1), protocol layers (RAN2/RAN3), core network (SA2) and core network signalling aspects (CT1).

There are two main of types of paging – Core Network-initiated paging and RAN-initiated paging. Core Network-initiated paging is due to presence of downlink data or Core Network signaling for the UE while RAN-initiated paging is for a UE in RRC_INACTIVE (which may also be due to downlink data or signaling from a core network). A UE in RRC_IDLE only monitors for Core Network-initiated paging, where a UE may use a default or UE-specific cycle. A UE in RRC_INACTIVE monitors for both Core Network-initiated and RAN-initiated paging. For monitoring for paging in RAN-initiated paging the UE applies a UE-specific cycle. The Paging Frame (the Radio Frame that a Paging Occasion occurs) and Paging Occasion (the specific subframe within a Radio Frame where the paging is signaled) of a UE is derived partly based on network configurations but also based on the ID of the UE, which is derived based on temporary UE identifier 5G-S-TMSI. The UE_ID that is used to determine paging occasion is calculated as UE_ID = 5G-S-TMSI mod 1024 (except for eDRX where it is calculated as UE_ID = 5G-S-TMSI mod 4096). If the UE has not been allocated a 5G-S-TMSI, UE_ID = 0 is used.

When a UE has detected a page being sent in one of its Paging Occasions the UE checks whether the paging record matches its own identity. In general, a UE in RRC_IDLE checks whether the paging record matches its ng-5G-S-TMSI while a UE in RRC_INACTIVE checks whether the paging record matches its i-RNTI. A UE in RRC_IDLE indicates to the upper layers regarding the received page, where the upper layers (NAS) will determine the course of action. A UE in RRC_INACTIVE will perform the RRC Resume procedure if the identity matches.  The ng-5G-S-TMSI is given by an AMF while an i-RNTI is given by a gNB in SuspendConfig.

As part of 5G-specific enhancements to protect against so called IMSI-catchers whereby a relatively simple device can log and track IMSIs based on paging, SA3 has specified that a UE shall be assigned a new identifier after every successful paging attempt. This means that the 5G-S-TMSI and I-RNTI is required to be replaced after every successful paging procedure from RRC_IDLE and RRC_INACTIVE [6].

Paging is also used to notify a UE of system information updates and ETWS/CMAS updates, but this uses Short Message.

RRC_IDLE will be mandatory to support for an NCR-MT, and the most likely state that the NCR-MT will be in while forwarding. Thus as a first proposal to enable proper NCR operation, we propose that Donor gNB should be able to initiate paging of an NCR-MT in RRC_IDLE.

Proposal 5: Donor gNB can initiate the paging procedures to reach an NCR-MT in RRC_IDLE.

To be able to support a Donor gNB to initiate the paging procedure to reach an NCR-MT in RRC_IDLE there are some questions that need to be considered

- Security requirements – How to keep paging related security procedures the same for paging an NCR-MT as a UE

- Keeping paging procedures local to RAN – How to minimize network procedure impact for paging an NCR-MT

For the direction is taken above, there can be some different options.

Perform paging by as in normal procedures, but initiated by Donor gNB. This approach requires SA3 and possibly SA2 input as 5G-S-TMSI that is used during the paging procedures may not be automatically update as is required by SA3 specifications.

Observation 3: Paging NCR-MT in RRC_IDLE using 5G-S-TMSI requires SA3/SA2 input.

Paging using NCR-specific RNTI. In RRC_INACTIVE, the UE is assigned an i-RNTI which is used to identify a PagingRecord being directed to the UE. The UE still however applies 5G-S-TMSI to derive the UE_ID, but it does not need to be updated in-between. In RRC_IDLE for an NCR-MT, a similar approach can be taken. In this case the NCR-MT is given an NCR-RNTI to be used for identifying its identity when being paged. In this case the NCR-RNTI may be changed in-between each paging procedure.

Paging using Short Message. Short message is as explained above used for system information update and ETWS/CMAS updates. The Short Message contains 1-bit fields indicating either the that the UE shall perform some type of action, such as acquiring system information or acquiring ETWS and CMAS notifications. This can be modified to include an indication that a NCR-MT is being paged.

We propose that RAN2 should discuss these options, decide on an approach and then indicate to SA3/SA2/RAN2 regarding our solution and ask for feedback.

Proposal 6: RAN2 to discuss options on paging an NCR-MT in RRC_IDLE.

Proposal 7: Send LS to SA3 regarding our solutions/options for paging an NCR-MT in RRC_IDLE.

### Releasing an NCR-MT

- For releasing an NCR-MT, if the problems related to how to reach an NCR-MT in RRC_IDLE are resolved, a NCR-MT need to be able to be released. Thus to resolve the FFS above, this can be done in the same way that a UE is released, i.e through RRCRelease.

Proposal 8: An NCR-MT can be released to RRC_IDLE (or RRC_INACTIVE) via RRCRelease.

In some cases, according to what was agreed in the past meeting, it should be possible for the NCR-MT to be in idle mode while the NCR-Fwd is ON. This means that the NCR-MT can be released via RRCRelease and still maintain NCR-Fwd ON.

However, in some cases the release of the NCR might for instance be due to issue with NCR operation or due to any other reason. As an example the connection to the Donor gNB may be poor, but NCR-Fwd is still ON. In these cases releasing the NCR-MT might also require NCR-Fwd to be turned off in the RRCRelease message. Thus we think that it should be possible to turn NCR-Fwd OFF in a RRCRelease message.

Proposal 9: An NCR-Fwd can be turned OFF in a RRCRelease message.

# Conclusion

In this contribution we discussed issues related to NCR RRC states and procedures.

Observation 1: Paging a UE in RRC_IDLE is only initiated by the Core Network.

Observation 2: How to page an NCR-MT and how an NCR-MT is released needs to be solved or clarified to have an NCR-MT operating in RRC_IDLE.

Observation 3: Paging NCR-MT in RRC_IDLE using 5G-S-TMSI requires SA3/SA2 input.

Proposal 1: Confirm the working assumption on RRC_INACTIVE being supported.

Proposal 2: Do not introduce separate capability bit for NCR-MT support of RRC_INACTIVE and define RRC_INACTIVE to be optional as for IAB-MT in 38.306.

Proposal 3: After an NCR-MT camps on a cell following cell reselection, the NCR-Fwd is turned OFF and NCR-MT notifies the cell of its presence.

Proposal 4: If an NCR-MT finds no suitable cell during cell reselection, the NCR-Fwd is turned OFF if ON.

Proposal 5: Donor gNB can initiate the paging procedures to reach an NCR-MT in RRC_IDLE.

Proposal 6: RAN2 to discuss options on paging an NCR-MT in RRC_IDLE.

Proposal 7: Send LS to SA3 regarding our solutions/options for paging an NCR-MT in RRC_IDLE.

Proposal 8: An NCR-MT can be released to RRC_IDLE (or RRC_INACTIVE) via RRCRelease.

Proposal 9: An NCR-Fwd can be turned OFF in a RRCRelease message.

# Reference

RP-213700, New SI: Study on NR Network-controlled Repeaters, ZTE Corporation, RAN#94-e, December 2021.

RP-223505, Revised WID on NR network-controlled repeaters, ZTE, Sanechips, RAN#98-e, December 2022

RAN2#119bis-e Chairman’s notes.

RAN2#120 Chairman’s notes.

3GPP TR 38.822, “User Equipment (UE) feature list”, Release 16, V16.4.0, December 2022

3GPP TS 33.501, “Security architecture and procedures for 5G system”, Release 18, V18.0.0, December 2022

3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”, V17.3.0, December 2022.