3GPP TSG-RAN WG2 Meeting #121	R2-22xxxxx

Feb 27 – Mar 3, 2023

Agenda Item		:	8.4.2.3

Source		:	LG Electronics Inc.

Title		:	Remaining issues of LTM execution procedure

Document for		:	Discussion and Decision

1.	Introduction

In Rel-18, L1L2 mobility is one of the important features for further NR mobility enhancements, which is introduced in order to reduce latency, overhead and interruption time [1]. In this paper, we discuss the remaining issues of L1/L2-Triggered Mobility (LTM) execution procedure.

2.	Discussion

Resource starvation at a candidate cell

After configuring the UE with a legacy conditional mobility (e.g., CHO, CPA, or CPC), the candidate cell reserves the resource indicated in the pre-configuration for the UE until the reconfiguration with sync for the UE is completed or the network releases the pre-configuration. This means that the candidate cell does not allocate the resource indicated in the pre-configuration to other UEs until the mobility completion of the UE or the release of the pre-configuration by the network. Since, in legacy conditional mobility, it is assumed that a candidate cell will soon become a serving cell for the UE, candidate cells other than the candidate cell to be the serving cell reserve the resources for conditional mobility of the UE for a short time. Hence, a candidate cell, in legacy conditional mobility, may not suffer from resource shortage caused by reserving resources for conditional mobility.

For Rel-18 LTM, a candidate cell may not assumed to be a serving cell soon because it seems that a cell, which cannot soon become a serving cell for the UE, can be designated as a candidate cell for LTM dynamic cell switch. Further, the UE does not discard the configurations of candidate cells for subsequent LTM, which means that the UE preserves the pre-configurations for LTM after successful completion of an LTM execution. Consequently, a candidate cell may reserve the resource for the UE for a long time. If one cell is configured as an LTM candidate cell for a plurality of UEs, the cell would experience resource starvation caused by reserving many resources for LTM.

Observation 1. Since a candidate cell may undergo resource starvation caused by reserving many resources for LTM, it is not desirable for the candidate cell to reserve the resource for LTM.

In order to execute LTM dynamic cell switch, the UE would firstly apply the RRC configuration of the candidate cell indicated by the MAC CE carrying LTM related information (e.g. configuration index of candidate cell). After application of the RRC configuration, for CFRA-based LTM execution case, the UE would transmit, to the candidate cell, the RA preamble configured in the RACH-ConfigDedicated IE with the configuration of first active UL BWP (i.e. firstActiveUplinkBWP IE) as in legacy reconfiguration with sync.

If all CFRA preambles of a cell is used for configuring CFRA-based LTM dynamic cell switch for a plurality of UEs, the cell would reserve all CFRA preambles for LTM dynamic switch. Then, the cell may not admit a new UE other than the UEs configured with CFRA-based LTM by another mobility (e.g. legacy handover, CHO, CPA, CPC, etc.) due to lack of CFRA preambles. In other words, a cell may not allocate a CFRA preamble to a UE, which needs to move into the cell by a mobility other than LTM, even if one or more CFRA preambles are being not used for mobility (i.e., just reserved for UEs configured with CFRA-based LTM) at the moment the UE needs to move into the cell.

The resource starvation problem due to LTM described above can be addressed by configuring a physical channel of a BWP for LTM in a shared manner. Specifically, two or more sets of configurations for a physical channel of a BWP is configured for the UE with the pre-configuration of LTM. The physical channel configurations included in the pre-configuration are not reserved for the UE and can be used for UEs served by the candidate cell or a new UE, which needs to move into the cell by mobility, until the UE executes a cell switch to the candidate cell. If the source cell detects the need of LTM dynamic cell switch to the candidate cell for the UE, the source cell initiates negotiations with the candidate cell, where the negotiation may be on the coordination over F1 among serving DU, target DU, and CU. During the negotiation between the source cell and the candidate cell to determine whether to perform an LTM dynamic cell switch from the source cell to the candidate cell for the UE, the candidate cell may reserve one physical configuration out of the configurations for the physical channel of the BWP initially applied at LTM execution in the pre-configuration for the UE, and then the candidate cell indicates the reserved physical channel configuration to the source cell. The reserved physical channel configuration is indicated by the MAC CE triggering LTM dynamic cell switch (e.g. Cell Switch MAC CE).

Observation 2. The resolve the resource starvation problem due to LTM could be addressed by configuring a physical channel of a BWP for LTM in a shared manner, e.g., by the followings:

- Two or more sets of configurations for a physical channel of a BWP is configured for the UE with the pre-configuration;

- The physical channel configurations included in the pre-configuration are not reserved for the UE and can be used for UEs served by the candidate cell or a new UE, which needs to move into the cell by mobility, until the UE executes a cell switch to the candidate cell;

- The source cell initiates negotiations with the candidate cell if the source cell detects the need of LTM dynamic cell switch to the candidate cell for the UE;

- During the negotiation between the source cell and the candidate cell to determine whether to perform an LTM dynamic cell switch from the source cell to the candidate cell for the UE, the candidate cell may reserve one physical configuration out of the configurations for the physical channel of the BWP initially applied at LTM execution in the pre-configuration for the UE;

- The reserved physical channel configuration is indicated by the MAC CE triggering LTM dynamic cell switch (e.g. Cell Switch MAC CE).

Based on the above observations, we propose the followings (i.e. Proposals 1 and 2):

Proposal 1. For the pre-configuration of a candidate cell, the UE is configured with two or more sets of configurations for a physical channel of a BWP.

Proposal 2. For each physical channel presented as a set of configurations in the preconfiguration, Cell Switch MAC CE indicates one physical channel configuration out of the corresponding set of configurations. FFS inter-node signaling for determining the physical channel configuration to be indicated is up to RAN3.

TA and beam management for RACH-less LTM

In the meeting RAN2#119bis-e, RAN2 made the following agreement:

RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this.

In the meeting RAN1#111, the followings were agreed:

On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH. - The PDCCH order is only triggered by source cell - FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc. - Note: any other RACH-based solutions are for discussion separately TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW. - same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell

From the above agreements related to RACH-less LTM, the initial TA acquisition and TA updating for a candidate cell is triggered by the network, i.e., PDCCH order triggered RACH in a candidate cell. If a number of candidate cells are configured for the UE, the UE may need to frequently perform a PDCCH order triggered RACH in each candidate cell. This implies that the UE may spend a considerable amount of time for TA management of the candidate cells, which results in a communication interruption between the UE and serving cells.

To reduce the interruption due to TA management of the candidate cells, it may be beneficial to group candidate cells to share the same TA value. That is, the UE can be configured with multiple TAGs for candidate cells. For example, the UE is configured with 8 candidate cells for RACH-less LTM, where the candidate cells can be classified into three groups to share the same TA value, e.g., TAG#1 ={CCell#1, CCell#4}, TAG#2={CCell#2, CCell#3, CCell#7}, and TAG#3={CCell#5, CCell#6, CCell#8}. The UE performs PDCCH order triggered RACH eight times if TA is managed at the cell-level, while the UE performs PDCCH order triggered RACH three times if TA is managed at the TAG-level. It is observed that TAG-level TA management shows less interruption due to TA update than cell-level TA management. Therefore, it is beneficial to group candidate cells to share the same TA value, i.e., the UE can be configured with multiple TAGs for candidate cells.

Proposal 3. Candidate cells can be grouped to share the same TA value for RACH-less LTM, i.e., UE can be configured with multiple TAGs for candidate cells.

How to indicate UE arrival in the target cell

In the case of RACH-based LTM execution, the UE may transmit, to the candidate cell, an RA preamble indicated in the pre-configuration. As described in Observation 1, with legacy principle, configurations for CFRA-based LTM may cause the resource starvation problem. If Proposal 1 and Proposal 2 to resolve the resource starvation problem are agreed, Cell Switch MAC CE indicates the RA preamble to be used for executing LTM by referring to one of configurations for PRACH in the pre-configuration. The UE transmits the RA preamble indicated in Cell Switch MAC CE if the MAC CE indicates the RA preamble to be used for LTM execution, the UE transmits the RA preamble indicated in the pre-configuration otherwise.

Proposal 4. In the case of RACH-based LTM execution, the UE transmits an RA preamble to the candidate cell. Cell Switch MAC CE can indicate the RA preamble used for executing LTM by referring to one of configurations for PRACH in the pre-configuration.

In the case of RACH-less LTM execution, the UE transmits an UL signal to the candidate cell to notify its arrival. It is FFS on which UL signal will be used for indicating UE arrival in the candidate cell (i.e. target cell). In legacy mobility, the UE executes the mobility by transmitting the RRCReconfigurationComplete message to the target cell, where the RRCReconfigurationComplete message is a response to the RRCReconfiguration message triggering the mobility. Since the MAC CE (e.g. Cell Switch MAC CE) is transmitted to the UE for triggering LTM, it is straightforward that the UE transmits to the target cell a kind of Complete MAC CE (e.g. Cell Switch Complete MAC CE) to respond to the MAC CE triggering the LTM.

Proposal 5. In the case of RACH-less LTM execution, the UE indicates the arrival to the target cell by transmitting Cell Switch Complete MAC CE.

In legacy LTE RACH-less handover, the UE transmits the RRCConnectionReconfigurationComplete message to the target cell when the RRCConnectionReconfiguration message including mobilityControlInfo and rach-Skip. Then, the UE stops T304 upon the successful reception of a PDCCH transmission addressed to C-RNTI, which means that the UE determines whether LTE RACH-less handover is successfully completed or not based on PDCCH reception. Since a timer is introduced to detect a failure of LTM execution, the UE needs to know whether LTM is successfully or not. As in the legacy LTE RACH-less handover, the UE may consider LTM is successfully completed when a PDCCH from the target cell is successfully received. Therefore, during LTM execution procedure, the UE monitors PDCCH from the target cell after transmitting the first UL signal to the target cell.

Proposal 6. The UE monitors PDCCH from the target cell after transmitting the first UL signal to the target cell.

UE burden due to candidate cells

The solution for TA acquisition and update developed by RAN1 is a NW-triggered RACH-based method, i.e., the initial TA between the UE and the candidate cell is obtained by PDCCH order triggered RACH and then the TA is updated by PDCCH order triggered RACH if needed. Meanwhile, it is still FFS on how to acquire/update TA in a RACH-less manner, e.g., SRS based TA acquisition, Rx timing difference based, RACH-less mechanism as in LTE, UE based TA measurement (including UE based TA measurement with one TAC from serving cell).

RACH-less TA management (i.e. TA acquisition/update in a RACH-less manner) would be useful because the NW-triggered RACH-based TA management may not be faultless for Rel-18 LTM. Since most solutions for RACH-less TA management on the table are UE-based approaches, the UE stops communicating with a serving cell and performs UE-based TA management to a candidate cell if the condition for TA acquisition/update to the candidate cell is met. If a number of candidate cells are configured for LTM, the UE may frequently stop communicating with a serving cell to manage TAs of the candidate cells.

In a similar context to the above, the UE may suspend communication with a serving cell due to L1 measurement reporting for candidate cells. If many candidate cells are configured for LTM, the UE may frequently stop communicating with a serving cell to report L1 measurement results of the candidate cells. To reduce the suspension caused by L1 measurement reporting for candidate cells, the network can configure the UE with long periods for L1 measurement reporting for some candidate cells. However, configuring the L1 measurement reporting with a long period cannot resolve the problem for some cases. In an environment in which wireless channels fluctuates very quickly (e.g., FR2 environment), the UE needs to report L1 measurement results of candidate cells with short periods in order to perform LTM dynamic cell switch frequently.

In summary, the UE may stop communicating with a serving cell due to TA management and L1 measurement reporting for the candidate cells. This may cause communication interruption. In our view, deactivating some candidate cells could mitigate the communication interruption due to TA management and L1 measurement reporting for LTM. Based on L1 or L3 measurement results of candidate cells, the network may deactivate some candidate cells for the UE among all candidate cells configured for LTM. The UE performs TA management and L1 measurement reporting for activated candidate cells, while the UE does not perform TA management and L1 measurement reporting for deactivated candidate cells.

Proposal 7. A new mechanism to support deactivation/activation of candidate cells is needed.

Proposal 8. For a deactivated candidate cell, the UE does not perform TA management and L1 measurement reporting.

3.	Conclusion

In this paper, we have made the following observations and proposals:

Resource starvation at a candidate cell

Observation 1. Since a candidate cell may undergo resource starvation caused by reserving many resources for LTM, it is not desirable for the candidate cell to reserve the resource for LTM.

Observation 2. The resolve the resource starvation problem due to LTM could be addressed by configuring a physical channel of a BWP for LTM in a shared manner, e.g., by the followings:

- Two or more sets of configurations for a physical channel of a BWP is configured for the UE with the pre-configuration;

- The physical channel configurations included in the pre-configuration are not reserved for the UE and can be used for UEs served by the candidate cell or a new UE, which needs to move into the cell by mobility, until the UE executes a cell switch to the candidate cell;

- The source cell initiates negotiations with the candidate cell if the source cell detects the need of LTM dynamic cell switch to the candidate cell for the UE;

- During the negotiation between the source cell and the candidate cell to determine whether to perform an LTM dynamic cell switch from the source cell to the candidate cell for the UE, the candidate cell may reserve one physical configuration out of the configurations for the physical channel of the BWP initially applied at LTM execution in the pre-configuration for the UE;

- The reserved physical channel configuration is indicated by the MAC CE triggering LTM dynamic cell switch (e.g. Cell Switch MAC CE).

Proposal 1. For the pre-configuration of a candidate cell, the UE is configured with two or more sets of configurations for a physical channel of a BWP.

Proposal 2. For each physical channel presented as a set of configurations in the preconfiguration, Cell Switch MAC CE indicates one physical channel configuration out of the corresponding set of configurations. FFS inter-node signaling for determining the physical channel configuration to be indicated is up to RAN3.

TA and beam management for RACH-less LTM

Proposal 3. Candidate cells can be grouped to share the same TA value for RACH-less LTM, i.e., UE can be configured with multiple TAGs for candidate cells.

How to indicate UE arrival in the target cell

Proposal 4. In the case of RACH-based LTM execution, the UE transmits an RA preamble to the candidate cell. Cell Switch MAC CE can indicate the RA preamble used for executing LTM by referring to one of configurations for PRACH in the pre-configuration.

Proposal 5. In the case of RACH-less LTM execution, the UE indicates the arrival to the target cell by transmitting Cell Switch Complete MAC CE.

Proposal 6. The UE monitors PDCCH from the target cell after transmitting the first UL signal to the target cell.

UE burden due to candidate cells

Proposal 7. A new mechanism to support deactivation/activation of candidate cells is needed.

Proposal 8. For a deactivated candidate cell, the UE does not perform TA management and L1 measurement reporting.

4.	References

[1] RP-223520	Revised WID on Further NR mobility enhancements, 3GPP TSG RAN Meeting #98-e, 12 – 16 December, 2022

[2] RP-223235	Status report: Further NR Mobility Enhancements, MediaTek Inc., 3GPP TSG RAN Meeting #98-e, 12 – 16 December, 2022