3GPP TSG-RAN2 WG2 Meeting #121	R2-2301502

# Athens, Greece,  27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3903 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on RLF cause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-16, T312 expiry in PCell was introduced as a new RLF cause: 1> else: 2> during a DAPS handover: the following only applies for the target PCell; 2> upon T310 expiry in PCell; or 2> upon T312 expiry in PCell; or 2> upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running; or 2> upon indication from MCG RLC that the maximum number of retransmissions has been reached; or 2> if connected as an IAB-node, upon BH RLF indication received on BAP entity from the MCG; or 2> upon consistent uplink LBT failure indication from MCG MAC while T304 is not running: 3> if the indication is from MCG RLC and CA duplication is configured and activated for MCG, and for the corresponding logical channel allowedServingCells only includes SCell(s): 4> initiate the failure information procedure as specified in 5.7.5 to report RLC failure. 3> else: 4> consider radio link failure to be detected for the MCG, i.e. MCG RLF; 4> discard any segments of segmented RRC messages stored according to 5.7.6.3; However, a cause value for the T312 expiry was not specified in Rel-16 RLF Report, i.e. the corresponding cause, t312-Expiry-r17 has been specified from Rel-17. It is because T312 was introduced in Rel-16. Thus, as UE declares RLF due to T312 expiry, which is not indicated by current values of rlf-Cause, how to set rlf-Cause remains ambiguous. On the other hand, ASN.1 impact has to be avoided for the current implementation. Accordingly, an option to avoid misleading in NW side is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a NOTE in clause 5.3.10.4: NOTE 1: UE is allowed to set rlf-Cause as any spare value if the UE declares radio link failure due to a cause (e.g. T312 expiry) not indicated by current values of rlf-Cause. Impact analysis Impacted functionality: RLF report Inter-operability: - if the network is implemented according to the CR and the UE is not, There is no inter-operability problem. - if the UE is implemented according to the CR and the network is not, as the network may not comprehend rlf-Cause set to a spare value, which means that the radio link failure occured due to a cause (e.g. T312 expiry) not indicated by current values of rlf-Cause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | if UE declares radio link failure due to T312 expiry, how to set rlf-Cause remains ambiguous. If the UE sets one of current cause values in rlf-Cause, it results in a misleading in NW side. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== The unchanged skipped =====

#### 5.3.10.4	RLF cause determination

The UE shall set the rlf-Cause in the VarRLF-Report as follows:

1>	if the UE declares radio link failure due to T310 expiry:

2>	set the rlf-Cause as t310-Expiry;

1>	else if the UE declares radio link failure due to the random access problem indication from MCG MAC:

2>	if the random access procedure was initiated for beam failure recovery:

3>	set the rlf-Cause as beamFailureRecoveryFailure;

2>	else:

3>	set the rlf-Cause as randomAccessProblem;

1>	else if the UE declares radio link failure due to the reaching of maximum number of retransmissions from the MCG RLC:

2>	set the rlf-Cause as rlc-MaxNumRetx;

1>	else if the UE declares radio link failure due to consistent uplink LBT failures:

2>	set the rlf-Cause as lbtFailure;

1>	else if the IAB-MT declares radio link failure due to the reception of a BH RLF indication on BAP entity:

2>	set the rlf-Cause as bh-rlfRecoveryFailure.

NOTE 1: UE is allowed to set rlf-Cause as any spare value if the UE declares radio link failure due to a cause (e.g. T312 expiry) not indicated by current values of rlf-Cause.

===== The unchanged skipped =====