3GPP TSG-RAN WG2 Meeting #121	R2-2301533

Athens, Greece, 27th February – 3th March 2023

Agenda Item:	8.5.2.1

Source:	ASUSTeK

Title:	Discussion on PDU set information and remaining time for PSDB

Document for:	Discussion and Decision

1	Introduction

In this contribution, we clarify the principle of remaining time in delay information, and discuss how to determine the remaining time in the delay information based on PDU set information.

2	Discussion

According to TR 23.700-60 v18.0.0 [1], PDU Set Delay Budget (PSDB) is defined as the time between reception of the first PDU of a PDU set and the successful delivery of the last arrived PDU of the PDU set. In addition, the principle of PSDB is applied to both DL PDU set and UL PDU set.

The following PDU Set QoS parameters are defined to support PDU Set handling: … - PDU Set Delay Budget: The PSDB defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. time between reception of the first PDU and the successful delivery of the last arrived PDU of a PDU Set. PSDB applies to the DL PDU Set received by the UPF over the N6 interface, and to the UL PDU Set sent by the UE. For a certain 5QI the value of the PSDB is the same in UL and DL.

- According to TR 23.700-60 [1], PDU Set Delay Budget is also applied to UL PDU set.

Even if the transfer delay between UE and gNB and between gNB and UPF are ideal, in our understanding, it cannot guarantee that all PDUs of a PDU set are always simultaneously delivered from upper layers to AS layer for transmission, since upper layers may deliver each PDU of one PDU set to lower layer for transmission once a PDU is generated. This situation would cause some latency between the first PDU and other PDU(s) within a PDU set.

In Figure 1, we illustrate the principle of PSDB for UL PDU set. Ideally, all PDUs of a PDU set (e.g. PS1) should be sent within the PSDB. However, it is possible that some remaining PDU may lately arrive at AS layer of UE (e.g. the latest PDU of PS2) or is not sent in time (e.g. the latest PDU of PS3). In this situation, the PSDB cannot be satisfied. Thus, there is no need to further perform transmission of these remaining PDUs (e.g. the latest PDU of PS2 and the latest PDU of PS3) to free up radio resources.

PSDB

Reception @ NW

Arrival @ UE

PS1 delivery deadline

First PDU of PS1 arrival time

First PDU of PS1 reception time

The latest PDU of PDU Set (PS)

The first PDU of PDU Set (PS)

PSDB

PS2 delivery deadline

First PDU of PS2 reception time

Latest PDU of PS1 successful delivery time

Latest PDU of PS1 arrival time

First PDU of PS2 arrival time

Latest PDU of PS2 arrival time

PSDB

PS3 delivery deadline

First PDU of PS3 reception time

Latest PDU of PS3 arrival time

First PDU of PS3 arrival time

Latest PDU of PS3 retransmission

Figure 1

- It cannot guarantee upper layers always deliver all PDUs of an UL PDU set to AS layer at a time.

- A remaining PDU of a PDU set may lately arrive at AS layer that would cause requirement of the PSDB for the PDU set cannot be satisfied.

According to TR 38.835 v1.0.0 [2], some L2 improvements are envisioned for supporting XR as quoted below.

### 5.3.2 Layer 2 Enhancements In order to enhance the scheduling of uplink resources for XR, the following improvements are envisioned: - One or more additional BS table(s) to reduce the quantisation errors in BSR reporting (e.g. for high bit rates); - Delay knowledge of buffered data, consisting of e.g. remaining time, and distinguishing how much data is buffered for which delay. It is to be determined whether the delay information is reported as part of BSR or as a new MAC CE. Also, how the delay information can be up to date considering e.g. scheduling and transmission delays needs to be investigated further. - Additional BSR triggering conditions to allow timely availability of buffer status information can be investigated further. - Delivery of some assistance information (e.g. periodicity) reusing TSCAI as a baseline. Whether additional mechanism is required can be further considered with an assumption that all information may not be always available at UE application.

According to above highlight, the delay information contains: (1) remaining time and (2) amount of buffered data associated with the remaining time. However, it is not clear what the remaining time refers to.

According to the current PDCP specification, each PDCP SDU is associated with a discardTimer and the corresponding PDCP PDU is buffered in the PDCP layer. The past time of the running discardTimer means time of the data buffered in PDCP layer. If the remaining time in delay information refers to the time of the data buffered in PDCP layer, we think the PSDB to be exhausted for the buffered data cannot be accurately reflected to gNB for proper resource scheduling. For example in Figure 2 shown below, the UE could send one delay information report to inform gNB about the remaining time of discardTimer for the latest PDU of PS4. The UE could also send another delay information report about the remaining time of discardTimer for the latest PDU of PS5. In this example, the remaining time of discardTimer for the latest PDU of PS4 would be similar with the one for the latest PDU of PS5. In fact, the PSDB to be exhausted in case of PS5 occurs earlier than the case of PS4.

PSDB

Reception @ NW

Arrival @ UE

PS4 delivery deadline

First PDU of PS4 arrival time

The latest PDU of PDU Set (PS)

The first PDU of PDU Set (PS)

PSDB

PS5 delivery deadline

Latest PDU of PS4 arrival time

First PDU of PS5 arrival time

Latest PDU of PS5 arrival time

Time to be exhausted for PSDB

Delay information report

Figure 2

- If the remaining time in the delay information to be reported from UE to gNB is determined based on duration of data buffered in AS layer, it cannot accurately reflect the PSDB to be exhausted.

Thus, we think the remaining time in the delay information should be clarified, and we also propose:

- RAN2 clarifies that the remaining time in delay information to be reported from UE to gNB considers PSDB to be exhausted, instead of considering the duration of data buffered in AS layer.

In the following, we discuss how to realize the remaining time considering PSDB to be exhausted.

According to the definitions in [2], a PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services). Normally, different frames or video slices are generated at different timing. Thus, each application data unit (e.g. a frame or video slice) should be associated with one time information. In this principle, all PDUs within a PDU set should be associated with the same time information (i.e. time for start of the PDU set). Thus, we think the remaining time for the PDU set can be determined based on the PSDB and the time information associated with the PDU set. Based on the time information common for all PDUs within the PDU set, the remaining time for the buffered data associated with the same PDU set can be consistent.

- All PDUs within a PDU set for carrying one application data unit are associated with the same time information.

- If remaining time is determined based on PSDB and time information associated with PDU set, the remaining time is consistent for all PDUs within the same PDU set.

We think it should be feasible to consider for upper layer to provide time information associated with PDU set to AS layer for determining the corresponding remaining time. Thus, we propose:

- RAN2 assume that upper layer tags each PDU of a PDU set with a time information associated with the PDU set when delivering each PDU of the PDU set to lower layer.

- The remaining time for a PDU set in the delay information to be reported from UE to gNB is determined based on PSDB and time information associated with the PDU set.

Support of XR considers data burst which is a set of PDUs from one or more PDU sets generated and sent by the application within a short period of time. Therefore, the amount of buffered data in the delay information can contain those PDUs from one or more PDU sets associated with the same or similar remaining time.

- In the delay information to be reported from UE to gNB, each amount of buffered data corresponds to one or more remaining times.

3 	Conclusion

In this contribution, we observe and propose:

- According to TR 23.700-60 [1], PDU Set Delay Budget is also applied to UL PDU set.

- It cannot guarantee upper layers always deliver all PDUs of an UL PDU set to AS layer at a time.

- A remaining PDU of a PDU set may lately arrive at AS layer that would cause requirement of the PSDB for the PDU set cannot be satisfied.

- If the remaining time in the delay information to be reported from UE to gNB is determined based on duration of data buffered in AS layer, it cannot accurately reflect the PSDB to be exhausted.

- All PDUs within a PDU set for carrying one application data unit are associated with the same time information.

- If remaining time is determined based on PSDB and time information associated with PDU set, the remaining time is consistent for all PDUs within the same PDU set.

- RAN2 clarifies that the remaining time in delay information to be reported from UE to gNB considers PSDB to be exhausted, instead of considering the duration of data buffered in AS layer.

- RAN2 assume that upper layer tags each PDU of a PDU set with a time information associated with the PDU set when delivering each PDU of the PDU set to lower layer.

- The remaining time for a PDU set in the delay information to be reported from UE to gNB is determined based on PSDB and time information associated with the PDU set.

- In the delay information to be reported from UE to gNB, each amount of buffered data corresponds to one or more remaining times.

4 	References

[1] TR 23.700-60 v18.0.0, Study on XR (Extended Reality) and media services (Release 18).

[2] TR 38.835 v.1.0.0, Study on XR enhancements for NR (Release 18).