3GPP TSG-RAN WG2 Meeting #121	R2-2301537

Athens, Greece, 27th February – 3th March 2023

Agenda Item:		8.7.4.2

Source:				ASUSTeK

Title:		Discussion on handover enhancement with common signalling

Document for: 	Discussion and Decision

- Introduction

In RAN2#119bis meeting, it is agreed to consider common signalling for handover enhancements in NTN.

Agreements

- RAN2 can further consider whether some information in the handover command that can be common to all UEs, can be delivered to UEs in common signalling and if there is real benefit (in terms of signalling overhead reduction) in this

- Send an LS to RAN1 (cc RAN4) listing the scenarios (intra-satellite, inter-satellite with same or different feeder links) and check with RAN1 in which scenarios RACH-less is possible (with no indication of RAN2 preference)

In this contribution, we further discuss the common configuration for handover in NTN.

- Discussion

The cell size of NTN is larger than the cell size of TN and the earth-moving cell would keep moving on the ground. As a result, there would be a large number of UEs needed to perform handover in a NTN cell. Based on TR 38.821 [1], a UE served by a LEO cell of diameter 50 km and 1000 km may need to perform handover per 6.61s and 132.38s respectively. The frequent handover may result in signalling overhead and impact power consumption.

In addition, some configurations of target cell are cell-specific or group-specific, and these configurations could be shared by multiple UEs which handover from the source cell to the same target cell. When there are a large number of UEs performing handover to the same target cell, the network is not needed to transmit the configurations to each UE individually. It is beneficial that these information can be reduced in the dedicated signalling for handover.

The shared configuration for handover could be provided in a system information of the source cell. The network could broadcast the shared configuration including at least servingCellConfigCommon to the UEs. Then the network could omit the shared configuration (e.g. servingCellConfigCommon) in each dedicated handover command. The UE could use the shared configuration from the system information if the configuration is not provided in the handover command.

Proposal 1: Some shared configurations including servingCellConfigCommon for the target cell can be provided in a system information and be omitted in the dedicated handover command.

Proposal 2: The UE could use shared configurations (e.g. servingCellConfigCommon) for target cell from a system information of source cell if the configurations are not provided in the handover command.

On the other hand, for handover without feeder link switching or for intra-satellite handover, some configurations may remain the same when a UE move from the source cell to the target cell. The satellite information would not be changed for the case that the source cell and target cell are served by the same satellite. The network configuration would not be changed for the case that the source cell and target cell are served by the same gNB. To reduce signalling overhead, the same configuration between the source cell and target cell could be further reduced in the shared configuration. The same configuration between the source cell and target cell could be omitted in shared configuration for handover. The UE could reuse a configuration of source cell for the target cell, if the configuration is not provided in the system information including shared configuration.

Proposal 3: The same configurations between source cell and target cell could be omitted in the shared configuration.

Proposal 4: The UE can reuse some configurations of source cell for the target cell if the configurations are not provided in the system information including shared configuration.

- Conclusion

In this contribution, we discuss the shared configuration for handover in NTN and have following proposals:

Proposal 1: Some shared configurations including servingCellConfigCommon for the target cell can be provided in a system information and be omitted in the dedicated handover command.

Proposal 2: The UE could use shared configurations (e.g. servingCellConfigCommon) for target cell from a system information of source cell if the configurations are not provided in the handover command.

Proposal 3: The same configurations between source cell and target cell could be omitted in the shared configuration.

Proposal 4: The UE can reuse some configurations of source cell for the target cell if the configurations are not provided in the system information including shared configuration.

- Reference

- TR 38.821 V16.1.0 Solutions for NR to support NTN.