3GPP TSG-RAN WG2 Meeting #121	R2-2301554

Athens, Greece, 27 February – 03 March 2023

Agenda item:	8.9.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Multipath sidelink relay

WID/SID:	NR_SL_relay_enh-Core

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#120, RAN2 made a conclusion that the study part of the multipath relay is complete and the objectives of the work item is updated [RP-223501] to proceed normative work accordingly.

This contribution continues the discussion on multi-path relay including a couple of new issues as well as the FFS parts from the last RAN2 meeting.

# 2	Discussions

## 2.1	Split operation

In DC, for a split DRB, split operation is performed with the primary RLC entity as follows:

- PDCP control PDU is fixed to be sent over the primary RLC entity.

- PDCP data volume controls the transmission path of the PDCP PDU, i.e., to send PDCP PDU only to the primary RLC entity if data volume is less than a threshold.

In RAN2#120, RAN2 agreed not to duplicate PDCP control PDU for MP split RB as in DC. However, it is still open, e.g., to which RLC entity to send the non-duplicated PDCP control PDU. Also, it is not yet discussed whether to rely on data volume to determine a path to which PDCP PDU is delivered. Both issues are related to the question whether we need a concept of primary RLC entity.

In DC, the secondary cell can be configured by considering the link quality and the traffic load of the secondary cell so that the secondary cell serves the UE for a certain amount of time, i.e., not to serve the UE just for a short time. As a result, the gNB can distribute the traffic load from a long/mid-term perspective, for which RRC reconfiguration is sufficient.

In multi-path (MP) sidelink relay, however, the relay UE has its own traffic and mobility. It means that the relay UE may not be available unpredictably due to e.g., sudden explosion of its own data or moving away. Therefore, the remote UE should utilize the resources of both the relay UE and the remote UE as much as possible. Transmission path control based on data volume, in MP sidelink relay, only reduces the efficiency because it restricts the use of both paths to the transmission of large amount of data, implying that only one path is used even when the relay UE is configured to cope with the coverage or link quality issues immediately.

Considering scenario 2, the main difference is that the indirect path includes a non-3GPP – and assumed ideal – link between the remote UE and the relay UE. This difference does not introduce any reason to specify an RLC primary entity or a threshold for PDCP and RLC data volume for an MP split RB.

Therefore, it would be logical to use both links as a baseline if duplication is not activated for an MP split RB.

Proposal 1: For MP split RB in scenario 1 and scenario2, PDCP control PDU is transmitted to either of the RLC entities, i.e., not fixed to one RLC entity assigned as the primary RLC entity.

Proposal 2: For MP split RB in scenario 1 and scenario2, PDCP data PDU is transmitted to either of the RLC entities regardless of the total amount of PDCP and RLC data volume, i.e., no need of introducing the ul-DataSplitThreshold.

Proposal 3: For MP split RB in scenario 1 and scenario2, primary RLC entity is not configured.

If duplication is not activated for an MP split RB, the PDCP PDU will be delivered to either PC5 RLC or Uu RLC. Although it would be optimal to transmit the PDCP PDU to the best path, it may not be easy to define. For example:

- In general, it is considered that transmission over indirect path brings some delay because there are two links, i.e., PC5 and Uu, which requires separate scheduling and transmission steps. However, the indirect path can be more reliable because the relay UE will be selected based on at least RSRP and the distance matters. Therefore, it would be hard to say that one path is always better than the other.

- When MP is established, the UE will respond the RRCReconfigurationComplete message to the gNB. If split SRB1 is configured, the UE can respond it either via the path from which the RRCReconfiguration was received (existing path) or the newly established path. If the UE responds via the existing path, RRCReconfigurationComplete message will be correctly received by the gNB but it cannot guarantee that the newly added path is successfully established. If the UE responds via the newly added path, on the other hand, there is a risk that RRCReconfigurationComplete message is not correctly received by the gNB due to potential failure in the newly added path. This may be more critical if indirect path is a newly added path as it requires successful setup on both PC5 link and Uu link of relay UE side. So, it is also difficult to say that which path is the best to respond.

From the above examples, we see that defining an optimal rule for path selection is not easy, and the optimal rule may also depend on what message is sent. Therefore, it is better to leave it to the UE implementation rather than standardization.

Proposal 4 : For an MP split RB in scenario 1 and scenario 2, if both paths are available, it is up to UE implementation where to send the PDCP PDU.

## 2.2	Duplication operation

Currently, two different MAC CEs are supported for duplication, i.e., duplication activation/deactivation MAC CE and duplication RLC activation/deactivation MAC CE.

In DC, SRB duplication is supported but only activated/deactivated via RRC, and there can be more than two RLC entities for a split RB, which is the reason why we only have DRB ID field and three bits of RLC ID field in the MAC CE.

For an MP split RB, RAN2 already agreed to support SRB duplication. So, there are some questions like, is dynamic duplication activation/deactivation via MAC CE applied to SRB duplication? MAC CE based duplication allows fast adaptation of duplication behaviour according to e.g., link quality. So, duplication should also benefit SRB as well as DRB. Therefore, the MAC CE needs to indicate whether the duplication activation/deactivation is applied to SRB or DRB.

Proposal 5 : For MP sidelink relay, activation/deactivation of SRB duplication is controlled by MAC CE.

## 2.3	Handling of RLF

In RAN2#119bis and RAN2#120, RAN2 agreed:

Proposal 12	[21/21] (modified) When UE operating in multi-path Relay, it performs RLM for Uu interface, for Scenario-1 and Scenario-2. For PC5 interface in Scenario-1, it performs sidelink RLF detection based on Rel-16 V2X specification [20/21]. For UE-UE link in Scenario-2, whether/how to have failure detection is out of 3GPP scope.

Agreement:

Proposal 17	[Easy] Upon detection of 3GPP-defined RLF failure in one path, remote UE (configured with MP) can report path failure via the alternative available path if SRB1 is configured on the alternative path or split SRB1 is configured.

What the RAN2 has not yet agreed in RAN2#120 is:

Proposal 18	[RAN2 to discuss] Whether to initiate RRC re-establishment if RLF is detected on a path carrying non-split SRB, or if RLF is detected on a path and alternative path with split SRB is suspended.

Although majority companies consider that it is logical to initiate RRC connection re-establishment if RLF is detected on a path carrying non-split SRB, or alternative path with split SRB is suspended, there was a concern from a couple of companies that this would not be aligned with the legacy behavior.

An example case would be that non-split SRB1 is configured on an indirect path and RLF is detected on the direct path where PCell exists. In this case, the above statement implies that the remote UE initiates RRC connection re-establishment even if the path with PCell is of no problem. However, in this case there is no way but to initiate RRC connection re-establishment because SRB1 is non-split RB on the problematic link. In addition, the gNB can configure the SRB1 as a split RB or a non-split RB on direct path if the gNB wants to avoid this situation. So, we don’t see any technical reason that prevents RRC connection re-establishment in this case.

Proposal 6 : When UE operating in multi-path relay, the UE initiates RRC connection re-establishment if RLF is detected on a path carrying non-split SRB, or if RLF is detected on a path and alternative path with split SRB is suspended.

When RLF is detected, it needs to be decided further what information is to be reported. In DC, FailureReportMCG includes failureType, e.g., T310 expiry, RA failure, RLC max retx, etc.

In MP sidelink relay, if indirect path fails, it is due to either PC5 link or Uu link of the relay UE. Thus, it may be beneficial to inform which link of the indirect path is problematic so that the gNB fixes the problematic link. In addition, when RLF is detected, the gNB may determine whether to continue the MP sidelink operation based on the link quality and/or the buffer size. Relying on the periodic or retransmission BSR may not be sufficient to receive the latest buffer size information at an appropriate time, which means we may need e.g., to trigger a BSR upon detection of RLF.

Proposal 7: RAN2 discuss what kind of information needs to be reported if RLF is detected and reported, e.g., buffer size, problematic link/interface, etc.

## 2.4	How to transit the target relay UE to RRC_CONNECTED from RRC IDLE/INACTIVE?

In RAN2#120, RAN2 agreed the followings:

Agreement:

RAN2 will downselect the solution for triggering IDLE/INACTIVE relay UE to enter CONNECTED state from:

-Option 1 (SL-RLC or UP-based approach (excluding SL-RLC1)),

-Option 3 (PC5-RRC approach)

-Option 4( RRCReconfigurationComplete-based approach),

Discovery/PC5-S-based solution can be further discussed if initiated from SA2.

Each option should be looked at carefully before continuing the discussion because the agreed options in RAN2#120 are slightly different from the original options discussed in RAN2#119bis.

- [Uu-RRC message-based approach] Option 1 and Option 4: They originally intend to initiate RRC Connection establishment based on RRCReconfigurationComplete (Uu-RRC) message. Uu-RRC message based approach seems to be assuming two different things on SRB configuration, i.e., how to split:

- Assumption 1: gNB ensures transmission of RRCReconfigurationComplete message via the indirect path (as in Rel-17). The first option is to configure a split SRB1 with duplication. The second option is to configure split SRB1 without duplication and allow SRB1 transmission over indirect path exceptionally even if duplication is not configured.

- Assumption 2: gNB may configure non-split SRB1 on direct path. A new method is required because the RRCReconfigurationComplete message cannot be delivered to the relay UE if non-split SRB1. It was proposed to introduce a new SL-RLC channel so that Uu-RRC message can be delivered to the relay UE or to introduce a new indicate which is a kind of PC5-RRC message.

- [PC5-RRC message-based approach] Option 3: The relay UE initiates RRC Connection establishment if RRCReconfigurationSidelink message is received over PC5, which remains the same between RAN2#119bis and RAN2#120.

In the meanwhile, UP-based approach in option 1 is not so clear and we found only two contributions on this:

- It was mentioned that UL data may trigger the transition of the relay UE, but it is not clear how the UL data can be delivered to the RRC INACTIVE/IDLE relay UE. In addition, delivery of UL data to the relay UE before the relay UE establishes its own RRC connection may lead to the longer delay of UP data transmission and back-and-forth UP data exchange between the remote UE and relay UE over PC5 link if the relay UE is not able to establish its own RRC connection with the gNB. [R2-2211752]

- It was mentioned that the remote UE can initiate an RRC connection at the relay UE when UL data transmission becomes available via the indirect path, but this is more about when to trigger the transition but not about how to trigger the transition. [R2-2211633]

Proposal 8: Exclude UP-based approach in option 1.

In Uu-RRC message-based approach, assumption 1 requires exceptional handling of SRB1, i.e., transmission of duplicate SRB1 without duplication configuration or it restricts the network’s configuration on SRB1, and assumption 2 requires additional solution to deliver Uu-RRC message to the relay UE if non-split SRB1.

In the meanwhile, PC5-RRC message-based approach allows the gNB to decide whether to configure the SRB1 as split or non-split by considering various conditions, e.g., latency or reliability, and the UE/gNB can maintain a single procedure to initiate RRC connection at the relay UE side regardless of the SRB1 configuration.

In our view, it is not beneficial to complicate the specification just to support one case, i.e., to initiate RRC connection for the RRC_IDLE or RRC_INACTIVE relay UE. It is simpler and cleaner to remove the dependency between the SRB1 configuration and how the relay UE initiates the RRC connection, which can be achieved by defining a separate procedure over PC5 based on PC5-RRC message.

One may argue that Rel-17 approach should be followed, but since only the indirect path exists in Rel-17, the Uu-RRC message must be delivered to the indirect path. The restriction doesn’t exist in Rel-18 MP SL relay. Therefore, we should target to specify the solution for relay UE to transit to the RRC_CONNECTED state even if Uu-RRC message cannot not be delivered via the indirect path.

Proposal 9: RAN2 develops PC5-RRC message based approach to transit the target relay UE in RRC_INACTIVE/IDLE to the RRC_CONNECTED mode.

The latency for the relay UE to transit to the RRC_CONNECTED leads to the delay in transmission of the SL data via indirect path or using multi-path. If multi-path establishment is triggered due to e.g., URLLC type of data transmission that cannot be achieved via direct path only, the indirect path needs to be setup as fast as possible and such delay is not desirable regardless of which approach is taken to trigger initiation of RRC connection establishment of the relay UE.

One possible improvement would be that the gNB sends the relay UE’s RRC connection establishment configuration, e.g., similar to RRCSetup, to the remote UE, e.g., as a container, in advance, and the remote UE forwards it to the relay UE when triggering initiation of the RRC connection to the relay UE side. Additionally, depending on geographical location of the remote/relay UE, the gNB may even make the relay UE to skip RA procedure by including TA information in the relay UE’s configuration, which gets the indirect path ready even faster.

Proposal 10: RAN2 discuss the possible enhancement on relay UE’s RRC connection establishment in the context of multi-path establishment via relay UE in RRC_IDLE/INACTIVE.

## 2.5	Path control of MP split RB

Having MP split RB, alternating two paths by, e.g., deactivating or activating one path, may not be good because use of multiple paths, in general, achieves good performance over the use of one path only. However, using one path more actively than the other path would be beneficial in terms of scheduling flexibility as well as link adaptation. For example, if CBR (Channel Busy Ratio) is low over the PC5 link, the gNB may want to boost the transmission over indirect path to improve the overall capacity. To boost the transmission over the indirect path, the gNB may want to minimize dynamic scheduling on a direct path.

To minimize the dynamic scheduling on the direct path, the UE may be configured with a CG resources on the direct path. Thus, activation of a CG can be considered as an indication that the gNB may not allocate dynamic grants on that path. In this case, the UE may only initiate unnecessary Random Access procedure after the consecutively ignored SR procedure unless the resource request procedure is suspended.

In MP relay, the gNB should be able to dynamically control which path to use more because the link quality/load may change dynamically. Therefore, we propose that RAN2 study a dynamic mechanism to boost transmission capacity of one link via dynamic scheduling or to limit the transmission capacity of one link via configured grant.

Proposal 11: For MP relay, RAN2 study a dynamic mechanism to boost transmission capacity or to limit the transmission capacity on one path.

# 3	Conclusion

In this contribution, we present our view on how the split/duplication works for MP split RB, how the RLF detection works, and how the relay UE should transit to RRC_CONNECTED and provide the following proposals:

Proposal 1: For MP split RB in scenario 1 and scenario2, PDCP control PDU is transmitted to either of the RLC entities, i.e., not fixed to one RLC entity assigned as the primary RLC entity.

Proposal 2: For MP split RB in scenario 1 and scenario2, PDCP data PDU is transmitted to either of the RLC entities regardless of the total amount of PDCP and RLC data volume, i.e., no need of introducing the ul-DataSplitThreshold.

Proposal 3: For MP split RB in scenario 1 and scenario2, primary RLC entity is not configured.

Proposal 4: For MP split RB in scenario 1 and scenario 2, if both paths are available, it is up to UE implementation where to send the PDCP PDU.

Proposal 5: For MP SL relay, activation/deactivation of SRB duplication is controlled by MAC CE.

Proposal 6: When UE operating in multi-path relay, the UE initiates RRC connection re-establishment if RLF is detected on a path carrying non-split SRB, or if RLF is detected on a path and alternative path with split SRB is suspended.

Proposal 7: RAN2 discuss what kind of information needs to be reported if RLF is detected and reported, e.g., buffer size, problematic link/interface, etc.

Proposal 8: Exclude UP-based approach in option 1.

Proposal 9: RAN2 develops PC5-RRC message based approach to transit the target relay UE in RRC INACTIVE/IDLE to the CONNECTED mode.

Proposal 10: RAN2 discuss the possible enhancement on relay UE’s RRC connection establishment in the context of multi-path establishment via relay UE in RRC_IDLE/INACTIVE.

Proposal 11: For MP relay, RAN2 study a dynamic mechanism to boost transmission capacity or to limit the transmission capacity on one path.