3GPP TSG-RAN WG2 Meeting #121                           R2-2301594

Athens, Greece, 27th Feb – 3rd Mar 2023

Agenda item:	8.11.2.2

Source:	TCL

Title:	Session state change for UEs receiving Multicast in RRC_INACTIVE state

Document for:	Discussion and Decision

- Introduction

At RANP#94-e meeting, a new RAN2 work item on ‘Enhancements of NR Multicast and Broadcast Services’ [1] was agreed. The goal of the work item is to further enhance the NR Multicast/Broadcast functions based on Rel -17 MBS. One of the objectives in the WID is mentioned as the following.

- Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3]

  - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2]

  - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3]

During R2#120e meeting, the following agreements have been reached.

    - When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases).

    - MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications.

    - We assume that the UE can only receive multicast service after it joined the session.

    - FFS whether MCCH configuration is initially provided to the UE via dedicated signalling.

In this contribution, we provides the discussion on the session state change for UEs receiving multicast in RRC_INACTIVE.

- Discussion

During the RAN2#120 meeting, it has been agreed that MCCH is used in case there is a need to indicate a PTM configuration, andthere is a need for change in PTM configuration or mobility beyond serving cell/gNB.  However the behaviour in case of session status change is left to be FFS.

It is clear that gNB will decide whether a multicast session can be received by UE(s) in inactive or not. However gNB should be provided some assistance information from 5GC, and/or UE. UE can inform gNB by providing its capability of receiving multicast session in inactive state and it preference of receiving multicast session in RRC active or inactive state. 5GC can provide some analytics information to gNB, such as the user data congestion analytics information, MBS session priority and UE session priority which have been discussed in SA2. MBS session priority denotes the priority level of an MBS session, and the priority level defines the relative importance of a MBS session. UE session priority denotes the priority level of a certain UE within a certain MBS session, and the priority level further defines the relative importance of a UE for an MBS session.

In our view, even though for the scenario that UE already has a valid PTM configuration, when a multicast session is activated, gNB might change the transmission mode (multicast session is received in inactive state or connected state). Since gNB makes the decision taking all of the assistance information into consideration, and the assistance information might change at any time. Therefore we propose that, when the multicast session is activated, UE should be indicated by gNB whether it can receive the multicast session in inactive state or not. Since group paging is used for indication of multicast session activation to the UE, it is obvious to indicate UE whether it can receive the multicast session in inactive state or not.

However when UE already has a valid PTM configuration and when the multicast session is activated, the transmission mode might stay the same. In this case there is no need for gNB to include such indication in group paging message. Therefore we propose that when the multicast session is activated, UE is indicated optionally by group paging whether it can receive the multicast session in RRC_INACTIVE or not.

Proposal 1: When the multicast session is activated, UE is indicated optionally by group paging whether it can receive the multicast session in RRC_INACTIVE or not.

During R2#120 meeting, it has been discussed that PTM configuration may apply to a certain area (i.e., a set of cells instead of a single cell). The views are diverse among the companies. Some companies think it is helpful in terms of UE power saving and signalling reduction and the opponents have the concerns that this is difficult to realize at the network side.

It seems companies agree that it is helpful anyway for UE in inactive state to be aware of whether the cells provide the same PTM configuration of a Multicast service and just have the concerns in terms of the complexity of implementation. Therefore simplified solution could be considered, such as a UE can be informed whether the neighbour cells have the same PTM configuration which wouldn’t bring too much complexity.

Proposal 2: Recommend RAN2 to introduce PTM configuration applicable area.

- Conclusion

Based on the above discussion, we recommend RAN2 to discuss and adopt the following proposal:

Proposal 1: When the multicast session is activated, UE is indicated optionally by group paging whether it can receive the multicast session in RRC_INACTIVE or not.

Proposal 2: Recommend RAN2 to introduce PTM configuration applicable area.

Reference

- RP-213568, New WID: Enhancements of NR Multicast and Broadcast Services, RAN Meeting #94-e.

- R2-2210068, Report of [Post119-e][610][eMBS] PTM configuration for INACTIVE