3GPP TSG-RAN WG2 Meeting #121 	 R2-221xxxx

Athens, Greece, 27th February – 3rd March, 2023

Agenda Item:    8.13.3

Source:         Xiaomi

Title:         	Considerations on the signaling based logged MDT override protection for E-UTRAN

Document for:	Discussion and Decision

Introduction

For the MDT override enhancement in inter-RAT scenario, RAN2 achieved the following agreements in the last meeting [1].

=> The scenario when the UE is configured with NR Signaling-based logged MDT measurement configuration and reselects to E-UTRAN is excluded in R18 scope.

=>	RAN2 will investigate UE and NW impacts due to EUTRA MDT configuration override protection in inter-RAT scenario realized by simultaneous LTE and NR configuration in the UE.

=> 	FFS if the extension of the LTE LoggedMeasurementConfiguration (with Logged MDT type indication) is needed.

=>	FFS Cross-RAT reporting for Logged MDT results (i.e. UE reports E-UTRAN logged MDT results in NR) is whether supported in R18.

=>	Intra-EUTRA case will not be considered.

In this contribution, we provide our considerations on the MDT override protection for the LTE signalling based logged MDT in the inter-RAT scenario.

Discussion

To achieve the MDT override protection for LTE signaling based logged MDT in inter-RAT scenario, there are some solutions raised in the previous discussion as follows: [2]

Solution 1: Override protection by simultaneous LTE and NR configuration

Solution 2: Override protection by cross-RAT signalling but no cross-RAT reporting of LTE logged MDT report

Solution 3: Override protection by cross-RAT reporting of LTE logged MDT report.

To enable the LTE signaling based logged MDT override protection in the inter-RAT scenario, anyway UE needs to get aware of whether the logged MDT type in LTE is signaling based logged MDT to decide whether to apply the override protection mechanism when UE reselects to NR. During the previous discussion, some companies commented it is not needed for solution1. However, from our view, even for solution 1, UE still needs to get aware of this as there is no need to have UE memory reserved for management based logged MDT to reduce the cost.

As there is no such network indication to achieve that according to TS 36.331, we propose to extend the LTE logged MDT configuration with the logged MDT type indication.

Proposal 1: Extend LTE LoggedMeasurementConfiguration with logged MDT type indication.

For solution 1, it has the benefits that it does not require inter-RAT interaction. However, as many companies commented in the previous discussion, it is against the existing MDT principle that UE keeps only a single MDT configuration, and it would result in heavy UE burden as it requires UE memory for maintaining two sets of logged MDT configuration for NR and LTE. For this issue, one potential solution is to have common memory allocation for LTE and NR. However, based on current spec as quoted below [3], it may cause failure to execute the NR logging as the reserved memory is full with the LTE logged MDT measurements.

5.1.1.2	Measurement collection

…

UE collects MDT measurements and continues logging according to the logged measurement configuration until UE memory reserved for MDT is full. In this case the UE stops logging, stops the log duration timer and starts the 48 hour timer.

Observation 1: Solution 1 violates the general principle of the logged MDT and would result in heavy burden on UE memory.

Observation 2: Sharing UE memory between the logged MDT in LTE and NR may cause failure to execute the NR logging as the reserved memory is full.

For solution 2 and solution 3, in the previous discussion, all companies agreed to support the cross-RAT signaling to indicate that UE is configured with LTE signaling based logged MDT while the controversial point is whether to support the cross-RAT reporting.

For this issue, if not support cross-RAT reporting, gNB may not configure a management based logged MDT before the LTE signaling based logged MDT report is retrieved. However, as the management based logged MDT is initiated without targeting a specific UE, it have minor impacts for the logged measurements collection if one UE cannot be configured with that. Besides, whether to configure the management based logged MDT depends on the network decision, even if the signaling based logged MDT report is not retrieved, network can still configure the management based logged MDT to override that.

Observation 3: Not supporting cross-RAT reporting has minor impacts on the management based logged MDT measurement collection.

If the cross-RAT reporting is supported, it enables the management based logged MDT to be configured immediately after retrieval. However, upon gNB fetched the LTE logged MDT report, it requires for signaling overhead to forwards the LTE logged MDT report to eNB or TCE or AMF which has impacts on RAN3 and needs to be further checked.

Observation 4: Supporting cross-RAT reporting introduces signaling overhead for report forwarding which has impacts on RAN3 and needs to be further checked.

Proposal 2: No need to support the cross-RAT reporting for Logged MDT results in R18.

Proposal 3: Solution 2 is considered for the LTE signaling based logged MDT override protection.

Conclusions

During the discussion above, we have the following observations and proposals:

Observation 1: Solution 1 violates the general principle of the logged MDT and would result in heavy burden on UE memory.

Observation 2: Sharing UE memory between the logged MDT in LTE and NR may cause failure to execute the NR logging as the reserved memory is full.

Observation 3: Not supporting cross-RAT reporting has minor impacts on the management based logged MDT measurement collection.

Observation 4: Supporting cross-RAT reporting introduces signaling overhead for report forwarding which has impacts on RAN3 and needs to be further checked.

Proposal 1: Extend LTE LoggedMeasurementConfiguration with logged MDT type indication.

Proposal 2: No need to support the cross-RAT reporting for LTE logged MDT results in R18.

Proposal 3: Solution 2 is considered for the LTE signaling based logged MDT override protection.

References

- R2-2209xxx Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online.

- draft R2-22xxxxx [801][R17 SON_MDT] MDT override solution direction (Nokia)_v9_Rapp

- 3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".