3GPP TSG-RAN WG2 Meeting #121	R2-2301634

Athens, Greece, February-March, 2022

Agenda Item:	8.16.2

Source:	AT&T

Title:	Protocol aspects of AI/ML framework for NR air interface

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Study Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], 3GPP has agreed to study the framework for AI/ML for air-interface corresponding to target use cases considering aspects such as performance, complexity, and potential specification aspects. Some of the aspects of the study item include RAN2-led objectives:

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

This contribution discusses the protocol aspects of an AI/ML framework applied to the NR air interface, specifically the impact on the RAN architecture and protocols.

# AI/ML framework for NR air interface

The 5G industry trends which enable network virtualization and deployment of low-latency/high bandwidth services are also making application of power Artificial Intelligence (AI) tools such as machine learning (ML) algorithms to 5G networks feasible and scalable.  These algorithms rely on historical data for deriving system models and training as well as real-time or near-real-time data collection to adapt to different network conditions. Furthermore, a variety of use cases can be supported by AI/ML techniques as noted in the SID including CSI feedback optimization, beam management, and positioning. Different use cases can have vastly different requirements in terms of the impact on network nodes or functionalities. This implies that the appropriate implementation of different AI/ML techniques may involve multiple interfaces, signalling procedures, and processing requirements (including requirements on data aggregation or co-location with different nodes/functions).

RAN2 should study how the life cycle management (LCM) for a given AI/ML model can be supported by existing Uu signaling and procedures and if any enhancements are needed, specifically in the areas of model delivery/updates, data collection, and model monitoring.

Model Delivery

In case model delivery is provided explicitly by 3GPP signaling, the network should inform UEs whether a model is available for delivery and/or whether it has been updated if provided previously. During RAN2#120 the following agreements were made:

For model transfer/delivery for AI/ML models (for the target use cases of this SI), RAN2 to study CP-based, UP-based solutions

Depending on the use case and how frequently training information is updated, this model delivery may be very infrequent, or may be updated based network and user conditions which dynamically change, necessitating an on-demand mechanism. In addition, the selection of user plane or control plane methods for model delivery may also depend on the size of the model, and the underlying architecture used for model training, testing, and deployment may influence which procedure is most suitable for model delivery and subsequent updates. However, in either case, the network needs to be able to understand (and potentially control) when a model is available to be delivered or updated, in order to optimize the delivery method as well as support model monitoring and triggering activation/deactivation procedures.

Proposal 1: For AI/ML use case/approaches which support either UP-based or CP-based delivery methods, RAN2 should study both periodic (e.g., SIB-based) as well as on-demand (e.g., UE-requested and/or via dedicated signaling) model availability and update information.

Model Activation/Deactivation

In case of UP-based or CP-based delivery methods, the network should have full ability to manage the activation/deactivation or model selection/fallback if supported for a given use case. While it is expected that applying a machine learning model at either the UE or gNB will result in improved performance, if the network or environmental conditions change quickly or drift beyond the training data of the model sufficiently, this may not be guaranteed. As a result, it is critical for the network to have full visibility into the LCM of the model at the UE/gNB and control of configuring any associated explicit and/or event/condition-based triggers.

Proposal 2: For AI/ML use case/approaches which support UP-based and CP-based LCM procedures, RAN2 should study both network triggered and event/condition-based mechanisms at the gNB and UE for model selection, activation, deactivation, switching, and fallback.

Model Training/Data Collection

In order to support data collection for AI/ML model training, extensive data from UEs and/or gNBs may be transferred either within the RAN or to a dedicated location for processing and model training (e.g. OAM). Given that models will continue to be refined over time, different approaches may require different types of information with different granularities/data collection intervals (even for the same use case). As a result, existing air interface measurement collection frameworks (including MDT) are not optimized for AI/ML applications since they lack the flexibility to adapt to different scenarios and use cases and would require either significant over-collection and subsequent filtering/processing at the device/data collection entity to extract the relevant information, or require frequent and almost dynamic parameter reconfigurations to ensure that relevant data is obtained when a scenario of interest is either recreated during testing or observed in the field for model retraining. As a result, RAN2 should study the drawbacks of existing data collection procedures and define requirements which are optimized for existing AI/ML approaches and can be extended to future use cases.

Proposal 3: RAN2 should consider a new data collection framework for supporting AI/ML use cases. Requirements for the data collection framework should include data collection for offline training as well as model monitoring (e.g. for model LCM including retraining). The framework should include procedures with network-configurable parameters for defining the time duration of the data as well as flexible selection of data type/format.

# Conclusion

In this contribution, we discussed the general framework of AI/ML applied to the NR air interface. The following proposals were made:

Proposal 1: For AI/ML use case/approaches which support either UP-based or CP-based delivery methods, RAN2 should study both periodic (e.g., SIB-based) as well as on-demand (e.g., UE-requested and/or via dedicated signaling) model availability and update information.

Proposal 2: For AI/ML use case/approaches which support UP-based and CP-based LCM procedures, RAN2 should study both network triggered and event/condition-based mechanisms at the gNB and UE for model selection, activation, deactivation, switching, and fallback.

Proposal 3: RAN2 should consider a new data collection framework for supporting AI/ML use cases. Requirements for the data collection framework should include data collection for offline training as well as model monitoring (e.g. for model LCM including retraining). The framework should include procedures with network-configurable parameters for defining the time duration of the data as well as flexible selection of data type/format.

# Reference

[1] RP-213599 Study on AI/ML for Air-Interface.