3GPP TSG RAN WG2 #121                                            R2-2301647

Althens, Greece, 27th February – 3rd March 2023

Agenda Item	: 8.5.2.3 (FS_NR_XR_enh)

Source	: LG Electronics Inc.

Title	: Discussion on the discard for XR

Document for	: Discussion and Decision

Introduction

In this contribution, we show our views on the discard operation for XR.

Discussion

Packet discard based on the PSDB and PSIHI

According to TR 23.700-60, a PDU set concept, a PSDB for PDU set, and PSIHI are introduced for XR traffic. More specifically, the PDU set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level, and the PSDB represents the QoS requirements of PDU set. In addition, the PSIHI indicates whether all PDUs are needed for the usage of the PDU set by the application layer in the receiver side.

For example, if the PSDB is exceeded for a PDU set and the PSIHI is indicated for the PDU set, all PDUs associated with the PDU set are considered as outdated, and thus do not need to be transmitted.

Observation 1. If the PSDB is exceeded for the PDU set and the PSIHI is indicated for the PDU set, all PDUs associated with the PDU set do not need to be transmitted.

In RAN2#119bis and RAN2#120, the following agreements were made.

RAN2#119bis e-meeting For UE transmitter, the PDCP discard should be performed per PDU set basis. For UE transmitter, The PDCP discard is managed per SDU for PDU set, the PDCP entity discards all PDCP SDUs associated with the PDU set RAN2#120 meeting RAN2 to support timer-based discarding of UL transmit side of PDCP PDU/SDUs of a PDU set. FFS how this is modelled in PDCP specification, can be discussed in WI phase.

Considering the observation 1 and the above agreements, we should discuss how to apply the PSDB in the PDCP. In order to apply the PSDB, we can consider the following two options.

- Option 1. Discard timer for a PDU set (new timer).

  - The PDCP entity configures a new timer for a PDU set based on the PSDB. The PDCP entity starts the new timer when the first PDCP SDU for a PDU set is received. If the new timer expires, the PDCP entity discards all PDCP SDUs associated with the PDU set.

- Option 2. Discard timer for a PDCP SDU (existing timer).

  - The PDCP entity configures the PDCP discardTimer for a PDCP SDU based on the PSDB. The PDCP entity starts the PDCP discardTimer for each PDCP SDU when it is received. If a PDCP discardTimer expires, the PDCP entity discards all PDCP SDUs associated with the PDU set even if the PDCP discardTimer is running.

In our view, both options are feasible. However, Option 2 can be easily implemented than Option 1 due to the reuse of the existing discardTimer. It would be better to reuse the legacy operation as much as possible.

In addition, Option 1 is performed only when the PSIHI is configured to the PDCP. If the PSIHI is not configured, the PDCP should use Option 2. There is no benefit to use two different mechanisms depending on the configuration of PSIHI.

Proposal 1: The PDCP discardTimer is configured for each PDCP SDU (same as legacy) with the value set to PSDB. If the PSIHI is configured and a PDCP discardTimer expires for a PDCP SDU, the PDCP entity discards all PDCP SDUs associated with the PDU set to which the PDCP SDU belongs.

Packet discard in congestion situation

In addition to the PDU set level discard operation based on the PDCP discardTimer, RAN2 may also need to discuss whether discarding should be considered in presence of congestion.

There may be a case where the PDU set with low importance arrives at PDCP before the PDU set with high importance arrives. Then, in network congestion situation, the network would provide very small UL grant to the UE, and the PDU set with high importance may be discarded (by discardTimer) due to the transmission of the PDU set with low importance. It severely degrades the user experience for XR service.

In addition, according to TR 23.700-60, it is already captured that the RAN may perform the PDU set discard based on the PDU set importance in presence of congestion. Since the PDU set with low importance, i.e., the P-frame and the B-frame, are decoded based on the PDU set with high importance, i.e., the I-frame, discarding only the PDU set with low importance at congestion could minimize the user experience impact.

In summary, in order to minimize the degradation of the user experience for the XR traffic, we think that it is worth to support PDU set discard based on the PDU set importance in network congestion situation in addition to the timer based PDCP discard.

Proposal 2. RAN2 support PDU set discard based on the PDU set importance in network congestion situation in addition to the timer based PDCP discard.

In the last meeting, RAN2 discussed whether it is feasible for RAN to estimate congestion per QoS flow and/or per DRB in UL without UE impacts. The conclusion is that it is feasible.

Considering that the UE does not know whether the network suffers from the congestion situation or not, the network should indicate the congestion situation to the UE. In addition, considering that the XR traffic can be transmitted per DRB level or QoS flow level, the network should indicate the congestion to a UE using SDAP control PDU or PDCP control PDU.

Proposal 3. Network indicates the congestion to a UE using SDAP control PDU or PDCP control PDU.

Conclusion

In this contribution, we show our views on the discard operation for XR.

Proposal 1: The PDCP discardTimer is configured for each PDCP SDU (same as legacy) with the value set to PSDB. If the PSIHI is configured and a PDCP discardTimer expires for a PDCP SDU, the PDCP entity discards all PDCP SDUs associated with the PDU set to which the PDCP SDU belongs.

Proposal 2. RAN2 support PDU set discard based on the PDU set importance in network congestion situation in addition to the timer based PDCP discard.

Proposal 3. Network indicates the congestion to a UE using SDAP control PDU or PDCP control PDU.