3GPP TSG-RAN WG2 Meeting #121 	R2-2301673

Athens, Greece, Feb. 27th – Mar. 3th, 2023

Agenda item:	8.17.2

Source:	Sharp

Title:	Capability Restriction for eMUSIM

WID/SID:	NR_DualTxRx_MUSIM-Core

Document for:	Discussion and Decision

1	Introduction

In the Rel-18 eMUSIM WID [1], one of the objectives for eMUSIM is:

Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4].

- Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose

- RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR.

Applicable UE architecture: Dual-RX/Dual-TX UE

In RAN2#119bis meeting [2], RAN2 agreed aims to address at least the following scenario:

RAN2 aims to address at least the Scenario 1: the UE in network A in RRC_CONNECTED indicates (i.e. adds/removes) its preference on temporary UE capability due start/stop connection in NW B. This can be e.g. CA/DC capability restriction.

A7: The UE can initiate signaling for UE capability restrictions on NW A if NW A allows it. The specification will not capture NW B events which can cause such need.

In this discussion paper, we provide solution for requesting restriction of UE capability and remove of the restriction.

2 Discussion

In this section, we proposed our solutions/considerations to the agreed scenario. In the scenario, if UE is in RRC_CONECTED in NW A and intents to start connection in Network B, UE needs to indicate its preference on temporary UE capability restriction to Network A.

For DC case, the restriction of UE capability can be fulfilled by deactivating or releasing of SCG. UE assistance information message for SCG deactivation has already supported for UE power saving purpose. Similar mechanism can be defined for eMUSIM purpose. And it should be up to network implementation to deactivate or release the SCG.

If UE is in RRC_CONECTED in both NW A and NW B and has stopped connection in NW B, UE needs to indicate its preference on remove of the restriction on temporary UE capability to Network A to improve the resource efficiency in NW A.  Same as the case of start RRC connection to NW B, UE assistance information message can also be used for the case of remove of the restriction of UE capability.

So, for DC case, UE assistance information message can be used to indicate its preference on temporary UE capability restriction or remove of the restriction to Network A.

Proposal 1: For DC, UE assistance information message is used to indicate its preference on temporary UE capability restriction or remove of the restriction to Network A.

In addition, is UE assistance information message used to indicate its preference on temporary UE capability restriction or remove of the restriction to Network A delivered to MN or SN? If SRB3 is configured, we can see some benefits on delivering the UAI message to SN via SRB3, for example, for the case that the UL grant is not available in MCG but available in SCG. Otherwise, UE needs to ask for UL grant from MCG which may delay the SCG deactivation procedure. But it should up to UE implementation about whether SRB1 or SRB3 to be used. If the SRB3 is not configured, we cannot see any benefit if UE deliver the UAI message to SN via MN.

Proposal 2: if SRB3 is configured, it can be used to deliver the UAI message to indicate its preference on temporary UE capability restriction to NWA. Otherwise, SRB1 is used to deliver the UE assistance information message to indicate its preference on temporary UE capability restriction to NWA. It is up to UE implementation whether SRB1 or SRB3 is used for the UAI message.

For CA case, considering that SCell activation/deactivation is handled by MAC entity, it is reasonable that MAC CE is used for UE to indicate it preference on the SCells state to NW A for the purpose of eMUSIM. If UE is in RRC_CONECTED in NW A and intents to start connection in NW B, UE indicates SCells deactivation in MAC CE to request deactivation of SCells. And if UE is in RRC_CONNECTED in both NW A and NW B initially and has stopped connection in NW B, UE indicates its preference on no more need of SCells deactivation in MAC CE to Network A.

Proposal 3: For CA, MAC CE is used to indicate its preference on temporary UE capability restriction or remove of the restriction to Network A.

3 Summary

In this section, we summarized our proposal as showed below:

Proposal 1: For DC, UE assistance information message is used to indicate its preference on temporary UE capability restriction or remove of the restriction to Network A.

Proposal 2: if SRB3 is configured, it can be used to deliver the UAI message to indicate its preference on temporary UE capability restriction to NWA. Otherwise, SRB1 is used to deliver the UE assistance information message to indicate its preference on temporary UE capability restriction to NWA. It is up to UE implementation whether SRB1 or SRB3 is used for the UAI message.

Proposal 3: For CA, MAC CE is used to indicate its preference on temporary UE capability restriction or remove of the restriction to Network A.

4 References

[1] RP-213584 New WID on Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR.

[2] RAN2#119bis chairman’s notes.