3GPP TSG-RAN WG2 Meeting #121               	R2-2301678

Athens, Greece, 27th Feb– 3rd Mar 2023

Agenda Item:	8.8.5

Source: 	vivo

Title:         	UAV identification broadcast

Document for: 	Discussion and Decision

- Introduction

One of the objectives in this WI consists of Study and specify, if needed, enhancements to NR PC5 and LTE PC5 to support UAV identification broadcast [1], as shown below:

1. Specify the following enhancements on measurement reports [RAN2]: - UE-triggered measurement report based on configured height thresholds - Reporting of height, location and speed in measurement report - Flight path reporting - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible. 2. Specify the signaling to support subscription-based aerial-UE identification [RAN3/SA2 interaction/RAN2] Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible. 3. Study and specify, if needed, enhancements to NR PC5 and LTE PC5 to support UAV identification broadcast [RAN2]. Check at RAN#99 for further elaboration of this objective, based on RAN2 discussions and SA2 development. Note: This objective is not intended to introduce new band(s) with support for the PC5 interface. UAV use of LTE PC5 or NR PC5 is not addressing any operator licensed band. 4. Study UE capability signaling to indicate UAV beamforming capabilities and, if necessary, RRC signaling [RAN1, RAN2]: - FR1 with directional antenna at UE side

This contribution discusses UAV identification broadcast related issues for NR UAV.

- Discussion

The use for broadcast remote identification by a UAV to aid operation by broadcasting a unique UAV ID is considered by SA2. This broadcast information can be used to identify a UAV using the assigned ID. Further, this ID can be recognized by the network or third party for UAV tracking and Detect And Avoid purposes (DAA). The DDA function can be used among different UAVs to avoid collision between UAVs. Thus, RAN2 can consider the support of UAV identification broadcast between UAVs.

RAN2 to consider UAV identification broadcast solution.

As SA2 is considering Broadcast Remote ID using PC5, RAN2 solutions for UAV broadcast may consider enhancements to NR PC5 and LTE PC5 to support UAV identification broadcast. RAN2 can first consider UAV identification broadcast solutions for NR, first, then further discuss whether to extend these solutions to LTE.

Therefore,

As a baseline, UAV identification broadcast solutions target NR. FFS whether to extend UAV identification broadcast solutions to LTE.

For UAV identification broadcast, some UAV assistance information such as, flight info, moving velocity, etc., can be broadcasted on PC5.

Therefore,

RAN2 to discuss information, e.g., UAV flight info, moving velocity, etc., to be broadcast for UAV identification on PC5.

- Conclusion

In this paper, the following proposal are given:

- RAN2 to consider UAV identification broadcast solution.

As a baseline, UAV identification broadcast solutions target NR. FFS whether to extend UAV identification broadcast solutions to LTE.

RAN2 to discuss information, e.g., UAV flight info, moving velocity, etc., to be broadcast for UAV identification on PC5.

- References

[1] RP-2 23545, New WID on NR support for UAV (Uncrewed Aerial Vehicles), Nokia.