3GPP TSG-RAN WG2 Meeting #121	                                                        R2-2301689

Athens, Greece, 27th February – 3th March, 2023

Source:	CATT

Title:	Discussion on the CG-SDT configuration for RedCap UE

Agenda Item:	6.8.2

Document for:	Discussion and Decision

Introduction

The issue of SDT operation in RedCap BWP without SSB was discussed in last meeting [1], without any agreement:

R2-2213022	[offline-109] CG-SDT support	ZTE Corporation	discussion	Rel-17	NR_redcap-Core

- VDF thinks CG-SDT is for small amount of data for stationary UEs and thinks there is no harm to provide the configuration for CG-SDT even if there is no SSB in the BWP. So wonders if there is any disadvantage with option 3

- Apple prefers option 2 but at least we should honour the agreement (no option 3)

- ZTE clarifies that the main change in the spec is to indicate that the UE uses the NCD-SSB for CG-SDT in this case

- QC thinks we have a plenary agreement not to use NCD-SSB for non-connected mode in Rel-17

- Vivo has a different understanding on the RAN plenary agreement, as it was referring to paging, reselection only. ZTE agrees.

- Nokia agrees with QC and thinks we can rule out option 2 and maybe we can consider option 3, as for RA-SDT.

- Samsung does not support option 2 and is open to support option 3.

- MTK agrees with Apple. MTK thinks we can’t assume that the UE can perform UL tx with no reference signals

- LG agrees to rule out option 2 and thinks only option 1 is acceptable

- Ericsson thinks option 3 should be supported

- Intel would like to postpone this discussion.

- HW thinks that to understand the impact of option 2 we need to clarify the spec impact. For option 3 we anyway need a UE capability. HW prefers to postpone this discussion.

- ZTE cannot accept option1, as it rules out a valid use case.

- Continue in the next meeting (all 3 options are still on the table).

In this contribution, we give some discussion on this issue, and give our proposal.

Discussion

In [1], the following three options are listed with the Pros and Cons analysis:

Option 1: For CG-SDT purpose, RAN2 has basic assumption that CD-SSB will be configured in initial BWP with CG-SDT (i.e. CG-SDT can only be configured if the BWP has CD-SSB) - Pros: no UE impacts - Cons: Restrictive (excludes the combination of REDCAP initial BWP without CD-SSB + CG-SDT) Option 2: For CG-SDT purpose, RAN2 has basic assumption that NCD-SSB will be configured (if there is no CD-SSB) in initial BWP with CG-SDT. - Pros: Allows the combination of REDCAP initial BWP without CD-SSB + CG-SDT - Cons: Needs some spec changes and may need a separate UE capability (?) and also RAN1 confirmation Option 3: Allow CG-SDT on an initial BWP without SSB and it is up to UE implementation how to handle this scenario (potentially with separate UE capability) - Pros: Allows the combination of REDCAP initial BWP without CD-SSB + CG-SDT at least in some specific scenarios. Needs no functional changes to the specs - Cons: Only supports some specific scenarios where the channel is stable during SDT. Needs UE implementation-based support

For CG-SDT, we have the assumption with some companies that, CG-SDT is mainly used for small amount of data by stationary UEs. For this case, UE doesn’t need to evaluate the channel quality during the short duration. So it is possible to provide the configuration for CG-SDT even if there is no SSB in the BWP. Otherwise, if a NCD-SSB is configured just for this target, specification impact will be introduced. So we think Option 3 should be used.

But for Option 3, separate UE capability, for example, to indicate whether UE support CG-SDT operation on a BWP without any SSB, is not needed. The network can just configure (by network implementation) CG-SDT resources on a BWP without any SSB to the UE which is stationary blindly.

Proposal： Allow CG-SDT on an initial BWP without SSB and it is up to network/UE implementation how to handle this scenario.

Conclusion

In this contribution, we give some discussion on SDT operation in RedCap BWP without SSB, and the following proposal is given:

Proposal： Allow CG-SDT on an initial BWP without SSB and it is up to network/UE implementation how to handle this scenario.

Reference

- R2-2213022 Offline discussion 109 summary (SDT on REDCAP BWP without SSB)