3GPP TSG-RAN WG2 #121	R2-2301699

Athens, Greece, Feb 27th – Mar 3rd, 2023

Agenda Item:	8.19.3

Source:	Ericsson

Title:	Discussion on initial BWP configuration for eRedCap UEs

Document for:	Discussion, Decision

# 1	Introduction

The discussion on enhanced support of reduced capability (eRedCap) devices WI (RP-223544) [1] has started in RAN2. The WID has the following objective on the introduction of early indication for UE baseband (BB) bandwidth reduction feature:

- Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2]

RAN2 discussed early indication in Rel-17 within the context of RedCap devices in general, but there are a few remaining aspects that require further discussion in Rel-18 within the context of eRedCap devices.

# 2	Initial BWP configuration for eRedCap UEs

RAN1 has made the following agreement (R1-2212982) [3] regarding initial BWP:

Agreement: For a cell supporting both Rel-17 and Rel-18 RedCap UEs, - The Rel-18 RedCap UEs can share the same separate initial DL/UL BWP as the Rel-17 RedCap UEs. - FFS: whether to support an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs

We do not foresee a need for capacity reasons to support an additional separate initial DL/UL BWP specific to Rel-18 eRedCap UEs. Currently, up to two initial BWPs can be configured in a cell:

- Normal initial BWP

  - Used by normal UEs

  - Can be used by Rel-17 RedCap UEs and Rel-18 eRedCap UEs

- Separate initial BWP

  - Can be used by Rel-17 RedCap UEs and Rel-18 eRedCap UEs

In our view, the question here is whether a configuration should be possible where Rel-17 RedCap UEs use the normal initial BWP and Rel-18 eRedCap UEs use the separate initial BWP, i.e., the following configuration:

- Normal initial BWP

  - Used by normal UEs

  - Used by Rel-17 RedCap UEs

- Separate initial BWP

  - Used by Rel-18 eRedCap UEs

We think there may be merits with such a configuration, but its necessity and feasibility may require some further discussion.

FFS: whether to support a configuration where Rel-17 RedCap UEs use the normal initial BWP and Rel-18 eRedCap UEs use the separate initial BWP.

# 3	Conclusion

Based on the discussion in the section above, we propose the following:

Proposal 1	FFS: whether to support a configuration where Rel-17 RedCap UEs use the normal initial BWP and Rel-18 eRedCap UEs use the separate initial BWP.

# References

RP-223544 “Revised WID on Enhanced support of reduced capability NR devices”, RAN#98e, Online, Dec 2022.

TR 38.875 “Study on support of reduced capability NR devices”, V17.0.0 (2021-03).

R1-2212982, “RAN1 agreements for Rel-18 NR RedCap”, RAN1#111, 14th – 18th Nov 2022.

R1-2212536, “RAN1 discussion on Rel-18 RedCap UE complexity reduction”, RAN1#111, 14th – 18th Nov 2022.

R1-2300500, “Further RedCap UE complexity reduction”, Ericsson, 3GPP TSG-RAN WG1 Meeting #112, 27th Feb – 3rd Mar 2023.