# 3GPP TSG RAN WG2 Meeting #120                     	     	    R2-2301705
Athens, Greece, 27 February – 3 March 2023

Agenda item:	8.15.2

Source:	Qualcomm Incorporated

Title:	Discussion on sidelink CAPC

WID/SID:	NR_SL_enh2 – Release 18

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss remaining issues of sidelink channel access priority class (SL CAPC).

# Discussion

## Confirm working assumptions

RAN2 made the following working assumptions on sidelink CAPC at RAN2 #120 meeting [1].

Agreements on SL CAPC mapping table:

2:	Working assumption

- Mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1. FFS on other SL CAPC mapping criterion.

- Mapping PQI 59/61 to CAPC priority class 3.

- Mapping PQI 25 to CAPC priority class 2.

- Mapping PQI 24/26/60 to CAPC priority class 1

Agreement on SL CAPC rules

1: 	Working assumption: If PQI-based CAPC mapping is agreed, as in NR-U, the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs.

Agreements on SL CAPC for RRC inactive/idle/OOC UE

2:	Working assumption: Use the CAPC of the standardized PQI or the CAPC of non-standardized PQI configured in SIB/pre-configuration which best matches the QoS characteristics of the current non-standardized PQI based on one or more QoS characteristics

Proposal 1. RAN2 confirms the following working assumptions.

Working assumption:

- Mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1. FFS on other SL CAPC mapping criterion.

- Mapping PQI 59/61 to CAPC priority class 3.

- Mapping PQI 25 to CAPC priority class 2.

- Mapping PQI 24/26/60 to CAPC priority class 1

Working assumption:

- If PQI-based CAPC mapping is agreed, as in NR-U, the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs.

Working assumption:

- Use the CAPC of the standardized PQI or the CAPC of non-standardized PQI configured in SIB/pre-configuration which best matches the QoS characteristics of the current non-standardized PQI based on one or more QoS characteristics.

## SL CAPC for logical channel multiplexing

For some sidelink communications, the PDB and priority may be defined differently.

For example, V2X services with PQI 56 and PQI 57 as shown in Table 5.4.4.4-1 below may be mapped with a high SL CAPC priority if only PDB is used for mapping the SL CAPC while the Priority Level is relatively low (e.g., Priority at 6 or 5 respectively as shown in the table).

For another example, delay critical ProSe services with PQI 92 and PQI 93 as shown in Table 5.6.1-1 below may be mapped with a very high SL CAPC priority if only PDB is used for mapping the SL CAPC while the Priority Level is relatively low (e.g., Priority at 5 or 6 respectively as shown in the table).

SL CAPC is used mainly for the fairness of accessing the channel, which may not reflect sidelink QoS requirement fully, e.g., Priority Level. However, the sidelink process is currently built on Priority Level for resource selection, preemption, transmission dropping, logical channel selection and multiplexing, etc. With possible confliction between SL CAPC priority (if mapped per PDB only) and the Priority Level for a sidelink communication, there is no simple solution. To avoid unnecessary spec impact, the logical channel multiplexing should consider the Priority only as currently specified, e.g., not considering the SL CAPC for logical channel multiplexing like NR-U.

Proposal 2. Keep the priority based logical channel multiplexing like NR-U.

TS 23.287 Table 5.4.4-1: Standardized PQI to QoS characteristics mapping [2]

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | GBR | 3 | 20 ms | 10-4 | N/A | 2000 ms | Platooning between UEs – Higher degree of automation. Platooning between UE and RSU – Higher degree of automation |
| 22 | (NOTE 1) | 4 | 50 ms | 10-2 | N/A | 2000 ms | Sensor sharing – higher degree of automation |
| 23 |  | 3 | 100 ms | 10-4 | N/A | 2000 ms | Information sharing for automated driving – between UEs or UE and RSU - higher degree of automation |
| 55 | Non-GBR | 3 | 10 ms | 10-4 | N/A | N/A | Cooperative lane change – higher degree of automation |
| 56 |  | 6 | 20 ms | 10-1 | N/A | N/A | Platooning informative exchange – low degree of automation; Platooning – information sharing with RSU |
| 57 |  | 5 | 25 ms | 10-1 | N/A | N/A | Cooperative lane change – lower degree of automation |
| 58 |  | 4 | 100 ms | 10-2 | N/A | N/A | Sensor information sharing – lower degree of automation |
| 59 |  | 6 | 500 ms | 10-1 | N/A | N/A | Platooning – reporting to an RSU |
| 90 | Delay Critical GBR | 3 | 10 ms | 10-4 | 2000 bytes | 2000 ms | Cooperative collision avoidance; Sensor sharing – Higher degree of automation; Video sharing – higher degree of automation |
| 91 | (NOTE 1) | 2 | 3 ms | 10-5 | 2000 bytes | 2000 ms | Emergency trajectory alignment; Sensor sharing – Higher degree of automation |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |

TS 23.304 Table 5.6.1-1: Standardized PQI values that are additionally defined to QoS characteristics mapping [3]

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | GBR (NOTE 1) | 1 | 150 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 25 |  | 2 | 200 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 26 |  | 2 | 200 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 60 | Non-GBR | 1 | 120 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signaling (e.g. MC-PTT signaling) |
| 61 |  | 6 | 400 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9 as specified in TS 23.501 [4]) |
| 92 | Delay Critical GBR (NOTE 1) | 5 | 5ms | 10-4 | 20000 bytes | 2000 ms | Interactive service - consume VR content with high compression rate via tethered VR headset (See TS 22.261 [6]) |
| 93 |  | 6 | 10ms | 10-4 | 20000 bytes | 2000 ms | interactive service - consume VR content with low compression rate via tethered VR headset; Gaming or Interactive Data Exchanging (See TS 22.261 [6]) |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we further discussed remaining issues of sidelink channel access priority class (SL CAPC) and concluded with the following proposals.

Proposal 1. RAN2 confirms the following working assumptions.

Working assumption:

- Mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1. FFS on other SL CAPC mapping criterion.

- Mapping PQI 59/61 to CAPC priority class 3.

- Mapping PQI 25 to CAPC priority class 2.

- Mapping PQI 24/26/60 to CAPC priority class 1

Working assumption:

- If PQI-based CAPC mapping is agreed, as in NR-U, the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs.

Working assumption:

- Use the CAPC of the standardized PQI or the CAPC of non-standardized PQI configured in SIB/pre-configuration which best matches the QoS characteristics of the current non-standardized PQI based on one or more QoS characteristics.

Proposal 2. Keep the priority based logical channel multiplexing like NR-U.

# References

[1] RAN2 NR SL-enh session chair final notes, “RAN2-120_R16_NR SL_R17&R18_NR SL enh_EoM”, RAN2 #120 Toulouse, France, November 14th – 18th, 2022.

[2] TS 23.287 “Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services (Release 17)”, Version 17.3.0 (2022-06).

[3] TS 23.304 “Proximity based Services (ProSe) in the 5G System (5GS) (Release 17)”, Version 17.3.0 (2022-06).