3GPP TSG-RAN WG2 Meeting #121	R2-2301707

27 February – 3 March, 2023

Source: 			ZTE Corporation, Sanechips

Title:           	Further Consideration on the IDC TDM Solutions

Agenda item:	8.10.3

Document for: 	Discussion and Decision

- Introduction

In the previous meeting, many agreements were achieved for the IDC TDM solutions. In the last meeting, a post meeting email discussion was also left to further discuss the details of the TDM enhancement, and the below preliminary proposals are given based on the companies’ inputs.

Proposal 1: With changing version to “UEAssistanceInformation-v18xy-IEs” ,TDM-AssistanceInfo-r18 to “SEQUENCE” and removing “periodicPatternInfo-r11”, UEAssistanceInformation of the ASN.1 framework and field description in section 4 for the periodic pattern is taken as starting point. Proposal 2: The NR values of long DRX cycle and start offset are used for periodic pattern. FFS short DRX cycle and 3.75ms cycle length Proposal 3: The slot offset with 1/32ms granularity is included in UEAssistanceInformation-v18xy-IEs for start offset. Proposal 4: Multiple periodic patterns for IDC are not supported in R18. Proposal 5: Per CG pattern is supported for EN-DC, FFS NR-DC. SN can configure the UE to report the TDM assistance information directly to SN, either through SRB 1 or SRB 3. Proposal 6: Postpone the text proposal of TDM assistance information signalling procedure, and discuss whether idc-AssistanceConfig-r18 for TDM assistant information allowing shall be added. Proposal 7: RAN2 to confirm which time unit (subframe or slot) is used for autonomous denial. FFS values of Validity period and number of Subframe or slot. Proposal 8: Take the text proposal in section 4 of autonomous denial as the baseline. Proposal 9: Send an LS to RAN4 including the agreements of TDM solution after online discussion.

In this paper, we share our further views on the above TDM issues.

- Discussion

In this chapter, we discuss the below TDM issues one by one.

- Issue 1: FFS short DRX cycle and 3.75ms cycle length

- Issue 2:Whether the slot offset with 1/32ms granularity is needed

- Issue 3: Whether per CG pattern for the NR-DC is needed?

The other FFS issue is about the values of the autonomous denial, obviously, these shall be determined by RAN4. Thus we focus on the Issue 1~3 in this paper.

- Issue 1: FFS short DRX cycle and 3.75ms cycle length

According to the companies’ inputs, the short DRX was mainly proposed to solve the BT eSCO case. The BT radio Tx/Rx occurs during BT slot duration of 0.625 ms with 3.75 ms period. In the LTE, the BT eSCO case can be solved by HARQ based TDM pattern. In NR, it has been agreed that the HARQ based TDM pattern would not be adopted, instead TDM periodic Gap based scheme would be specified. Thus, to solve this with TDM scheme, a TDM gap pattern with 3.75 ms period and 0.625ms duration can be supported.

Proposal 1: Besides the long DRX value, the TDM gap pattern with 3.75 ms period and 0.625ms duration can also be supported for the BT eSCO case. No other short DRX values are needed.

- Issue 2:Whether the slot offset with 1/32ms granularity is needed

Based on the above proposal 1, if the TDM gap pattern with 3.75 ms period and 0.625ms (20*1/32 ms) duration was supported, besides the slot offset, 1/32ms granularity would also be needed for the activeDuration.

Proposal 2: If proposal 1 was agreed, then the 1/32ms granularity would be needed for both the slot offset and the activeDuration.

- Issue 3: Whether per CG pattern for the NR-DC is needed?

According to the post email discussion inputs, companies think that if the SN can configure the IDC independently, then per CG gap would be needed.

Proposal 3: For the NR-DC, if the SN can configure the IDC independently, per CG gap pattern can be supported.

In the last meeting it has been agreed that the MN configure IDC shall be supported, by which the MN can configure the IDC candidate frequency list from both MN and SN, with this case, if per CG gap was supported, the UE need to distinguish which candidate frequency are from SN. Thus for this case whether per CG gap was supported would depends on the final solution of MN IDC configuration.

Observation 1: For the MN configured IDC, if per CG gap was supported, the UE need to distinguish which candidate frequency are from SN.

Proposal 4: For the MN configured IDC case, Ran2 to discuss whether per CG gap was supported after the MN configured IDC procedure was confirmed.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: Besides the long DRX value, the TDM gap pattern with 3.75 ms period and 0.625ms duration can also be supported for the BT eSCO case. No other short DRX values are needed.

Proposal 2: If proposal 1 was agreed, then the 1/32ms granularity would be needed for both the slot offset and the activeDuration.

Proposal 3: For the NR-DC, if the SN can configure the IDC independently, per CG gap pattern can be supported.

Observation 1: For the MN configured IDC, if per CG gap was supported, the UE need to distinguish which candidate frequency are from SN.

Proposal 4: For the MN configured IDC case, Ran2 to discuss whether per CG gap was supported after the MN configured IDC procedure was confirmed.