3GPP TSG-RAN WG2 Meeting #121	R2-2301718

# 27 February – 3 March, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the Simultaneous Rx-Tx

Agenda item:	5.1.3.2

Document for: 	Discussion and Decision

- Introduction

A LS [1] was received from RAN4 about the Simultaneous Rx-Tx for Band Pairs, in which it has discussed the following changes for CA and EN-DC.

Considering the fact that simultaneous Rx-Tx for overlapping TDD bands or TDD intra-band CA is not supported by UEs in general, RAN4 has discussed the following changes for CA, - simultaneousRxTxInterBandCA is indicated also if the UE supports simultaneous RxTx across all band entires of the BC except intra-band TDD and inter-band TDD-TDD band pairs of overlapping or partially overlapping TDD bands - simultaneousRxTxInterBandCAPerBandPair is not indicated - if the UE does not support simultaneous Rx-Tx for any band pair of the combination (no change) - if the UE indicates simultaneousRxTxInterBandCA unless the BC contains a TDD intra-band CA or an TDD-TDD inter-band CA overlapping or partially overlapping band pair for which the UE supports simultaneous Rx-Tx. and similarly for EN-DC, - simultaneousRxTxInterBandENDC is included also if the UE supports simultaneous RxTx across all band entires of the BC except intra-band TDD and inter-band TDD-TDD band pairs of overlapping or partially overlapping TDD bands within or across cell groups - simultaneousRxTxInterBandENDCPerBandPair is not indicated - if the UE does not support simultaneous Rx-Rx for any band pair of the combination (no change) - if the UE indicates simultaneousRxTxInterBandENDC unless the BC contains a TDD intra-band CA or an TDD-TDD inter-band CA overlapping or partially overlapping band pair for which the UE supports simultaneous RxTx within or across cell groups

In this paper, we share our further consideration on this LS.

- Discussion

In this chapter, we discuss this issue for the CA case first, then we analyze its impact to the EN-DC case.

2.1 CA Case

For the discussion convenience, the related CA BC can be divided into 3 types:

- Type 1: Inter-Band CA without any (partially) overlapping band pairs (only composed with non-overlapping band pairs);

- Type 2: Inter-Band CA with only (partially) overlapping band pairs;

- Type 3: Inter-Band CA with both (partially) overlapping band pairs and non-over lapping band pairs.

Obviously, this LS has no impact to the Type 1 BC. For the type 2, according to below Table 1, we can see that there is also no discrepancy between the RAN2’s spec and Ran4’s LS.

Table 1: RAN2/4’s Understanding to the Type 2

| Type 2 BC | According to RAN2 spec | According to RAN4 LS |
| --- | --- | --- |
| Case 1:None band pairs support simultaneousRxTx | simultaneousRxTxInterBandCA = absent simultaneousRxTxInterBandCAPerBandPair = absent | Same as RAN2 |
| Case 2: Part of band pairs support simultaneousRxTx | simultaneousRxTxInterBandCA = absent simultaneousRxTxInterBandCAPerBandPair = present with dedicated bits sent to 1 | Same as RAN2 |
| Case 3: All of band pairs support simultaneousRxTx | simultaneousRxTxInterBandCA = supported simultaneousRxTxInterBandCAPerBandPair = absent | Same as RAN2 |

Observation 1: The LS has no impact to the Type 1 and Type 2 BC.

- Type 1: Inter-Band CA without any (partially) overlapping band pairs (only composed with non-overlapping band pairs);

- Type 2: Inter-Band CA with only (partially) overlapping band pairs.

For the Type 3, it further includes 2 scenarios:

- Scenario 1: Part of or none of the non-overlapping band pairs support simultaneousRxTx.

- Scenario 2: All of the non-overlapping band pairs support simultaneousRxTx.

For the scenario 1, there is no discrepancy between the RAN2’s spec and Ran4’S LS, e.g.  simultaneousRxTxInterBandCA shall be set to absent while the simultaneousRxTxInterBandCAPerBandPair  shall be set to present with dedicated bits.

According to the LS, simultaneous Rx-Tx for overlapping TDD bands or TDD intra-band CA is not supported by UEs in general, based on this, there are 2 bullets that may cause discrepancy between RAN2 and RAN4:

- “simultaneousRxTxInterBandCA is indicated also if the UE supports simultaneous RxTx across all band entires of the BC except intra-band TDD and inter-band TDD-TDD band pairs of overlapping or partially overlapping TDD bands”.

- If the “the BC contains a TDD intra-band CA or an TDD-TDD inter-band CA overlapping or partially overlapping band pair for which the UE supports simultaneous Rx-Tx.” the UE shall report it with simultaneousRxTxInterBandCAPerBandPair.”

For better understanding, we can summarize the RAN2/4’s understanding on the type 3 scenario 2 as below:

Table 2: RAN2/4’s Understanding to the Type 3 (Scenario 2)

| Type 3 BC Scenario 2 | According to RAN2 spec | According to RAN4 LS |
| --- | --- | --- |
| Case 1:None of (partially) overlapping band pairs support simultaneousRxTx | simultaneousRxTxInterBandCA = absent simultaneousRxTxInterBandCAPerBandPair = present with dedicated bits sent to 1 | simultaneousRxTxInterBandCA = present simultaneousRxTxInterBandCAPerBandPair = absent |
| Case 2: Part of (partially) overlapping band pairs support simultaneousRxTx | simultaneousRxTxInterBandCA = absent simultaneousRxTxInterBandCAPerBandPair = present with dedicated bits sent to 1 | simultaneousRxTxInterBandCA = present simultaneousRxTxInterBandCAPerBandPair = present with dedicated bits sent to 1 |
| Case 3: All of (partially) overlapping band pairs support simultaneousRxTx | simultaneousRxTxInterBandCA = supported simultaneousRxTxInterBandCAPerBandPair = absent | Same as RAN2 |

Based on the Table 2, we can see that the main different is for the case 1, which can save the signaling bits by absence of the simultaneousRxTxInterBandCAPerBandPair for the case that None of (partially) overlapping band pairs support simultaneousRxTx. Considering that the simultaneousRxTxInterBandCAPerBandPair was first introduced in at R15.17.0 which may not be deployed in the real network or implemented by the UE, there may be no strong concern on the NBC issue, RAN2 can accept Ran4’s suggestion.

Proposal 1: For the Type 3 BC with all of the non-overlapping band pairs support simultaneousRxTx and none of (partially) overlapping band pairs support simultaneousRxTx, the UE can set simultaneousRxTxInterBandCA to present while set the simultaneousRxTxInterBandCAPerBandPair to absent.

For the case 2, from the signaling overhead aspect, there is no difference, to keep aligned with the case 1, RAN4’s suggestion can be accepted.

Proposal 1: For the Type 3 BC with all of the non-overlapping band pairs support simultaneousRxTx and none of (partially) overlapping band pairs support simultaneousRxTx, the UE can set simultaneousRxTxInterBandCA to be present while set the simultaneousRxTxInterBandCAPerBandPair to be absent.

Proposal 1a: For the Type 3 BC with all of the non-overlapping band pairs support simultaneousRxTx and part of (partially) overlapping band pairs support simultaneousRxTx, the UE can set simultaneousRxTxInterBandCA to be present and indicates the band pairs that support simultaneousRxTx with dedicated bits in the simultaneousRxTxInterBandCAPerBandPair.

Based on the above proposals, the CRs have also been provided in the [2][3][4].

Proposal 2: RAN2 to agree the CRs [2][3][4].

2.2 EN-DC case

For the EN-DC case, similar to the CA, there are 3 BC types and RAN2 can focus the discussion on the Type 3 BC with all of the non-overlapping band pairs support simultaneousRxTx.

- Type 1: Inter Band EN-DC BC without any (partially) overlapping EN-DC band pairs (only composed with non-overlapping band pairs);

- Type 2: Inter Band EN-DC BC with only (partially) overlapping band pairs;

- Type 3: Inter Band EN-DC BC with both (partially) overlapping band pairs and non-over lapping band pairs.

Similar to the CA case, there are 2 bullets that may cause discrepancy between RAN2 and RAN4:

- SimultaneousRxTxInterBandENDC is included also if the UE supports simultaneous RxTx across all band entires of the BC except intra-band TDD and inter-band TDD-TDD band pairs of overlapping or partially overlapping TDD bands within or across cell groups

- If the “the BC contains a TDD intra-band or an TDD-TDD inter-band EN-DC overlapping or partially overlapping band pair for which the UE supports simultaneous Rx-Tx.” the UE shall report it with simultaneousRxTxInterBandENDCPerBandPair .”

However, in the current field description of the SimultaneousRxTxInterBandENDC, it has been clearly clarified that it’s not applicable to the inter-band (NG)EN-DC/NE-DC combination with overlapping band pairs. This description was based on the RAN4’s LS [5], which is conflict with the above bullet 2. Thus an reply LS would be needed to clarify such case.

simultaneousRxTxInterBandENDC Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band (NG)EN-DC/NE-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-3 [4]. ... This capability is not applicable to the inter-band (NG)EN-DC/NE-DC combination, where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4]).

Proposal 3: RAN2 send an reply LS Ran4 to further confirm whether “ the simultaneousRxTxInterBandENDC would be applicable to the inter-band (NG)EN-DC/NE-DC combination, where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4])”.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: The LS has no impact to the Type 1 and Type 2 BC.

- Type 1: Inter-Band CA without any (partially) overlapping band pairs (only composed with non-overlapping band pairs);

- Type 2: Inter-Band CA with only (partially) overlapping band pairs.

Proposal 1: For the Type 3 BC with all of the non-overlapping band pairs support simultaneousRxTx and none of (partially) overlapping band pairs support simultaneousRxTx, the UE can set simultaneousRxTxInterBandCA to be present while set the simultaneousRxTxInterBandCAPerBandPair to be absent.

Proposal 1a: For the Type 3 BC with all of the non-overlapping band pairs support simultaneousRxTx and part of (partially) overlapping band pairs support simultaneousRxTx, the UE can set simultaneousRxTxInterBandCA to be present and indicates the band pairs that support simultaneousRxTx with dedicated bits in the simultaneousRxTxInterBandCAPerBandPair.

Proposal 2: RAN2 to agree the CRs [2][3][4].

Proposal 3: RAN2 send an reply LS Ran4 to further confirm whether “ the simultaneousRxTxInterBandENDC would be applicable to the inter-band (NG)EN-DC/NE-DC combination, where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4])”.

- Reference

- R4-2220520 LS to RAN2 on simultaneous Rx-Tx for band pairs of an advertised BC Ericsson RAN4

- R2-2301715	CR on Simultaneous Rx-Tx for Band Pairs_R15	38306 ZTE Corporation, Sanechips

- R2-2301716	CR on Simultaneous Rx-Tx for Band Pairs_R16	38306 ZTE Corporation, Sanechips

- R2-2301717	CR on Simultaneous Rx-Tx for Band Pairs_R17	38306 ZTE Corporation, Sanechips

- R4-210790  Reply LS to R2-2104550 on the intra-band and inter-band (NG)EN-DC or NE-DC Capabilities ZTE