3GPP TSG-RAN WG2 Meeting #121	R2-2301721

Athens, Greece, 27th Feb. – 3rd Mar., 2023

Agenda Item:		8.5.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on MAC enhancement for XR-specific capacity improvement

Document for:	Discussion and decision

1	Introduction

In previous RAN2 meetings [1] [2], the following agreements for XR-specific capacity improvement were reached.

RAN2#119bis e-meeting agreement: - 1: introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase). - Delay information consists of at least “remaining time”. - 2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase. - If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information. RAN2#120 meeting agreement: - RAN2 thinks we need one or more additional BSR table(s) for XR. FFS whether these are static (=specified) or dynamic (e.g. generated, differs according to some RRC parameter), can be discussed in WI phase. - RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay. - RAN2 needs to discuss additional BSR triggering conditions to allow timely availability of buffer status information at gNB. This can be discussed in WI phase.

And in RAN#98, a new Work Item on XR was approved [3] in which BSR enhancements including new BS table(s) and delay reporting were listed.

Specify the enhancements related to capacity: - Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1); - BSR enhancements including at least new BS Table(s); (RAN2); - Delay reporting of buffered data in uplink; (RAN2); - Provision of XR traffic assistance information for DL and UL (e.g. periodicity); (RAN2); - Discard operation of PDU Sets (RAN2);

In this contribution, we will discuss the following three issues related to BSR enhancements based on the above agreements and objectives of the WI:

- Design of BS table: how to design the new BS table(s);

- Trigger of BSR: how to trigger BSR;

- BSR content: how to report remaining time associated with BS.

- Discussion

## 2.1 Design of BS table

According to the agreement in RAN2#119bis [1] and RAN2#120 [2], RAN2 has agreed to introduce new BS table(s) to reduce the quantisation error. A straightforward idea is to pre-define a new buffer size table in specifications. However, according to the table 5.1.1.1-1 of 3GPP TR 38.838 [4] shows in below, the packet size of XR service is determined by the data rate and frame rate, and follows the truncated Gaussian distribution.

Table 5.1.1.1-1: Statistical parameters for packet size following truncated Gaussian distribution Parameter; unit; Baseline values for evaluation; Optional values for evaluation for single eye buffer. Mean: M; byte; R×1e6 / F / 8; R×1e6 / F / 8. STD; byte; 10.5% of M; 3 % of M. Max; byte; 150% of M; 109% of M. Min; byte; 50% of M; 91% of M. R: data rate of the flow in Mbps. F: frame generation rate of the flow in fps. Note that the mean and STD apply before truncation applies. Note that the value of R, F depend on application..

Considering that different XR services with different data rates and/or frame rates will have different ranges of data volume, introducing only one new buffer size table is not sufficient. A new BS table is difficult to match with all XR services well.

- If only one new buffer size table is pre-defined, the quantization error may still exist since the new buffer size table cannot match with all XR services well.

Therefore, multiple buffer size tables may be needed. There are two types of solutions to define multiple buffer size tables, as follows.

Pre-specified BS tables: Specify multiple new BS tables in specifications. In this approach, a problem that needs to be further resolved is which of the multiple BS tables should be used for a transmission.

Alt 1: network indicates the UE to use a specific BS table. However, since the network does not know the traffic information before receiving the BSR, the indicated BS table may be unsuitable for the data volume in the buffer. This may cause quantisation error to remain high, but it can be addressed by letting the UE report statistical information about the frame/data size;

Alt 2: UE selects one BS table based on the data volume in the buffer, and the UE indicates which BS table is selected for use to the network via BSR MAC CE or MAC sub-header. This alternative is more complex compared to Alt 1.

Generated BS tables: UE generates a new BS table according to some parameters, such as the number of steps and the maximum/minimum values. Those parameters can be configured by the gNB or pre-specified.

Alt 1: UE dynamically generates a new BS table according to the configured parameters from the gNB and reports one index to indicate the BS value associated with the new table. However, it has the same problem as the Alt 1 of the pre-specified BS tables solution, network is difficult to configure UE with proper parameters because it cannot know the traffic information beforehand. If we assume that the network can derive the traffic information from, for example, the statistics reported by UE, then the Alt1 of pre-specified solution can also work in this way and is simpler;

Alt 2: UE uses two indexes to indicate the buffer size where a first index refers to a legacy BSR table to indicate a rough buffer size while the second index indicates more precise buffer size. This can be seen as UE generating a 2nd-level BS table under the legacy BS tableas shown in Figure 1. The UE firstly uses an index according to the current BS table, i.e., Table 6.1.3.1-1 or Table 6.1.3.1-2 of TS 38.321, to indicate a rough range of the buffer size. In addition to the index in the current BSR table, the UE uses another index of a new buffer size table, to indicate the buffer size more precisely. The 2nd-level tables values can be easily derived according to the bit width of the additional index by dividing the original BS range into several small ranges evenly, hence the 2nd-level tables do not have to be hardcoded in the specifications. For example, if the buffer size is 2000000 bytes, UE will use the index 195 of Table 6.1.3.1-2 firstly, which indicates the BS value is between 1989774 and 2118933. Furthermore, UE uses an additional index 0 of the newly defined buffer size table, which indicates the BS value is between 1989774 and 2022064. From these two indexes, network can get a more accurate buffer size information and alleviate the quantisation error. Actually, the maximal quantisation error of the current long BS table is about 6.5%, if the additional index uses 2 bits, the error will be reduced to 1.5%. If 8 bits are used, then the error will be less than 0.1%. Another advantage of this two-index solution is that the additional index can be optional. UE can report the additional index only when the quantisation error exceeds the threshold e.g. x% or x bytes.

Figure 1: two indexes solution

Table 1: Comparisons among solutions

| Solution |  | Potential specification impacts | Comparison |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Spec impact | Quantisation error | Complexity | Overhead |
| Pre-defined BSR tables | Alt 1： Network indicates which BS table should be used | - Specify multiple new BS tables in specification. - Mechanism for network to indicate a BS table to UE. - To enable network to indicate a proper BS table, UE may need to collect statistics on some information and report the statistics to the network. | Normal | High (if no statistics report) or Normal (statistics based) | Simple | Low |
|  | Alt 2: UE selects a BS table | - Specify multiple new BS tables in specification - Mechanism for UE to select a BS table - Mechanism for UE to indicates the selected BS table | Normal | Normal | Complex | Normal |
| Generated BSR tables | Alt 1: one index solution | - Network configuration of the parameters for generating new BS table to the UE. - Specify how to generate new BS table based on these parameters. - To enable network to configure proper parameters, the UE may need to report the data/frame size statistics to the network. | High | High (if no statistics report) or Normal (statistics based) | Complex | Low |
|  | Alt 2: two indexes solution | - Define a new BSR MAC CE format with two indexes (one pointing to the legacy BSR table and the other one pointing to a new table). - Specify how to derive the actual BS value based on the additional index. | Small | Minor | Simple | Normal |

We compare the above solutions in Table 1. Based on the comparison results we propose RAN2 to consider the Alt 1 of the pre-specified BS table solution or Alt 2 of the generated BS table solution.

- In order to alleviate the quantization error, RAN2 can consider the following approaches:

- pre-define multiple buffer size tables in combination with network indicating which buffer size table the UE should use.

- for the large buffer size, UE uses an index in the current BSR table to “roughly” indicate the range of buffer size and UE uses an additional index to indicate a more precise range of buffer size (within the “roughly” indicated range).

## 2.2 Trigger of BSR

According to the current specification [5], BSR will be triggered by some specific events, see below:

### 5.4.5 Buffer Status Reporting …… A BSR shall be triggered if any of the following events occur for activated cell group: - UL data, for a logical channel which belongs to an LCG, becomes available to the MAC entity; and either - this UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available UL data which belong to any LCG; or - none of the logical channels which belong to an LCG contains any available UL data. in which case the BSR is referred below to as 'Regular BSR'; - UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC CE plus its subheader, in which case the BSR is referred below to as 'Padding BSR'; - retxBSR-Timer expires, and at least one of the logical channels which belong to an LCG contains UL data, in which case the BSR is referred below to as 'Regular BSR'; - periodicBSR-Timer expires, in which case the BSR is referred below to as 'Periodic BSR'. NOTE 1: When Regular BSR triggering events occur for multiple logical channels simultaneously, each logical channel triggers one separate Regular BSR.

Note that the PDB of UL AR traffic is larger than its periodicity, so when a new data burst arrives, the previous data burst may still exist in the LCH. As shown in Figure 2, when data burst 2 arrives, data burst 1 may still exist in the LCH. In this case, none of the trigger events listed above happens and there will be no BSR for data burst 2. The details are as follows:

- Regular BSR: since the previous data burst may still exist in the LCH, the Regular BSR will not be triggered.

- Periodic BSR: as shown in Figure 2, even if the timer length of periodicBSR-Timer is set to the periodicity and there is no jitter, Periodic BSR still does not work for data burst 2 since as long as there is a Padding BSR triggered between the two data bursts, the periodicBSR-Timer will be restarted.

- Padding BSR: since there is still data in the LCH, the Padding BSR may not be triggered.

Figure 2: Exemplary BSR trigger events as per current specifications

In order to let the network know the data volume of a data burst as soon as possible, it would be beneficial to allow BSR to be triggered whenever a new data burst arrives at the UE. Alternatively, when the uplink jitter is negligible which is the case at least for some XR services, we can also consider to enhance the periodicBSR-Timer, for example, by avoiding periodicBSR-Timer to be restarted by other transmitted BSRs.

In addition, considering that the UE may perform PDU set discarding for XR service, for example, timer based PDU set discarding, PDU loss triggered PDU set discarding or congestion triggered PDU set discarding, if the network cannot perceive the change of data volume in UE, the network may schedule excessive resources to the UE leading to a waste of resources. So, in order to let the network know the data volume in the UE in the case of PDU set discarding, we propose RAN2 to introduce the trigger of BSR when the data volume of discarded packets exceeds a threshold.

- BSR trigger can be enhanced in the following aspects:

      - trigger BSR when a new data burst arrives, or enhance the Periodic BSR by allowing the  periodicBSR-Timer not to be restarted by other transmitted BSRs;

      - trigger BSR when the data volume of discarded packets exceeds a threshold.

## 2.3 BSR content

In RAN2#119bis, RAN2 agreed that the delay information should be able to indicate how much data is buffered for which delay value. In RAN2#120, RAN2 agreed to introduce the delay information (e.g. remaining time) associated with the data volume information in a MAC CE. The buffered data volume information has been reported via BSR today, so the most straightforward way is to report the associated delay information via the BSR also. But the current BSR MAC CE format is not sufficient in the following two aspects:

- UE reports the buffer size for each LCG today.

Since the PSDB is larger than the periodicity for the UL AR traffic, a LCH may buffer more than one data bursts which have different remaining PSDBs. If the UE still uses the current BSR MAC CE format, the network will not be able to identify the exact data volume for each data burst. In order to enable the network to identify the exact data volume for each data burst, the UE needs to report the data volume in data burst granularity.

- The current BSR does not contain the delay information.

The remaining PSDB (i.e. “remaining time”) can be included in a MAC CE explicitly or indicated implicitly.

- Define a new BSR MAC CE format to indicate the data volume of data burst and the associated remaining PSDB (i.e., “remaining time”).

Next, we need to discuss whether the BS with remaining time information for XR traffic and the BS for traditional traffic should be included in the same BSR MAC CE. Considering the following reasons, it seems more reasonable to use separate BSR MAC CEs for the XR and non-XR BSR.

Reason 1: New BSR trigger event(s) will be defined for XR traffic. XR traffic and other non-XR traffic will have different BSR trigger events. If once a BSR is triggered (for example be triggered by the newly defined XR event), the UE will report all BS information (for both XR and non-XR traffic), resources will be wasted.

Reason 2: BSR contents for XR traffic and non-XR traffic are different. For example, the BSR of XR traffic contains time information, and the granularity is data burst, while the BSR of non-XR traffic does not contain the time information and the granularity is LCG. Using one BSR MAC CE to report BS of both XR and non-XR traffic, the BSR MAC CE format will be very complex.

- BS with remaining time information for XR traffic and BS for other non-XR traffic are included in different BSR MAC CEs.

As mentioned above, a LCH may buffer more than one data burst which have different remaining PSDBs. To enable the network to distinguish different remaining time of different data bursts, the most straightforward way is to let the UE report the data volume of each data burst (or PDU set) and its associated remaining PSDB (i.e., “remaining time”) to the network.

In addition, considering that there may be multiple LCHs/LCGs related to XR traffic, to enable network to allocate UL grant to a specific LCHs/LCGs, UE also needs to report LCH/LCG corresponding to data volume.

So, we propose that UE reports the data volume of each data burst (or PDU set) in one LCH/LCG and the associated remaining PSDB (i.e., “remaining time”) to network.

- UE reports the data volume of each data burst (or PDU set) in one LCH/LCG and the associated remaining PSDB (i.e., “remaining time”) to network (e.g., the new BSR MAC CE format is per LCH/LCG per PDU set/data burst).

3	Conclusions

In this contribution, we discussed the three issues related to BSR for XR-specific capacity improvement. Based on our discussion, we conclude with the following observation and proposals:

- For the design of new BS tables:

- If only one new buffer size table is pre-defined, the quantization error may still exist since the new buffer size table cannot match with all XR services well.

- In order to alleviate the quantization error, RAN2 can consider the following approaches:

- pre-define multiple buffer size tables in combination with network indicating which buffer size table the UE should use.

- for the large buffer size, UE uses an index in the current BSR table to “roughly” indicate the range of buffer size and UE uses an additional index to indicate a more precise range of buffer size (within the “roughly” indicated range).

- For the trigger of BSR:

- BSR trigger can be enhanced in the following aspects:

      - trigger BSR when a new data burst arrives, or enhance the Periodic BSR by allowing the  periodicBSR-Timer not to be restarted by other transmitted BSRs;

      - trigger BSR when the data volume of discarded packets exceeds a threshold.

- For the content of BSR:

- Define a new BSR MAC CE format to indicate the data volume of data burst and the associated remaining PSDB (i.e., “remaining time”).

- BS with remaining time information for XR traffic and BS for other non-XR traffic are included in different BSR MAC CEs.

- UE reports the data volume of each data burst (or PDU set) in one LCH/LCG and the associated remaining PSDB (i.e., “remaining time”) to network (e.g., the new BSR MAC CE format is per LCH/LCG per PDU set/data burst).

4	References

- R2_119bis-e_meeting_report.

- Draft_R2_120_meeting_report_v1.

- RP-223502, New WID on XR Enhancements for NR.

- 3GPP TR 38.838 V17.0.0, Study on XR (Extended Reality) Evaluations for NR (Release 17).

- 3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".