3GPP TSG-RAN WG2 Meeting #121	R2-2301724

27 February –  3 March, 2023, Athens, GR

Agenda Item	: 6.10.2 (NR_SL_enh-Core)

Source	: LG Electronics Inc.

Title	: Discussion on L2 ID for GC/BC IUC

Document for	: Discussion and Decision

1.	Introduction

Based on the below RAN1 agreement in RAN1 LS [1], RAN2 is to specify related text in MAC specification.

- In RAN1#111 meeting, RAN1 made the following agreement on cast type(s) for inter-UE coordination information transmission triggered by a condition other than explicit request reception in Scheme 1.

Agreement: - The following working assumption is confirmed as follows: - Working Assumption (RAN1#107bis-e meeting): - For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set - FFS: Under which conditions groupcast/broadcast can be supported - Unicast for preferred resource set and non-preferred resource set - FFS: Under which conditions unicast can be supported

2.	Discussion

The following options can be discussed to specify the support of groupcast/broadcast cast type(s) for condition based inter-UE coordination information transmission in the RAN2 specification.

Option 1. The UE can use the GC/BC/UC-related L2 IDs that are already in use or interested in transmitting and receiving IUC messages, and which L2 ID to select from the GC/BC/UC-related L2 IDs that are already in use or interest is up to implementation.

Option 2. Define and use a dedicated L2 GC/BC ID for IUC GC/BC IUC information transmission.

Observation 1. The following options can be discussed to specify the support of groupcast/broadcast cast type(s) for condition based inter-UE coordination information transmission in the RAN2 specification.

-  Option 1. The UE can use the GC/BC/UC-related L2 IDs that are already in use or interested in transmitting and receiving IUC messages, and which L2 ID to select from the GC/BC/UC-related L2 IDs that are already in use or interest is up to implementation.

- Option 2. Define and use a dedicated L2 GC/BC ID for IUC GC/BC IUC information transmission.

Proposal 1. RAN2 decides how L2 GC/BC ID for IUC GC/BC IUC information transmission is specified in MAC specification.

3.	Conclusion

we made the following observation and proposal:

Observation 1. The following options can be discussed to specify the support of groupcast/broadcast cast type(s) for condition based inter-UE coordination information transmission in the RAN2 specification.

-  Option 1. The UE can use the GC/BC/UC-related L2 IDs that are already in use or interested in transmitting and receiving IUC messages, and which L2 ID to select from the GC/BC/UC-related L2 IDs that are already in use or interest is up to implementation.

- Option 2. Define and use a dedicated L2 GC/BC ID for IUC GC/BC IUC information transmission.

Proposal 1. RAN2 decides how L2 GC/BC ID for IUC GC/BC IUC information transmission is specified in MAC specification.

4.	Reference

[1] R1-2212822, LS on cast types for IUC scheme 1, LG