3GPP TSG-RAN2 #121	R2- 2301733

# Athens, Greece, February 27th – March 3rd, 2023

Agenda item:		8.3.2 (Netw_Energy_NR-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on DTX/DRX mechanism

Document for:	Discussion and Decision

1.	Introduction

This document discusses DTX/DRX mechanism for NES. The agreements in R2-120 related to DTX/DRX mechanism are as follows.

Agreements

1 	Clarify previous agreement to: periodic cell DTX/DRX pattern is configured by UE-specific RRC.   Periodic cell DTX/DRX can be activated/deactivated by L1/L2 signalling and UE-specific RRC signaling.

2 	Capture in TR 38.864 that both UE specific and common L1/L2 signalling can be considered for at least activating/deactivating the cell DTX/DRX pattern, per the agreement in 119b-e.

3	Cell DTX and Cell DRX modes can be configured and operated separately (e.g. one RRC configuration set for DL and the other set for UL).  Cell DTX/DRX can also be configured and operated together.

4	It is up to NW whether legacy UEs can access cells with Cell DTX/DRX

5	Cell DTX/DRX can be configured per serving cell and can be applicable for different cells in CA.  No additional RAN2 impacts or enhancements are foreseen.

6	Whether to support multiple Cell DTX/DRX configurations can be discussed later in the normative phase.

7	At least the following parameters can be configured per cell DTX/DRX configuration: periodicity, start slot/offset, on duration.  Details related to UE behaviour can be discussed during WI phase

8	From RAN2 perspective DTX/DRX is feasible

The related objective of the work item are as follows.

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

2.	Discussion

Cell DTX mechanism

Enhancement on Cell DTX/DRX mechanism includes the alignment of Cell DTX/DRX and UE DRX in RRC_CONNECTED mode. This document starts with discussion on what the alignment of Cell DTX and UE DRX means. We think that the alignment means that all UE DRX patterns need to be covered by Cell DTX pattern. In other words, the alignment does not need to mean that Cell DTX pattern and all UE DRX patterns are exactly matched. For example, a UE may be less active and its Active Time can be shorter than others. For Cell DTX operation, all UE DRX patterns need to be covered by Cell DTX pattern. It can be achieved by network implementation because NW configures UE DRX, NW is aware of the Active Time of each UE DRX, and NW can adjust Cell DTX accordingly. Figure 1 shows an example scenario of UE DRX for supporting Cell DTX and the alignment of UE DRX and Cell DTX.

Figure 1 UE DRX for supporting Cell DTX

Proposal 1. Confirm that Cell DTX/UE DRX alignment means that Cell DTX pattern covers all UE DRX patterns.

When the alignment of Cell DTX and UE DRX is assumed, Cell DTX configuration needs not to be informed to UEs. NW configures UE DRX to be aligned with Cell DTX. Since NW can be aware of the Active Time of each UE DRX, NW can decide On duration and Off duration of Cell DTX accordingly. NW transmits data when any of UEs is in the Active Time of its UE DRX. On the other hand, NW does not transmit any data when none of UEs is in the Active Time of its UE DRX. For Cell DTX operation, a UE does not need to know Cell DTX configuration. UE performs only UE DRX according to the configuration.

Observation 1. UE DRX can be used for supporting Cell DTX.

Proposal 2. Cell DTX configuration is not informed to a UE.

It is beneficial that NW have multiple Cell DTX patterns to cope with traffic changes efficiently for achieving high NES gain. In case that Cell DTX is achieved by UE DRX, NW can activate a proper UE DRX configuration to change Cell DTX pattern according to traffic changes if a UE is configured with multiple DRX configurations.

Proposal 3. Allow multiple UE DRX configurations to support multiple Cell DTX patterns.

In addition, whether to activate multiple UE DRX configurations at a time or not can be discussed. If multiple UE DRX configuration can be active at the same time, it is helpful to reduce amount of pre-configured UE DRX configurations because lots of new UE DRX patterns can be created from various combinations of multiple UE DRX configurations. An example scenario is the DRX pattern of UE3 shown in Figure 1.

Proposal 4. Allow multiple UE DRX configurations to be active at the same time.

Based on the R2-120 agreement, both UE specific and common L1/L2 signalling can be considered for at least activating/deactivating the cell DTX/DRX pattern. Similarly, the above-mentioned multiple UE DRX configurations can be activated/deactivated by L1/L2 signalling (e.g. MAC CE).

Proposal 5. Allow multiple UE DRX configurations to be activated/deactivated by L1/L2 signalling.

Cell DRX mechanism

For Cell DRX operation, a Cell DRX configuration needs to be known to a UE because the UE needs to know when the cell can receive its UL transmissions. To keep On duration of Cell DRX compact, a single Cell DRX configuration per serving cell is shared by UEs.

Proposal 6. A single Cell DRX configuration per serving cell is informed to a UE.

Cell DTX/DRX behavior

The following examples on Cell DTX/DRX behavior are captured in TR 38.864.

- Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods. - Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals) - Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS). - Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

The WID has the following Notes for the objective of Cell DTX/DRX mechanism.

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

Based on the Notes, Example 1 and Example 4 are deprioritized for Cell DTX/DRX mechanism because they include turning off SSB transmission. Example 2 and Example 3 turn off all data transmission and dynamic data transmission respectively. Turning off dynamic data transmission only during Off duration of Cell DTX/DRX (Example 3) seems more aligned with the legacy UE DRX behavior. Instead of turning off configured transmission during Off duration of Cell DTX/DRX, NW can configure periodic resource for data or control taking Cell DTX/DRX pattern into account. Therefore, turning off dynamic data transmission only (Example 3) can be baseline for Cell DTX/DRX behavior. Turning off all data transmission including configured data transmission (Example 2) can be further discussed.

Proposal 7. Consider turning off dynamic data transmission only during Off duration of Cell DTX/DRX as a baseline for Cell DTX/DRX behaviour.

Delay sensitive traffic handling

Packets may arrive during Off duration of Cell DTX/DRX, and cannot be scheduled until On duration starts. Enhancements may be considered to resolve the delayed scheduling issue for delay sensitive traffic. Based on the R2-120 agreement, Cell DTX/DRX can be configured per serving cell and can be applicable for different cells in CA. If different Cell DTX/DRX patterns are configured, the issue can be mitigated. However, it cannot be a solution, particularly for UEs which is served by PCell only. To handle the situation that delay sensitive traffic (e.g. emergency call) is triggered in PCell only case, UE may performs handover to a cell available for transmission when delay sensitive traffic arrives during Off duration of Cell DTX/DRX.

Proposal 8. Allow a UE to perform handover to a cell available for transmission when delay sensitive traffic arrives during Off duration of Cell DTX/DRX.

3.	Conclusion

This document discusses DTX/DRX mechanism for NES.

Proposal 1. Confirm that Cell DTX/UE DRX alignment means that Cell DTX pattern covers all UE DRX patterns.

Observation 1. UE DRX can be used for supporting Cell DTX.

Proposal 2. Cell DTX configuration is not informed to a UE.

Proposal 3. Allow multiple UE DRX configurations to support multiple Cell DTX patterns.

Proposal 4. Allow multiple UE DRX configurations to be active at the same time.

Proposal 5. Allow multiple UE DRX configurations to be activated/deactivated by L1/L2 signalling.

Proposal 6. A single Cell DRX configuration per serving cell is informed to a UE.

Proposal 7. Consider turning off dynamic data transmission only during Off duration of Cell DTX/DRX as a baseline for Cell DTX/DRX behaviour.

Proposal 8. Allow a UE to perform handover to a cell available for transmission when delay sensitive traffic arrives during Off duration of Cell DTX/DRX.