3GPP TSG-RAN WG2 Meeting #121		R2-2301742

Athens, Greece, Feb 27 – Mar 3, 2023

Agenda Item:	8.17.2

Source: 	LG Electronics

Title:  	General Solution for Rel-18 MUSIM

Document for:	Discussion

Introduction

For Rel-18 Multi-SIM WI, RAN2 has mainly made the following agreements in RAN2#119Bis meeting [1]:

RAN2 aims to prioritize only few solutions and avoid multiple solutions for the same problem (FFS pending on solution details). A7: The UE can initiate signaling for UE capability restrictions on NW A if NW A allows it. The specification will not capture NW B events which can cause such need. A4: RAN2 to discuss whether the following UE capabilities (not a complete list) are impacted for dual-active MUSIM: MIMO layers, BC capabilities, Measurement capabilities, Bandwidth, srs-TxSwitch, UL tx power, Power Class. For proposals A1-A2, the solution details need more discussion. Other solutions are not precluded (requires company input with details). Will discuss further over email on the solutions (after this meeting) and which capabilities can be affected. For B1-B3, B5, the solution details need more discussion. May prioritize B1, B2 and B5. FFS on signalling details. Other solutions are not precluded (requires company input with details) and none of B1-B5 are agreed as solutions for this WI. Do not consider solution B4 in Rel-18 (since it may have CN impacts which are precluded in this WI)

In this contribution, we discuss the general solution for UE capability restrictions based on the agreements.

Discussion

In RAN2#119Bis meeting, RAN2 discussed several candidate solutions for the temporary UE capability restriction. Since RAN2 agreed to avoid using multiple solutions and will try a few solutions, we discuss what the most suitable solution is.

We think that the UE does not need to change the real UE capabilities and that information must be transferred to the core network. This is because the temporary UE capability restriction that the UE tries to perform to resolve the conflict is a temporary problem that will be resolved as soon as the RRC connection of one of the networks is released.

If the UE transmits real capability information to the core network to resolve the conflict, very frequent signaling will be required as UE capability updates. This is because signaling is required not only for the case that the UE capability is restricted but for the case that the UE capability is no longer to be restricted. This will cause a bottleneck problem because frequent signaling will be more unavoidable than before. Also, since the size of this UE capability information is very large, the bottleneck problem can be more serious.

Also, we do not think lower-layer signaling is necessary for the temporary capability restriction. The main benefit of lower-layer signaling is to reduce latency. However, Rel-18 Multi-SIM functions do not require a new requirement for low latency of any procedure. Therefore, the lower layer signalling based solution should be discussed first in the WI aiming for the purpose of latency reduction.

Considering the above reasons, a solution for the temporary UE capability restriction seems suitable to AS-level function on RRC signaling. Among the proposed candidate solutions, the temporary UE capability restriction using the UAI procedure is the most suitable. Besides, UAI already has a similar function to solve the power saving or overheating problem. Thus, we propose the following.

Proposal 1.	The UE uses the UAI procedure to request the temporary UE capability restriction for dual-active MUSIM, i.e. the temporary UE capability restriction doesn’t trigger the real UE capability update.

The next issue is what information the UE will provide to the network for the temporary UE capability restriction.

We think CG-level or carrier-level restriction will be simpler and more appropriate because most scenarios of dual active Multi-SIM operation seem caused by the frequency conflict between SIM A and SIM B.

Due to frequency conflict, many parameters such as MIMO layers, BC capabilities, Measurement capabilities, Bandwidth, srs-TxSwitch, UL tx power, and Power Class may need to be changed. However, this means that for the temporary UE capability restriction, the UE must request again whenever another parameter is affected.

We believe that the most effective way is to prevent data transmission until the restriction is unnecessary in the serving cell associated with the frequency. If the cell is SpCell, SCG release is the simplest way, and if the cell is SCell, SCell release is the simplest way.

Proposal 2.	For the request of the temporary UE capability restriction, the UE requests a carrier level restriction or a CG level restriction, e.g. SCG/SCell release or SCG/SCell deactivation.

Thus, when the UE transmits a UAI message for the temporary UE capability restriction to the network, the UE may request different restrictions depending on whether the serving cell related to the problematic frequency is SpCell or SCell. If the serving cell related to the problematic frequency is SpCell, the UE transmits SCG release or SCG deactivation as a preference. If the serving cell related to the problematic frequency is SCell, the UE transmits SCell release or SCell deactivation as a preference.

Proposal 3.	When the UE requests the temporary UE capability restriction, the UE indicates a preferred state of SCG/SCell as either release or deactivation.

After reception of the preference from the UE, the network may decide to release or deactivate SCell(s). If the network wants to maintain SCells, the network will command SCell deactivation instead of de-configuration with the existing procedure.

Even for the sPCell, the network may decide to release or deactivate SCG. If the network wants to maintain SCG, the network will command SCG deactivation instead of de-configuration with the existing procedure.

Proposal 4.	For the temporary UE capability restriction, the network may command SCG/SCell release or SCG/SCell deactivation.

However, the option for SCG deactivation requires further discussion because when the network commands SCG deactivation, the UE still needs to perform the UE behaviors such as CSI-RS measurement, beam measurement, or RLM required in the deactivated state for SCG. If the conflict is still not resolved because of these UE behaviors, the UE will not have any choice but to release SCG.

In our understanding, without input from RAN4 to RAN2, it is not likely to conclude whether the existing UE requirements are fine for SCG deactivation without problems in dual TRX Multi-SIM operation. Therefore, we think it is necessary to ask if there is no need for new requirements in RAN4 for SCG deactivation and SCell deactivation. For this, we prepare a draft LS to RAN4 in R2-2301743.

Proposal 5.	RAN2 sends LS to RAN4 to identify if RAN4 needs additional UE requirements for SCG/SCell deactivation in dual-active MUSIM operation. For this, RAN2 is asked to consider the draft LS in R2-2301743.

Finally, let’s consider whether the UE can request the temporary UE capability restriction via SRB3.

Considering the legacy UAI procedure, if SN configures UE assistance information to report and MN is not involved, the UAI message can be directly transmitted to the SN. That is, the UE sends the SCG-specific UAI message to SN directly if SRB3 is configured.

Therefore, if the UE receives a configuration to request for the temporary UE capability restriction directly to the SN, transmission of the SCG-specific UAI message via SRB3 can be supported without any new impact, so we propose the following.

Proposal 6.	If SRB3 is configured, the UE can send the SCG-specific UAI to request the temporary UE capability restriction for dual-active MUSIM to SN directly.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1.	The UE uses the UAI procedure to request the temporary UE capability restriction for dual-active MUSIM, i.e. the temporary UE capability restriction doesn’t trigger the real UE capability update.

Proposal 2.	For the request of the temporary UE capability restriction, the UE requests a carrier level restriction or a CG level restriction, e.g. SCG/SCell release or SCG/SCell deactivation.

Proposal 3.	When the UE requests the temporary UE capability restriction, the UE indicates a preferred state of SCG/SCell as either release or deactivation.

Proposal 4.	For the temporary UE capability restriction, the network may command SCG/SCell release or SCG/SCell deactivation.

Proposal 5.	RAN2 sends LS to RAN4 to identify if RAN4 needs additional UE requirements for SCG/SCell deactivation in dual-active MUSIM operation. For this, RAN2 is asked to consider the draft LS in R2-2301743.

Proposal 6.	If SRB3 is configured, the UE can send the UAI to request the temporary UE capability restriction for dual-active MUSIM to SN directly.

Reference

- R2-2210802 Report on LTE legacy, DCCA, MUSIM, Slicing, 71 GHz, XR and QoE,  Nokia