3GPP TSG-RAN WG2 Meeting #121	R2-2301753

Athens, Greece, 27 February – 03 March 2023

Agenda item:	8.11.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Bandwidth signalling for shared processing

WID/SID:	NR_MBS_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this paper we discuss what should be the bandwidth signalled in the MBS Interest Indication message from UE to gNB, where the UE is capable of simultaneous reception of MBS broadcast from non-serving cell and unicast from serving cell. It also discusses the scenarios to be supported.

# 2	Discussion

## 2.1	Background

RAN2 discussed [1] shared processing of MBS broadcast and unicast reception in RAN2#119bis-e and agreed the following:

For shared processing we adopt the following as a baseline: - 1) new IE is added in system information to control whether MBSInterestIndication for shared processing can be sent or not; - 2) MBSInterestIndication message content and related procedure is updated for shared processing. New IE to control whether MBSInterestIndication for shared processing can be sent or not is added to SIB1. In MBSInterestIndication, for a broadcast service that the UE is receiving or is interested to receive, at least the following information can be signalled: broadcast frequency, subcarrier spacing, and bandwidth. FFS details/exact parameters and other information. FFS in which scenarios the UE reports this information (e.g. intra-PLMN case, inter-PLMN case) FFS whether UE capability is needed to enable shared processing.

Although the proposal in [1] suggested to use the bandwidth of the common frequency resources (CFR) used for scheduling MBS broadcast, the agreement in RAN2#119bis left the details of signalled bandwidth for further study.

## 2.2	Bandwidth

Discussions so far in RAN2 on shared processing of MBS broadcast and unicast reception in NR is based on the LTE MBMS Receive-Only Mode (ROM) solution. Also, the RAN2 focus so far is on multi-Rx/single-Tx UE configuration for shared processing. In LTE, the UE signals the bandwidth configuration corresponding to the frequency used for the reception of MBMS service (mbms-Bandwidth) as shown in the signalling structure from LTE below:

MBMS-ROM-Info-r15 ::= SEQUENCE {

mbms-ROM-Freq-r15						ARFCN-ValueEUTRA-r9,

mbms-ROM-SubcarrierSpacing-r15		ENUMERATED {kHz15, kHz7dot5, kHz1dot25},

mbms-Bandwidth-r15					ENUMERATED {n6, n15, n25, n50, n75, n100}

}

mbms-Bandwidth Indicates the UE received MBMS service frequency bandwidth configuration, NRB in downlink, see TS 36.101 [42], table 5.6-1. n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on.

The bandwidth, mbms-Bandwidth, signalled in LTE is the channel bandwidth which is defined in terms of the number of resource blocks used as Transmission bandwidth corresponding to a channel bandwidth as defined in RAN4 specification TS 36.101 in Table 5.6-1 [2].

Observation 1: The bandwidth, mbms-Bandwidth, signalled in LTE is the channel bandwidth which is defined in terms of the number of resource blocks used as Transmission bandwidth corresponding to a channel bandwidth as defined in RAN4 specification TS 36.101.

However, in NR, the scheduling of MBS broadcast is using common MBS frequency resources (CFR) that defines an ‘MBS frequency region’ in the frequency domain. In addition, there are multiple cases for CFR configuration for MBS broadcast as shown in the figure below:

Figure 1 Broadcast CFR configuration cases

According to Section 4.4.6 in TS 38.211 [3], “A UE is not expected to receive PDSCH or PDCCH associated with MBS transmissions scheduled with G-RNTI, G-CS-RNTI or MCCH-RNTI outside the common MBS frequency resource”. Given this, it makes sense to signal the bandwidth of the Broadcast CFR, as opposed to the channel bandwidth for the frequency of reception of MBS service, in the MBSInterestindication message in NR.

In NR, the bandwidth of Broadcast CFR is signalled as part of cfr-ConfigMCCH-MTCH-r17 in SIB20, in the locationAndBandwidthBroadcast-r17 field as shown below:

locationAndBandwidthBroadcast Indicates starting PRB and the number of PRBs of CFR used for MCCH and MTCH reception. Value sameAsSib1ConfiguredLocationAndBW means the CFR for broadcast has the same location and size as the locationAndBandwidth for initial BWP configured in SIB1. Value locationAndBandwidth is used to configure CFR with bandwidth that is larger than and fully contains the bandwidth for the initial DL BWP and CORESET#0 configured in SIB1. If the field is absent, the CFR for broadcast has the same location and size as CORESET0.

It is to be noted that the signalling of locationAndBandwidthBroadcast also covers the Case A, where the bandwidth of the Broadcast CFR is the same as the Initial BWP, through the use of the value sameAsSib1ConfiguredLocationAndBW.

This then raises the question of how the UE knows the CFR configuration broadcast in SIB20 in a non-serving cell as currently the SIB20 is obtained by the UE only in the serving cell, either through broadcast system information acquisition in the serving cell or by dedicated signalling of SIB20 (of an SCell) by the serving gNB. If a UE is shared processing capable and can receive MBS broadcast from non-serving cell due it’s multi-Rx configuration capability, it is possible the UE can also acquire SIB20 from a non-serving cell and can be entirely up to UE implementation. Alternatively, it could be up to network implementation to provision the CFR configuration of neighbour cells via OAM and which can be provided to a shared processing capable UE via dedicated signalling.

Proposal 1: RAN2 to agree that the bandwidth signalled in the MBS Interest Indication message by a UE that is capable of simultaneous reception of MBS broadcast from non-serving cell and unicast from serving cell is the bandwidth of the Broadcast CFR.

## 2.3	Scenarios to be supported

Another open issue that is still not resolved is, the scenarios (e.g., intra-PLMN case, inter-PLMN case) in which the UE reports MBS Interest Indication for shared processing. Some companies pointed out that for intra-PLMN case it is not necessary to report MBS Interest Indication for shared processing because the serving cell (unicast service) and non-serving cell (broadcast service) may be under the same gNB or OAM configuration can be relied upon (even for inter-gNB, intra-PLMN case). While this may be the case, for simplicity reasons (from both UE implementation and specification efforts perspective), consistency in the solution for intra-PLMN (both intra-gNB and inter-gNB) and inter-PLMN scenarios is preferred and hence it is better to support reporting of MBS Interest Indication for shared processing irrespective of the scenarios supported. It is to be noted that the shared processing objective in the work item description states that support should be provided for “same or different operators” cases.

Proposal 2: RAN2 to agree to support reporting of MBS Interest Indication for shared processing irrespective of the scenarios supported i.e., for both intra-PLMN (both intra-gNB and inter-gNB) and inter-PLMN scenarios.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: The bandwidth, mbms-Bandwidth, signalled in LTE is the channel bandwidth which is defined in terms of the number of resource blocks used as Transmission bandwidth corresponding to a channel bandwidth as defined in RAN4 specification TS 36.101.

Proposal 1: RAN2 to agree that the bandwidth signalled in the MBS Interest Indication message by a UE that is capable of simultaneous reception of MBS broadcast from non-serving cell and unicast from serving cell is the bandwidth of the Broadcast CFR.

Proposal 2: RAN2 to agree to support reporting of MBS Interest Indication for shared processing irrespective of the scenarios supported i.e., for both intra-PLMN (both intra-gNB and inter-gNB) and inter-PLMN scenarios.

# Reference

- R2-2210385, Shared processing for simultaneous MBS broadcast and Unicast reception, Intel Corporation

- TS 36.101, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception

- TS 38.211, NR Physical channels and modulation