3GPP TSG RAN WG2 #121	R2-2301758

Athens, Greece, 27th Feb– 3rd Mar 2023

Agenda Item:	8.14.4

Source: 	China Unicom

Title:  	Discussion on QoE configuration and reporting for NR-DC

Document for:	Agreement

Introduction

In the previous RAN2 and RAN3 meeting [1-5], there are some open issues left on Rel-18 NR QoE WI, e.g. SRB selection for QoE configuring and QoE reporting, RAN visible QoE measurement. Discuss details in the next meeting.

In this paper, SRB selection for QoE configuration and reports, are discussed.

Discussion

- 2.1	SRB selection for QoE measurements

SRB selection for QoE measurement configuration

For SRB selection of QoE measurement configurations, RAN2 has made the following agreement at RAN2#119b-e meeting:

Either SRB1 or SRB3 can be used for providing SN configuration to UE (at least for m-based QoE). FFS if this requires additional MN-SN coordination.

For s-based QoE measurement, RAN3 has made the following agreement at RAN3#117-e meeting:

MN is responsible to configure the s-based QoE to UE.

For s-based QoE, it’s specified in Rel-17 that the gNB (SA) or MN provides QoE configurations to the UE by SRB1, so it’s better to take it as baseline. And to align with RAN3 agreement, only MN can configure all the s-based QoE to the UE, so it’s proposed as below:

Proposal 1: For signalling-based QoE measurement, SRB1 is used for providing all the QoE configurations to UE.

For m-based QoE measurement, there are two optional SRBs can be used for providing SN configurations. It’s unclear in which scenarios that and which node can use the SRB1 or SRB3 to transfer the SN configuration to UE. Generally, SRB1 is only used for MN and SRB3 is only used for SN. So in our views, using SRB1 means SN forwards SN-related m-based QoE configuration to MN, then MN can configured SN configuration by MN leg. And using SRB3 means SN can configured SN configuration directly by SN leg. So it’s proposed as below:

Proposal 2: For management-based QoE measurement, the UE can receive SN configurations from the MN via SRB1, or receive SN configurations from the SN via SRB3.

SRB selection for QoE measurement reporting

For SRB selection of QoE measurement reporting, RAN2 has made the following agreement at RAN2#119b-e meeting:

Use SRB4 as baseline for Rel-18 QoE. FFS how we can send QoE reports towards SN (e.g. only SRB4, define new SRB, reuse SRB3, split SRB). Discuss details in the next meeting.

For SA network, SRB4 is used for the QoE reporting with the lowest priority in the all SRBs. For NR-DC network, it’s found that the “leg” to the MN is always kept by the UE, while the “leg” to the SN only is kept when the UE is connected to both MN and SN. So, the SRB used for QoE reporting in SN can be defined with a same or lower priority than SRB4 used for QoE reporting in MN. Thus SRB3 can be precluded due to a higher priority than SRB4. And split SRB (split SRB4/SRB3) can also be precluded due to there is no need to transmit the duplicated QoE reports by both MN and SN legs. Thus RAN2 can consider to use SRB4 or a new SRB with a same or lower priority than SRB4 for QoE reporting towards SN.

Thus it’s proposed:

Proposal 3: The UE can send QoE reports towards SN only via SRB4 or a new SRB with a same or lower priority than SRB4.

- 2.2	RAN visible QoE measurement

    - RAN visible QoE measurement reporting

RAN3 has made the following agreement at RAN3#117b-e meeting:

Agree to ensure that the RVQoE report is sent to the node(s) that provide the bearer(s) associated to the corresponding RVQoE measurement result in the RVQoE report. FFS on how to ensure.

MN and SN can generate the RVQoE configuration by itself, and RAN3 has agreed that both MN and SN can receive the RVQoE report. In the RVQoE report, the PDU session information and QoS flow information are included in the report, if the PDU session and QoS flow are related the MN/SN, the RVQoE report will send to MN/SN separately from the UE.

Proposal 4: PDU session information (PDU session ID) and QoS flow information (QoS flow ID) included in the RVQoE report can be used to ensure the corresponding RVQoE measurement result sending to the associated MN or SN.

Summary

RAN2 is kindly asked to discuss the following proposals:

Proposal 1: For signalling-based QoE measurement, SRB1 is used for providing all the QoE configurations to UE from the gNB.

Proposal 2: For management-based QoE measurement, the UE can receive SN configurations from the MN via SRB1, or receive SN configurations from the SN via SRB3.

Proposal 3: The UE can send QoE reports towards SN only via SRB4 or a new SRB with a same or lower priority than SRB4.

Proposal 4: PDU session information (PDU session ID) and QoS flow information (QoS flow ID) included in the RVQoE report can be used to ensure the corresponding RVQoE measurement result sending to the associated MN or SN.

References

- RAN3_117-e_agenda_20220825_EOM1	RAN3 Chair

- RAN3_117bis-e_agenda_20221018_EOM	RAN3 Chair

- RAN3_119_agenda_20221230	RAN3 Chair

- RAN2-119bise LTE DCCA MUSIM Slicing 71 GHz QoE XR (Tero)_2022-10-19-EOM

- RAN2-120 LTE DCCA MUSIM Slicing 71 GHz QoE XR (Tero)_2022-11-18-1432