3GPP TSG-RAN WG2 Meeting #121                                    R2-2301778

Athens, Greece, 27th February – 3rd March, 2023

Agenda item:		8.17.3

Source:				Samsung

Title:					Further discussion on MN-SN MUSIM gaps coordination

Document for:		Discussion & Decision

1	Introduction

At RAN2#116-e meeting, RAN2 discussed whether MUSIM gaps are supported in MR-DC scenario in Rel-17 and made the following agreement:

- RAN2 will not specify MN-SN coordination of MUSIM gaps with MR-DC in Rel-17

- MUSIM with MR-DC is not explicitly supported in Rel-17 (i.e. no specification efforts done to allow or prevent use of MUSIM with MR-DC)

The discussion on the need of MN-SN coordination of R17 MUSIM gaps was revisited in Rel-18 and the following agreement was made in RAN2#119bis-e meeting:

- RAN2 thinks MN-SN coordination for Rel-17 MUSIM gaps requires WI clarification in RAN

Then, the scope of Rel-18 MUSIM WI [1] is revised at RAN#98e meeting as per below:

2.Specify MN-SN coordination of R17 MUSIM gaps when network A is NR-DC in Rel-18 [RAN2]

This contribution further discusses the MN-SN coordination of R17 MUSIM gap.

2	Discussion

In Rel-17, MUSIM gaps are only supported in NR and the UE is only allowed to provide MUSIM assistance information for gap preference to MN, as clarified below:

otherConfig Contains configuration related to other configurations. When configured for the SCG, only fields drx-PreferenceConfig, maxBW-PreferenceConfig, maxBW-PreferenceConfigFR2-2, maxCC-PreferenceConfig, maxMIMO-LayerPreferenceConfig, maxMIMO-LayerPreferenceConfigFR2-2, minSchedulingOffsetPreferenceConfig, minSchedulingOffsetPreferenceConfigExt, rlm-RelaxationReportingConfig, bfd-RelaxationReportingConfig, btNameList, wlanNameList, sensorNameList and obtainCommonLocation can be included.

Observation 1: In Rel-17, the UE is only allowed to provide MUSIM assistance information for gap preference to NR MCG.

Since network A can configure at most 4 MUSIM gap patterns (e.g. three periodic gaps and a single aperiodic gap) in Rel-17, one may consider whether UE can also indicate its preference on R17 MUSIM gaps to SN in Rel-18 when network A is NR-DC. However, we think that R17 MUSIM gaps are per UE so such enhancement seems not needed.

Proposal 1: Confirm that as in Rel-17, the UE is only allowed to provide MUSIM assistance information for gap preference to NR MCG and NR MCG configures the UE with MUSIM gap(s) in Rel-18.

Then, one may also consider whether MN-SN coordination is also needed when network A is NE-DC in R18. However, as per above objective, we understand that NE-DC is excluded.

Proposal 2: Confirm that RAN2 does not specify MN-SN coordination of R17 MUSIM gaps when network A is NE-DC in Rel-18.

In Rel-17, it is not supported for network to provide an alternative gap pattern instead of the one requested by the UE. Thus, we think that it is enough for each node to decide whether to accept or reject UE's requested MUSIM gap(s) for MN-SN coordination. That is, MN signals the MUSIM gap configuration (e.g. MUSIM-GapConfig) in CG-ConfigInfo to SN. Then, SN indicates the accepted MUSIM gap configuration to MN in CG-ConfigInfo.

Proposal 3: CG-Config and CG-ConfigInfo inter-node messages are used for MN-SN coordination of R17 MUSIM gaps as follows:

- CG-ConfigInfo: MN signals the MUSIM gap configuration (e.g. MUSIM-GapConfig) to SN

- CG-Config: SN indicates which part of the MUSIM gap configuration can be accepted to MN

3	Conclusion

In section 2, following observations are made:

Observation 1: In Rel-17, the UE is only allowed to provide MUSIM assistance information for gap preference to NR MCG.

Based on the above, RAN2 is requested to discuss and agree on the following proposal:

Proposal 1: Confirm that as in Rel-17, the UE is only allowed to provide MUSIM assistance information for gap preference to NR MCG and NR MCG configures the UE with MUSIM gap(s) in Rel-18.

Proposal 2: Confirm that RAN2 does not specify MN-SN coordination of R17 MUSIM gaps when network A is NE-DC in Rel-18.

Proposal 3: CG-Config and CG-ConfigInfo inter-node messages are used for MN-SN coordination of R17 MUSIM gaps as follows:

- CG-ConfigInfo: MN signals the MUSIM gap configuration (e.g. MUSIM-GapConfig) to SN

- CG-Config: SN indicates which part of the MUSIM gap configuration can be accepted to MN

4	Reference

[1] RP-223492, Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, vivo