3GPP TSG-RAN WG2 Meeting#121		       						R2-2301791

Athens, Greece, Feb 27th – Mar 3rd, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Considerations On RAN 1 LS for MIMO Evolution

Agenda item:		8.21.2

Document for: 	Discussion and Decision

- Introduction

There is an LS R1-2213004 received from RAN 1, in the LS, RAN1 request RAN2 to decide whether there is a need to enhance CBRA procedure to support per TRP UE-initiated RACH procedure.

Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it is up to RAN2 to decide whether there is a need to enhance CBRA procedure to support per TRP UE-initiated RACH procedure.

In this contribution, we share our views on this issue.

- Discussion

In RAN1 MIMO Evolution discussion, the CFRA based per TRP TA Acquisition (i.e. PDCCH ordered RACH）is supported. And they want to know whether the UE initiated per TRP CBRA is needed.

From RAN2 perspective, the target for UE to initiate the CBRA procedure for TA acquisition is for acquiring the UL resources due to UL data arrival, see TS 38.300 in below block. It does not matter that the TA of which TRP can be obtained.

- 9.2.6 Random Access Procedure The random access procedure is triggered by a number of events: - Initial access from RRC_IDLE; - RRC Connection Re-establishment procedure; - DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised"; - UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; - SR failure; - Request by RRC upon synchronous reconfiguration (e.g. handover); - RRC Connection Resume procedure from RRC_INACTIVE;

And hence, UE can initiate the legacy CBRA procedure to obtain the TA of any one TRP first in the case of the mPDCCH mTRP, and then send the BFR MAC CE and/or UL data to NW via the TRP whose TA value is obtained, it is up to NW to determine whether there is any need to initiate the PDCCH ordered RACH for obtaining the TA of the other TRP. it seems no need to introduce per TRP UE initiated CBRA. Considering RAN1 are waiting for the conclusion from us for their further study, we suggest to reply them with our conclusion.

RAN2 Confirms there is no need to enhance CBRA procedure to support per TRP UE initiated RACH procedure for obtaining TA value of TRP. And send a reply LS with RAN2 understanding to RAN1.

The draft LS is attached in the Annex.

- Conclusion and proposals

In this contribution, we discussed the UE initiated CBRA for TRP with the following proposal:

Proposal 1.	RAN2 Confirms there is no need to enhance CBRA procedure to support per TRP UE initiated RACH procedure for obtaining TA value of TRP. And send a reply LS with RAN2 understanding to RAN1.

- References

- R1-2213004  LS on Multi-DCI Multi-TRP with two TAs

- Annex

Title:	Draft reply LS to RAN1 on CBRA enhancement for per TRP UE initiated RACH

Response to:	R2-23xxxxx

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL-Core

Source:	ZTE [to be changed to RAN2]

To:	RAN1

Contact Person:

Name:	Fei Dong

E-mail Address:	dong.fei@zte.com.cn

1. Overall Description

RAN2 would like to thank RAN1 for asking RAN2 to confirm whether to enhance CBRA procedure for per TRP UE initiated RACH procedure.

From RAN2 point of view, the UE initiated CBRA procedure targets to acquire the UL resources with RACH in the case of the UL data arrival with the UL cynchornisation status is ‘non-synchronised’. Considering the legacy CBRA is able to obtain the TA of at least one of the TRP which is enough for acquiring the UL resources. So RAN2 concludes that there is no need to enhance the CBRA procedure for per TRP UE initiated RACH procedure.

2. Actions:

To RAN WG 1:

ACTION: 	RAN2 respectfully asks RAN1 to consider the above RAN2 point of view for the RAN1 further work.

3. Date of Next TSG-RAN WG 1 Meetings:

RAN2#121BIS emeeting, Electronic, 17-21 Apr 2023

RAN2#122 meeting, Incheon, 22-26 May 2023