3GPP RAN WG2 Meeting #121	R2-2301792

# Athens, Greece, Feb 27th – Mar 3rd, 2023

Agenda Item:	8.2.2

Source:	InterDigital Inc.

Title:	Discussion on Sidelink positioning

Document for:	Discussion

# 1. Introduction

This contribution discusses sidelink positioning procedures’ related open issues; priority handling for SL positioning signallings and the discovery procedure for the anchor UEs.

# 2. Discussion

## 2.1 Priorities for PHY signalling

For sidelink communication, SCI (Sidelink Control Information) is sent over PSCCH + MAC header and the SCI signals Layer 2 IDs (source and destination IDs), Priority (QoS value, either LCP value or PHY specific priority value), PSSCH resource assignment and so on. The priority information is used by the PHY layer to perform Mode 2 sensing and prioritization between SL transmissions or between SL and UL transmission/receptions.

For SL positioning, for the reception of SL-PRS at the PHY layer of the receiver side, a priority corresponding to the PRS should be present so that the PHY layer can perform Mode 2 sensing and prioritization between SL transmissions (e.g. SL-PRS <> PSSCH) or between SL and UL transmission/receptions (e.g. SL-PRS <> PUSCH) in the same way as legacy.

The priority of SL-PRS may be signalled by RRC.

Proposal 1: 	Support priority of SL-PRS

If Proposal 1 can be agreed, then the next question is a priority setting for the SL positioning measurement reporting.

UEs involved in SL positioning operation may also need to exchange the positioning measurement report and the report should be prioritised with a priority value thus the support of priority for the SL positioning measurement reporting should be considered. The priority of SL positioning measurement reporting may be signalled by RRC or can be set to a same value as the one assigned to the SL-PRS for the specific SL positioning context.

Proposal 2: 	Support priority of positioning measurement reporting

## 2.2 Discovery of anchor UEs

SA2 raised a key issue with regard to anchor UE discovery as key issue #5 in [1]. Sublcuase 5.5.1 in [1] describes the key issue for the case the target UE is out of coverage or in partial network coverage. Currently SA2 mention the following aspects to be stuied;

For this key issue, the following aspects need to be studied:

-	Discovery of one or more Located UEs with capabilities that offers Sidelink Positioning and/or Uu based positioning to support Network assisted Sidelink Positioning for in network coverage and partial network coverage.

-	Discovery of network capability that offers Network assisted Sidelink positioning for UE in network coverage and partial network coverage.

-	Signalling needed to support Sidelink positioning including both UE-based positioning and network-assisted/based positioning.

The SL positioning discovery procedure definitely has RAN2 impact and so for the sake of progress, it’s very good idea to start studying the potential solution for the discovery mechanism for the SL positioning.

As mentioned by SA2, the target UE needs to identify the SL capable UEs (e.g., anchor UEs), who also support the SL positioning capabilities. For Prose case, UEs exchange the relevant capability information during the discovery procedure (i.e. DISCOVERY_REQUEST and DISCOVERY_RESPONSE message exchange) and we can see some similarities and so it looks very reasonable to study the SL positioning discovery mechanism with an assumption that the ProSe direct discovery is a baseline for that.

Proposal 3: 	Study discovery mechanism for SL positioning. ProSe is considered as a starting point.

## 2.4 Measurement reporting by anchor UEs when target UE is out-of-covearge and anchor UEs are in-covearge

As described in Key Issue #5 in Section 8.5 of [1], the target UE may start positioning based on MT-LR received while the target UE is in coverage. In DL-like positioning methods, the target UE makes measurements on received SL-PRS and send the measurement report to the network. Due to mobility prior to the measurement reporting, if the target UE moves out of coverage and the target UE will not be able to send the measurement report to the network. Thus, the target UE may need to request one of the anchor UEs to forward the measurement report to the network. Thus, the followpoing proposal is made :

Proposal 4 : For DL-like positioning methods, study mechanism to send the measurement rerpot to the LMF when the target UE is out-of-coverage and anchor UEs are in coverage (partial coverage case)

## 2.5 Involvement of LMF in SL positioning and configuration of SL-PRS

The following illustrate the example signalling configuring and triggering of SL-PRS for IC and OOC scenarios:

Figure 1 An example of signalling between LMF, gNB, anchor UE and target UE for SL positioning in IC scenarios

Figure 2 An example of signalling between anchor UE and target UE for SL positioning in OOC scenarios

For IC scenarios, the target UE and/or anchor UEs can initially receive the SL-PRS configurations from network (e.g. gNB or LMF) via higher layer signalling. The anchor UEs can then be triggered via lower layer signalling (e.g. MAC CE or DCI) for initiating/activating the transmission of SL-PRS. In OOC scenarios, the target UE can receive the SL-PRS configurations from anchor UE or use any preconfigured SL-PRS configurations (e.g. preconfigured by network) via higher layer signalling. The anchor UEs and/or target UE can then be triggered via lower layer signalling (e.g. SL-MAC CE or SCI) for initiating/activating/deactiving SL-PRS transmissions.

Proposal 5: Support combination of higher-layer and lower-layer signalling (e.g. MAC CE, DCI, SCI) for configuring and triggering of SL-PRS transmissions in both in-coverage and out-of-coverage scenarios, use of higher layer signaling to control periodic SL-PRS, use of MAC-CE to activate/deactivate semi-persistent PRS

## 2.6 Group-based positioning

A Target UE and Anchor UE(s) can perform resource selection for SL-PRS transmissions specific to a SL positioning session using one of the group-based positioning such as SL-AoA or UL-TDOA like positioning where both methods require one-to-many PRS transmision. In addition, DL-TDOA like positioning methods where anchor UE transmits SL-PRS to the target UE requires formation of a group of UEs. It can thus be beneficial to define a SL positioning group to facilitate discussions on resource allocation by group member with potential coordination within the group, group-based resource allocation, etc.

In addition to absolute positioning information, SL positioning supports relative positioning and ranging information [2]. A target UE obtains the information based on ranging, which is defined as a determination of the distance and/or the direction between a target UE and another SL entity, e.g., anchor UE [2]. It would be helpful to discuss SL relative position and/or ranging information with respect to a reference UE. Therefore, we propose additional SL positioning terminology.

Proposal 6: Adopt the following terminology for the purpose of continued RAN1 discussion:

- Reference anchor UE: An reference entity e.g., anchor UE, based on which a target UE’s relative position and/or ranging information is determined.

- Sidelink positioning group: A group of UEs including a target UE and one or more anchor UEs, who participate in a SL positioning session using one of the supported SL positioning method.

Once we agree on the principle of having a group of UEs, we should consider maintenance of the group. For example, during a positioning session, it is desirable to maintain all UEs in a group for reliable positioning. For example, if one of the anchor UEs moves out of a group due to mobility, there needs to be a mechanism to continue or terminate the positioning session. Thus, RAN2 should discuss a framework to maintain a sidelink positioning group.

Proposal 7 : Study mechanism to maintain the sidelink positioning group

# Conclusion

In this contribution, the following proposals were made:

Proposal 1: 	Support priority of SL-PRS

Proposal 2: 	Support priority of positioning measurement reporting

Proposal 3: 	Study discovery mechanism for SL positioning. ProSe is considered as a starting point.

Proposal 4 : For DL-like positioning methods, study mechanism to send the measurement rerpot to the LMF when the target UE is out-of-coverage and anchor UEs are in coverage (partial coverage case)

Proposal 5: Support combination of higher-layer and lower-layer signalling (e.g. MAC CE, DCI, SCI) for configuring and triggering of SL-PRS transmissions in both in-coverage and out-of-coverage scenarios, use of higher layer signaling to control periodic SL-PRS, use of MAC-CE to activate/deactivate semi-persistent PRS

Proposal 6: Adopt the following terminology for the purpose of continued RAN1 discussion:

- Reference anchor UE: An reference entity e.g., anchor UE, based on which a target UE’s relative position and/or ranging information is determined.

- Sidelink positioning group: A group of UEs including a target UE and one or more anchor UEs, who participate in a SL positioning session using one of the supported SL positioning method.

Proposal 7 : Study mechanism to maintain the sidelink positioning group

# References

[1]	3GPP TR23.700-86 Study on Architecture Enhancement to support Ranging based services and sidelink positioning

[2] 	RAN1 Chairman’s note, RAN1#110, August 2022.