3GPP TSG-RAN WG2 Meeting #121 	R2-2301799

Athens, Greece, February-March, 2023

Agenda Item:	8.10.2

Source:	CATT

Title:	Discussion on IDC FDM solution enhancement

Document for:	Discussion and Decision

1	Introduction

In last meeting [1], some agreements related to IDC FDM solution enhancement were achieved as follow:

- Reconfirm, The Rel-18 IDC solution should allow for more granular IDC indications both on serving and on non-serving frequencies. - Only one single new finer granularity report is introduced, that applies for both serving and non-serving frequencies. - For LTE, problematic frequencies of E-UTRA are indicated by indicating measurement object IDs (same as existing LTE, no specification impact is foreseen.) - RAN2 down select one of solution 1, 2 or 2a based on ASN.1 details. FFS on the signalling details, how to configure, how to report. - MN can configure IDC, FFS whether SN can configure IDC for SN

There is a post email discussion related to the final solution based on ASN.1 details. And the coordination between MN and SN. In this paper, we will provide some view on these issues.

2	Discussion

2.1	FDM solution

According to the post email discussion for FDM solution [2], there are three solutions for FDM enhancement were discussed.

- Option 1: Central frequency + Bandwidth of the actual affected frequency range

- Option 2: Starting frequency + Ending frequency of the actual affected frequency range

- Option 2a: starting frequency + Bandwidth of the actual affected frequency range

Based on the feedback from each companies, most companies prefer the solution 1. So it was intended to agree to adopt the solution1 i.e. Central frequency +Bandwidth of the actual affected frequency range as the final FDM solution. Since the provided initial ASN.1 structure only includes the central frequency and the affected bandwidth, it is can be the baseline.

Proposal 1: The initial ASN.1 structure includes the central frequency and the affected bandwidth can be the baseline.

Most companies also agree to add the granular values for bandwidth (including BW in KHz), direction of interference, and combination of frequencies. In addition to the above information, we think it is also need to cover CA case and DC band combination case. So it is needed to add the combination of multiple frequency ranges. Besides, the victim type can also be considered.

Proposal 2: For the final FDM solution enhancement, the combination of multiple frequency range and the victim type need to be considered.

2.2 The coordination between MN and SN

According to the post email discussion on FDM, most companies agree that no additional coordination is need for IDC configuration, apart from the existing mechanism between MN and SN. But in case of IDM in NR-DC, if the combination of frequencies affected by the IDC issues involves MN and SN, the coordination between MN and SN is needed to decide the solution. As the MN the master node, UE can report the IDC issues information including the MN related information and SN related information to MN. Then it is decided by MN whether send the SN related information to SN to allow the SN to resolve the IDC issue. So it is beneficial to introduce the coordination between MN and SN to resolve the IDC issue in case of IDM in NR-DC.

Proposal 3: It is beneficial to introduce the coordination between MN and SN to resolve the IDC issue in case of IDM in NR-DC.

3 Conclusion

According to the above discussion, the following proposals are made:

Proposal 1: The initial ASN.1 structure includes the central frequency and the affected bandwidth can be the baseline.

Proposal 2: For the final FDM solution enhancement, the combination of multiple frequency range and the victim type need to be considered.

Proposal 3: It is beneficial to introduce the coordination between MN and SN to resolve the IDC issue in case of IDM in NR-DC.

4 References

- Draft Report of 3GPP TSG RAN WG2 meeting #120;

- Summary of [Post120][652][IDC] Further details of FDM solution (Huawei)