3GPP TSG-RAN WG2 Meeting #121 	R2-2301802

Athens, Greece, February-March, 2023

Agenda Item:	8.14.4

Source:	CATT

Title:	Discussion on support of QoE measurement for NR-DC

Document for:	Discussion and Decision

# 1	1 Introduction

In RAN2#119bis meeting [1], the QoE measurement in NR-DC was discussed and the following agreements were achieved.

- Observation: Rel-18 QoE configuration may be created by MN or SN. - Either SRB1 or SRB3 can be used for providing SN configuration to UE (at least for m-based QoE). FFS if this requires additional MN-SN coordination. - 1: In NR-DC scenario, both signalling-based and management-based QoE measurement collection shall be supported. - RAN2 assumes that there is a unique ID for QoE configurations across MN and SN. This can be accomplished by MN-SN coordination (e.g. similar as was done with measIds for NR-DC) - Use SRB4 as baseline for Rel-18 QoE. FFS how we can send QoE reports towards SN (e.g. only SRB4, define new SRB, reuse SRB3, split SRB). Discuss details in the next meeting.

In this discussion paper, we will further analyse the remaining issues.

# 2 Discussions

## 2.1 Container based QoE configuration

In NR-DC, there are two connections simultaneously between UE and gNB, one to MN and the other with SN. The service can be transmitted over SN. Therefore, we need to discuss how to support the QoE configuration and reporting over MN/SN in NR DC.

According to the last meeting agreements, either SRB1 or SRB3 can be used for providing SN QoE configuration to UE. And this may need some additional MN-SN coordination.

- Either SRB1 or SRB3 can be used for providing SN configuration to UE (at least for m-based QoE). FFS if this requires additional MN-SN coordination.

For M-based QoE configuration, there exist three cases: 1) only MN receives the QoE configuration; 2) both MN and SN receive the QoE configuration; 3) only SN receives the QoE configuration. As one node cannot know whether the other node receives the same QoE configuration and what is the content of the other node’s QoE configuration, so the coordination between MN and SN is necessary.

In the previous RAN3 meeting [2] [3], the following agreements related to QoE configuration in NR-DC were achieved.

RAN3#117 agreement： MN is responsible to configure the s-based QoE to UE. For M-based QoE configuration in NR-DC, coordination between MN and SN is needed. Details are FFS. If the M-based QoE configuration is received by the MN, the MN should make the decision on the UE selection and on which node sends the QoE configuration to the UE. If the M-based QoE configuration is received only by the SN, whether the MN or the SN performs UE selection and sends the QoE configuration to the UE needs to be further discussed. RAN3#118 agreement： - In case of management-based QoE, the MN decides which node to perform the QoE measurement configuration, FFS which node (MN or SN) performs UE selection. - When MN configures a UE with m-based QoE, it may indicate to SN: the QoE Reference, the MCE IP address. FFS for other information (e.g., RRC ID)

In our understanding, as the MN is the master node, it is reasonable that the MN decides which node performs the QoE configuration for m-based QoE. When the MN receives the QoE configuration, the MN can send the QoE configuration to UE via SRB1 directly. There is no need to wait for SN’s feedback. Besides, according to the RAN3 agreement, the MN may indicate the QoE Reference and MCE IP address to SN which may be used for QoE reporting via SN.

Proposal 1: When the MN receives the QoE configuration, MN can send the QoE configuration to UE via SRB1 directly.

In case of when SN receives QoE configuration, the SN cannot know whether the MN also receives the same QoE configuration or whether the MN allow the SN to send the QoE configuration to UE. So the SN need to send to the received QoE configuration to MN. If the MN want to send the QoE configuration to UE via SRB1, the MN can indicate the SN not to configure the QoE measurement to UE. If the MN want to let SN send the QoE configuration to UE directly, the MN can also indicate to SN. Or the MN can indicate that only part of QoE configuration can be send to UE via SN. Anyway, when SN receives the QoE configuration, the SN need to send the QoE configuration to MN and then decides whether send the QoE configuration to UE according to the feedback of MN.

Proposal 2: When SN receives the QoE configuration, the SN needs to send the QoE configuration to MN and then decides whether to send the QoE configuration to UE according to the feedback of MN.

## 2.2 Bearer handling for SN QoE reporting

For QoE reporting in NR-DC, the major motivation is that the report can be transferred via SN node when the radio quality of the MN leg is not good or the MN is overload. Then the network does not need to pause the reporting and can avoid discarding the report of QoE results. According to the LS [2] from RAN3, it is agreed that QoE reports can be transmitted to either MN or SN and the report leg can be changed during the application session. When the UE transfer the QoE reports to MN, UE can use the SRB4 as in R17.

But when UE transfer the QoE reports to SN, it is still FFS on how to send QoE reports towards SN.

- Use SRB4 as baseline for Rel-18 QoE. FFS how we can send QoE reports towards SN (e.g. only SRB4, define new SRB, reuse SRB3, split SRB). Discuss details in the next meeting.

Regarding whether split SRB can be used to transmit QoE reports in NR-DC, we think there is no big benefit to support the split SRB for transferring the QoE report. And if we adopt the split SRB approach, the QoE report should be transferred to another node via Xn. This will introduce the additional Xn signalling.

The similar issue also has been discussed in LTE and in NR R17, it was agreed to send the QoE reports over a separate SRB4. The benefits of introducing the separate SRB4 are that the QoE reports could be sent over the control plane but without impacting other RRC transmission. Therefore, when UE transfer the QoE report to SN, we can reuse the same solution in R17 to introduce a new SRB like SRB4.

Proposal 3: The UE can transfer the QoE report to SN using a new SRBx.

According to the LS from RAN3 [4], the report leg can be changed during the application session. So it need to be discussed how to trigger the UE to send the report via the other node. As when the node configured the UE with QoE measurement is overload or the radio quality of one node leg is not good enough to send the QoE report, this node can send the command to the UE and indicate the UE to send the QoE report via the other node.

Proposal 4: When the node configured the QoE measurement is overload or the radio quality is not good enough, UE can send the QoE report via the other node based on the received command.

## 2.3 RV QoE configuration and reporting in NR-DC

In NR-DC case, there are two NG-RAN nodes included. i.e. MN and SN. RAN visible QoE is configured by RAN node and the report is used by the RAN node. The service over MN or SN may be different and MN and SN may have the different requirement. So in NR-DC case, both MN and SN can generate and configure the RAN visible QoE measurements separately.

Proposal 5: MN and SN can configure the RV-QoE separately.

After the MN configures the legacy QoE, the MN may configure the RV-QoE to UE. Then MN also need send the available metrics and RRC ID to SN. And then the SN can configure the RV-QoE with same or different RRC ID for SN. How the SN sends the RV-QoE configuration to UE may have several options: 1. Via SRB3; 2. Send to MN, then MN sends to UE. If the option2 is used, the configuration from SN should carry the indication of SN node. So the UE may differentiate the RV-QoE configuration and report according the RRC ID or SN node indication.

Proposal 6: SN can send RV-QoE configuration via SRB3 or via MN with SN indication.

Proposal 7: UE may differentiate the RV-QoE configuration and report according to the RRC ID or SN node indication.

If we support the MN and SN configures the RV-QoE separately, the RV-QoE report should be separately sent to the configuration initialled node. There are several options for how to send the report to the configuration initialled node.

1). UE AS layer directly sends the report to the configuration initialled node, to MN via SRB4, to SN via SRB3 or new SRB in SN

2). UE AS layer sends the RV-QoE reports of MN and SN to one node, either MN or SN. In this case, the configuration and report should carry the indication of the node. Otherwise, the report cannot be relay to the configuration initialled node

Proposal 8: UE AS layer can directly send the RV QoE report to configuration initialled node separately or send the RV-QoE report of MN and SN to one node carrying the indication of the node.

# 3 Conclusions

According to the above discussion, the following proposals are made:

Proposal 1: When the MN receives the QoE configuration, MN can send the QoE configuration to UE via SRB1 directly.

Proposal 2: When SN receives the QoE configuration, the SN needs to send the QoE configuration to MN and then decides whether send the QoE configuration to UE according to the feedback of MN.

Proposal 3: The UE can transfer the QoE report to SN using a new SRBx.

Proposal 4: When the node configured the QoE measurement is overload or the radio quality is not good enough, UE can send the QoE report via the other node based on the received command.

Proposal 5: MN and SN can configure the RV-QoE separately.

Proposal 6: SN can send RV-QoE configuration via SRB3 or via MN with SN indication.

Proposal 7: UE may differentiate the RV-QoE configuration and report according to the RRC ID or SN node indication.

Proposal 8: UE AS layer can directly send the RV QoE report to configuration initialled node separately or send the RV-QoE report of MN and SN to one node carrying the indication of the node.

# 4 References

- Draft_R2_119bis-e_meeting_report

- R3-225301 Report of 3GPP TSG RAN3 meeting #117-e

- Draft Report of 3GPP TSG RAN3 meeting #118-e

- R2-2209330_R3-225256  LS to RAN2 on RAN3 agreement of QoE reporting in NR-DC