3GPP TSG-RAN WG2 Meeting #121	R2-2301805

Athens, Greece, 27th February – 3rd March 2023

Source:	III

Title:	Discussion on XR-specific capacity improvements

Agenda item:	8.5.4

Document for:	Discussion

- Introduction

During the RAN2#120-e online discussion [1], the following objectives were noted:

-	RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay.

-	RAN2 needs to discuss additional BSR triggering conditions to allow timely availability of buffer status information at gNB. This can be discussed in WI phase.

Topics relative to XR-specific capacity issues was raised as mentioned above. In this contribution, we will address “Importance of remaining time” and “BSR enhancements (with delay aware scheduling)”.

- Discussion

After previous RAN2 120-e meetings, there are some issues need to be further discuss,

-	RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay.

-	RAN2 needs to discuss additional BSR triggering conditions to allow timely availability of buffer status information at gNB. This can be discussed in WI phase.

To enhance capacity for XR-specific, the following issues need to be further considered:

- Importance of remaining time

- BSR enhancements (with delay aware scheduling)

  - Importance of remaining time

XR traffic from the application layer will be processed into several PDU sets. Each PDU set can consist multiple IP packets for XR traffic, i.e. I-frame/I-slice(s) or P-frame/P-slice(s). Each IP packet will become a PDU and every PDU Session has its QoS rule, including the requirement of PDU Sets Delay Budget (PSDB) and PDU Sets Error Rate (PSER). Normally XR traffic is transparent to the 3GPP network and UE/gNB will treat each PDU independently. However, to increase capacity, UE/gNB can drop PDUs belong to the same PDU set after any of them have violate the PSDB constrain based on the evaluation of XR traffic as mentioned in [2]. According to the objective to the revised SID above, in order to handle the dropping of the PDU properly, UE/gNB should have information from upper layer to know which PDU(s) belong to a PDU set, e.g. PDU set SN.

Meanwhile, a delay aware scheduler is also discussed in [2]. For a delay aware scheduling model, UE/gNB will take the remaining delay time of a PDU set into consideration to increase its overall capacity performance. If the remaining delay time of a PDU set is limited, PDU(s) of the PDU set waiting to be scheduled will be prioritized even though these PDU(s) are in the low priority at the beginning.

Hence, the priority of a PDU must be adjusted dynamically if a delay aware scheduling mechanism is enabled. Consequently, the existing prioritization mechanism won’t provide adequate PDU session treatment to the UE/gNB for XR application. Because the UE/gNB should schedule these PDU(s) according to the remaining delay time as its priority level(s), especially for time sensitive XR PDU(s), delay budget information relevant to a PDU must be considered by the scheduler of the UE/gNB. For example, the accurate remaining delay budget information of PDU(s) could be derived from the time stamp of the input XR PDU set(s).

Proposal 1: Add remaining delay time related PDU set based parameters e.g. remaining delay budget in L1/L2 for XR UL direction.

Proposal 2: How to derive the remaining delay time information and/or provide the time stamp information of a PDU toward RAN should be studied.

  - BSR enhancements (with delay aware scheduling)

A delay aware scheduler is discussed in [2]. For a delay aware scheduling model, UE/gNB will take the remaining delay time of a PDU set into consideration to increase its overall capacity performance. If the remaining delay time of a PDU set is limited, PDU(s) of the PDU set PDU(s) of the PDU set waiting to be scheduled will be prioritized even though these PDU(s) are in the low priority at the beginning.

Hence, the priority of a PDU must be adjusted dynamically if a delay aware scheduling mechanism is enabled. Consequently, the existing prioritization mechanism won’t provide adequate PDU session treatment to the gNB for XR application. Because the UE/gNB should schedule these PDU(s) according to the remaining delivery time as its priority level(s), especially for time sensitive XR PDU(s), delay budget information relevant to a PDU must be considered by the scheduler of the UE/gNB. For example, the accurate delay budget information of PDU(s) could be derived from the time stamp of the input XR PDU set(s).

In addition to the original data selection rules of BSR, a UE can select the data to be reported according to the remaining delay budget to improve the XR capacity. We propose that a delay aware scheduling should be required to enhance the BSR procedure to resolve the “violate PSDB” situation.  Moreover, since the UL resources are scheduled by gNB, sharing the remaining delay time information of each PDU set between UE and gNB side through BSR, which can also help data scheduling to improve the XR capacity.

Proposal#3: A delay aware scheduling should be required to enhance the BSR procedure to resolve the “violate PDB” situation.

Proposal#4: Sharing the remaining delay time information of each PDU set between UE and gNB through BSR to improve XR capacity.

- Conclusion

The related proposals and observations from above discussions are below:

Proposal 1: Add remaining delay time related PDU set based parameters e.g. remaining delay budget in L1/L2 for XR UL direction.

Proposal 2: How to derive the remaining delay time information and/or provide the time stamp information of a PDU toward RAN should be studied.

Proposal#3: A delay aware scheduling should be required to enhance the BSR procedure to resolve the “violate PDB” situation.

Proposal#4: Sharing the remaining delay time information of each PDU set between UE and gNB through BSR to improve XR capacity.

References

[1] 	R2-2213002, Report on LTE legacy, DCCA, MUSIM, Slicing, 71 GHz, XR and QoE, Nokia.

[2] 	TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17),” V17.0.0.