3GPP TSG-RAN WG2 Meeting #121	                                                                                 R2-2301834

Athens, Greece, February-March, 2023

Agenda Item:    	8.5.3

Source: 		III.

Title:			Discussion on various frame rates supported for XR-specific power saving

Document for:		Discussion

1 Introduction

In previous RAN2 meetings, the following agreements were reached for XR-specific power saving [1]:

RAN2#119-e: - 1: RAN2 to focus on the following issues for power saving, as well necessary parameters XR-awareness to support such enhancements, i.e.: - DRX enhancements to address the issues of DRX cycle mismatch and jitter RAN2#119bis-e: 1: At least RRC pre-configuration and switching of configurations of DRX could be considered for enhancements of XR power saving. Other solutions are not precluded and can be further discussed. RAN2#120: 1: DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling).

This contribution will discuss various frame rates supported for the DRX cycle mismatch in XR-specific power saving.

2 Discussion

2.1 Background: Align DRX cycle to non-integer traffic periodicity

The mismatch between non-integral periods of XR traffic and legacy DRX periods was discussed in study phase. It will introduce extra latency. According to [2], several mechanisms were introduced.

Option 1: This will align the traffic arrival interval with two DRX cycles in a DRX configuration. As shown in Figure 1, there are two DRX cycles: 17ms and 16ms. The two DRX cycles are applied in a sequence of [16, 17, 17].

Figure 1.  DRX cycle pattern (e.g. 16, 17, 17)

Option 2: Directly configure non-integer DRX cycle (e.g.16.67ms) that matches XR traffic periodicity.

Option 3: Adjust drx-startoffset to align traffic arrival interval and DRX cycle. As shown in Figure 2, the alignment will be adjust every three DRX cycles.

Figure 2.  drx-startoffset changed periodical to keep aligned

2.2 Various frame rates supported for the DRX cycle mismatch

In TR38.835 5.5.2 Layer 2 Enhancements, it mentioned XR frame rates (15, 30, 45, 60, 72, 90 and 120fps) correspond to periodicities which are not an integer (66.66, 33.33, 22.22, 16.66, 13.88, 11.11 and 8.33ms respectively) [1]. The technical proposal should consider above mentioned XR frame rates.

The mechanism adjustment/modification for the above 3 options is as following,

For option 1, 2 DRX cycle values is configured (e.g. 16ms and 17ms in IE DRX-Config: drx-LongCycleStartOffset value). Legacy long DRX cycle only configure with one value (in ms), how to configure two DRX cycle values need to further discussed, when XR traffic is 60fps (50/3ms), DRX cycle pattern needs 3-cycle period. In TR38.835, there mentioned many XR frame rate, design different suitable DRX cycle pattern is needed. For XR frame rates (15, 30, 60 and 120fps), 3-cycle period sequence is suitable. For XR frame rates (45, 72 and 90fps), 9-cycle period sequence is suitable. In general, design 9-cycle period pattern is sufficient for whole XR frame rates (15, 30, 45, 60, 72, 90 and 120fps) in TR 38.385.

Observation 1: 9-cycle period sequence of 2 DRX cycle values can be considered.

For option 2, Modification of IE DRX-Config by adding non-integer DRX cycle value (66.66, 33.33, 22.22, 16.66, 13.88, 11.11 and 8.33ms respectively). How to modify the DRX cycle value for recurring decimal of non-integer DRX cycles, it may need more bits to approach recurring decimal by floating point (e.g. 16.67ms) or rational number (two integer values to express recurring decimal, e.g. 50/3ms).

Observation 2: The spec impact need to further clarify.

For option 3, insert drx-startoffset(s) in each cycle period for aligning the non-integer traffic interval. Due to minimize the average delay time of DRX cycle starting, the drx-startoffset inserting can’t be done after each cycle period. Insert drx-startoffset(s) of 1 uniformly over the cycle period will be a feasible solution. The uniformly adjustment of drx-startoffset in each 9-cycle period can be considered.

Observation 3: The adjustment of drx-startoffset uniformly in each 9-cycle period can be considered.

Observation 4: Observation 1 and observation 3 are equivalent in the sense of average delay time, but different spec impact.

BTW, two mechanisms was proposed by Qualcomm [3] in RAN2#120 for non-integer DRX cycles. And the two related formulas can be extended to more frame rates for DRX cycle mismatch.

- Conclusions

The following observations were made:

Observation 1: 9-cycle period sequence of 2 DRX cycle values can be considered.

Observation 2: The spec impact need to further clarify.

Observation 3: The adjustment of drx-startoffset uniformly in each 9-cycle period can be considered.

Observation 4: Observation 1 and observation 3 are equivalent in the sense of average delay time, but different spec impact.

4 References

- TR 38.835 Study on XR enhancements for NR, November 2022.

- R1-2205412, Final Moderator Summary on XR specific power saving techniques, Qualcomm, August 2022.

- R2-2211180, DRX enhancements for XR, Qualcomm, November 2022.