3GPP TSG RAN WG2 #121	 R2-2301855

Athens, Greece, 27th February– 3rd Marth, 2023

Agenda Item:	6.9.4

Source: 	China Unicom, CMCC, CATT

Title:  	Introduction of packet loss rate with delay threshold

Document for:	Discussion and decision

Introduction

This document proposes a new formula to calculate packet loss rate with delay threshold to match the definition of PER for the QoS flows with resource type of delay-critical GBR.

Discussion

Definition and measurement for PER

Definitions and characteristics of QoS Flow are specified in TS 23.501. QoS Flows can be categorised into three types according to description in clause 5.7.3.1 of TS 23.501: Non-GBR QoS Flow, GBR QoS Flow and delay-critical GBR QoS Flow, according to resource type.

Observation 1: PER (packet error rate) is defined in TS 23.501 as a characteristic of a QoS Flow and it has different meanings for GBR QoS Flows with Delay-critical GBR resource type compared to other QoS Flows.

In clause 5.7.3.4 and 5.7.3.5 of TS 23.501, definitions for PDB and PER are depicted as follows:

********************************************************************************************************************

#### 5.7.3.4	Packet Delay Budget

The Packet Delay Budget (PDB) defines an upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF. The PDB applies to the DL packet received by the UPF over the N6 interface, and to the UL packet sent by the UE. For a certain 5QI the value of the PDB is the same in UL and DL. In the case of 3GPP access, the PDB is used to support the configuration of scheduling and link layer functions (e.g. the setting of scheduling priority weights and HARQ target operating points). For GBR QoS Flows using the Delay-critical resource type, a packet delayed more than PDB is counted as lost if the data burst is not exceeding the MDBV within the period of PDB and the QoS Flow is not exceeding the GFBR. For GBR QoS Flows with GBR resource type not exceeding GFBR, 98 percent of the packets shall not experience a delay exceeding the 5QI's PDB.

The 5G Access Network Packet Delay Budget (5G-AN PDB) is determined by subtracting a static value for the Core Network Packet Delay Budget (CN PDB), which represents the delay between any N6 termination point at the UPF (for any UPF that may possibly be selected for the PDU Session) and the 5G-AN from a given PDB.

NOTE 1:	For a standardized 5QI, the static value for the CN PDB is specified in the QoS characteristics Table 5.7.4-1.

NOTE 2:	For a non-standardized 5QI, the static value for the CN PDB is homogeneously configured in the network.

For GBR QoS Flows using the Delay-critical resource type, in order to obtain a more accurate delay budget PDB available for the NG-RAN, a dynamic value for the CN PDB, which represents the delay between the UPF terminating N6 for the QoS Flow and the 5G-AN, can be used. If used for a QoS Flow, the NG-RAN shall apply the dynamic value for the CN PDB instead of the static value for the CN PDB (which is only related to the 5QI). Different dynamic value for CN PDB may be configured per uplink and downlink direction.

#### 5.7.3.5	Packet Error Rate

The Packet Error Rate (PER) defines an upper bound for the rate of PDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PER defines an upper bound for a rate of non-congestion related packet losses. The purpose of the PER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access). For every 5QI the value of the PER is the same in UL and DL. For GBR QoS Flows with Delay-critical GBR resource type, a packet which is delayed more than PDB is counted as lost, and included in the PER unless the data burst is exceeding the MDBV within the period of PDB or the QoS Flow is exceeding the GFBR.

********************************************************************************************************************

Two conclusions can be drawn:

- PDB, used as delay threshold when defining PER for GBR QoS Flows with Delay-critical GBR resource type, is composed of 5G-AN PDB and CN PDB. 5G AN PDB defines packet delay budget of 5G access network, is upper bound of delay in access network and can be used as delay threshold.

- PER for GBR QoS Flows with Delay-critical GBR resource type has different meaning from that of other QoS Flows because the packets transmitted successfully but delayed more than PDB is also counted as lost for Delay-critical GBR.

Observation 2: The numerator of the measurement for DL packet loss rate formula defined in TS 38.314 doesn’t contain the packets that are transmitted successfully but delayed, which is only suitable for QoS Flows with non-GBR resource type and GBR resource type, but not suitable for Qos Flows with delay-critical GBR resource type.

In clause 4.2.1.5.1 of TS 38.314, a measurement named packet Uu loss rate in the DL per DRB per UE is defined as follows:

********************************************************************************************************************

Table 4.2.1.5.1-1: Definition for Packet Uu Loss Rate in the DL per DRB per UE

| Definition | Uu Packet Loss Rate in the DL per DRB per UE. One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed Definition: M(T,drbid)=Dloss(T,drbid)*1000000N(T,drbid)+Dloss(T,drbid), where explanations can be found in the table 4.2.1.5.1-2 below. |
| --- | --- |

NOTE 1:	Packet loss is expected to be upper bounded by the PER (packet error rate, as defined in TS 23.501 [4]) of the DRB which takes values between 10-6 and 10-2. The statistical accuracy of an individual packet loss rate measurement result is dependent on how many packets have been received, and thus the time for the measurement.

NOTE 2:	The granularity for Packet loss rate measurement is per DRB per UE, as defined in TS 28.552 [2].

Table 4.2.1.5.1-2: Parameter description for Packet Uu Loss Rate in the DL per DRB per UE

| M(T,drbid) | Packet Loss Rate in the DL per DRB per UE. Unit: number of lost packets per transmitted packets per DRB * 106, Integer. |
| --- | --- |
| Dloss(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, for which at least a part has been transmitted over the air but not positively acknowledged, and it was decided during time period T that no more transmission attempts will be done. If transmission of a packet might continue in another cell, it shall not be included in this count. |
| N(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, which has been transmitted over the air and positively acknowledged during time period T. |
| T | Time Period during which the measurement is performed, Unit: minutes. |
| drbid | The identity of the measured DRB. |

********************************************************************************************************************

The numerator of M(T,drbid)  is Dloss(T,drbid) which only counts the packets transmitted over the air not positively acknowledged. Through this kind of calculation method, the result only matches the PER definition for QoS Flows with non-GBR resource type and GBR resource type. As for that of QoS Flows with delay-critical GBR resource type, the measurement is not appropriate because the numerator haven't taken into account the number of packets which is transmitted over the air and positively acknowledged but delayed more than delay threshold.

Three conclusions can be drawn:

- The existing measurement of packet loss rate in TS 38.314 is only suitable for QoS Flows with non-GBR resource type and GBR resource type which can’t cover all the types of QoS Flow.

- When it comes to QoS Flows with delay-critical GBR resource type, the measurement algorithm of packets loss rate doesn’t align with the definition of PER which is used as upper bound of the measurement.

- A new measurement needs to be introduced to meet the definition of PER for delay-critical GBR resource type which taking the delay threshold (AN-PDB, part of PDB as defined in TS23.501) into consideration.

Proposal 1: It is proposed to introduce a new measurement of packet loss rate with delay threshold to align with the definition of PER for QoS Flow with delay-critical GBR resource type.

A new measurement designed for delay-critical GBR QoS Flow is proposed to be added in TS 38.314 so as to covering all types of QoS Flows when calculating packet loss rate which uses PER as upper bounded for evaluation.

38.314 CR to introduce packet loss rate with delay threshold

Based on the proposed solution in P1, the 38.314 CR to introduce DL packet loss rate with delay threshold is also proposed as follows:

<< Start of changes >>

#### 4.2.1.5	Packet Loss Rate

##### 4.2.1.5.1	Packet Uu Loss Rate in the DL per DRB per UE

The objective of this measurement is to measure packets that are lost at Uu transmission, for OAM performance observability or for QoS verification of MDT.

Protocol Layer: RLC

Table 4.2.1.5.1-1: Definition for Packet Uu Loss Rate in the DL per DRB per UE

| Definition | Uu Packet Loss Rate in the DL per DRB per UE. One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed Definition: M(T,drbid)=Dloss(T,drbid)*1000000N(T,drbid)+Dloss(T,drbid), where explanations can be found in the table 4.2.1.5.1-2 below. |
| --- | --- |

NOTE 1:	Packet loss is expected to be upper bounded by the PER (packet error rate, as defined in TS 23.501 [4]) of the DRB which takes values between 10-6 and 10-2. The statistical accuracy of an individual packet loss rate measurement result is dependent on how many packets have been received, and thus the time for the measurement.

NOTE 2:	The granularity for Packet loss rate measurement is per DRB per UE, as defined in TS 28.552 [2].

Table 4.2.1.5.1-2: Parameter description for Packet Uu Loss Rate in the DL per DRB per UE

| M(T,drbid) | Packet Loss Rate in the DL per DRB per UE. Unit: number of lost packets per transmitted packets per DRB * 106, Integer. |
| --- | --- |
| Dloss(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, for which at least a part has been transmitted over the air but not positively acknowledged, and it was decided during time period T that no more transmission attempts will be done. If transmission of a packet might continue in another cell, it shall not be included in this count. |
| N(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, which has been transmitted over the air and positively acknowledged during time period T. |
| T | Time Period during which the measurement is performed, Unit: minutes. |
| drbid | The identity of the measured DRB. |

##### 4.2.1.5.x	Packet Uu Loss Rate with delay threshold in the DL per DRB per UE

The objective of this measurement is to measure the DL packets loss including any packets not successfully transmitted or delayed more than a delay threshold at Uu transmission, for OAM performance observability or for QoS verification of MDT.

Protocol Layer: RLC

Table 4.2.1.5.x-1: Definition for Packet Uu Loss Rate with delay threshold in the DL per DRB per UE

| Definition | Uu Packet Loss Rate with daley threshold in the DL per DRB per UE: One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed definition: M_dt(T,drbid)=[DlossT,drbid+DexdT, drbid]*1000000N_dtT,drbid+DlossT,drbid+DexdT, drbid Where explanations can be found in the table 4.2.1.5.x-2 below. |
| --- | --- |

NOTE 1:	Packet loss rate with delay threshold can be used when the resource type of corresponding QoS Flow is Delay-critical GBR. It is expected to be upper bounded by the PER (packet error rate, as defined in TS 23.501[4]) of the DRB which takes values between 10-6 and 10-2. The statistical accuracy of an individual packet loss rate measurement result is dependent on how many packets have been received, and thus the time for the measurement.

NOTE 2:	Delay threshold of this measurement is determined by 5G-AN PDB defined in TS 23.501.

NOTE 3:	The granularity for Packet loss rate measurement is per DRB per UE, as defined in TS 28.552 [2].

Table 4.2.1.5.x-2: Parameter description for Packet Uu Loss Rate with delay threshold in the DL per DRB per UE

| M_dt(T,drbid) | Packet Loss Rate with delay threshold in the DL per DRB per UE. Unit: number of lost packets per transmitted packets per DRB * 106, Integer. Lost packets here means the packets that delayed more than delay threshold or not successfully transmitted. |
| --- | --- |
| Dloss(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, for which at least a part has been transmitted over the air but not positively acknowledged, and it was decided during time period T that no more transmission attempts will be done. If transmission of a packet might continue in another cell, it shall not be included in this count. |
| Dexd(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, for which is transmitted over air interface and positively acknowledged but the DL delay of the RLC SDU is more than corresponding 5G-AN PDB during time period T. The DL delay of a RLC SDU is calculated as defined in clause 5.1.1.1 in TS 28.552. |
| N_dt(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, which has been transmitted over the air and positively acknowledged and delayed no more than the threshold, 5G-AN PDB, during time period T. |
| T | Time Period during which the measurement is performed, Unit: minutes. |
| drbid | The identity of the measured DRB. |

<< End of change >>

Summary

RAN2 is kindly asked to discuss the following observation and proposals:

Observation 1: PER (packet error rate) is defined in TS 23.501 as a characteristic of a QoS Flow and it has different meanings for GBR QoS Flows with Delay-critical GBR resource type compared to other QoS Flows.

Observation 2: The numerator of the measurement for DL packet loss rate formula defined in TS 38.314 doesn’t contain the packets that are transmitted successfully but delayed, which is only suitable for QoS Flows with non-GBR resource type and GBR resource type, but not suitable for Qos Flows with delay-critical GBR resource type.

Proposal 1: It is proposed to introduce a new measurement of packet loss rate with delay threshold to align with the definition of PER for QoS Flow with delay-critical GBR resource type.

References

- 3GPP TS 38.314 “NR; Layer 2 measurements”

- 3GPP TS 23.501 “Technical Specification Group Services and System Aspects; System architecture for the 5G System (5GS); Stage 2”