3GPP TSG-RAN WG2 Meeting #121                                                 	R2-2301876

Athens, Greece, 27th – 3rd Mar, 2023

Source:	CATT

Title:	Leftover Issues on Flight Path Reporting

Agenda Item:	8.8.3

Document for:	Discussion and Decision

Introduction

In RAN2#120, the following agreements were reached for flight path reporting issue:

Agreements:

- A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355.

- A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline.  FFS on granularity

- No requirements are placed on spatial distribution of waypoints

- A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message.   Flight path reporting uses at the UE Information request/response procedure as baseline.

- UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report.

- UAI message can also be used to indicate the UE has flight path availability.

- FFS whether and what triggering conditions are specified for flight update.  FFS The maximum number of waypoints within flight path plan is left FFS.

In this contribution, we will discuss the leftover issues and provide our view.

Discussion

Granularity of timestamp

In RAN2#120 meeting, the issue about the granularity of timestamp was raised. There are two options: one is the IE AbsoluteTimeInfo indicates an absolute time in a format YY-MM-DD HH:MM:SS in LTE. Another is the IE DisplacementTimeStamp in NR. The main difference is the granularity for NR(ms) is finer. One issue which one is more suitable to NR UAV. In 36.777[1], it recorded the deployment scenarios and assumptions for UAV:

The maximum target height and the maximum horizontal speed requirement for aerial vehicles are 300 m AGL and 160 km/h, respectively. The maximum horizontal speed requirement is applicable to both urban and rural scenarios.

It records that the maximum horizontal speed is 160 km/h, 160km/h≈45m/s=4.5cm/ms. That is to say, the maximum horizontal displacement is 4.5cm per millisecond. Considering the current NR positioning accuracy is decimeter, 4.5cm maximum horizontal displacement seems useless. Hence, we think the granularity of timestamp for LTE can be reused.

Proposal 1: The granularity of timestamp for LTE UAV is reused for NR UAV.

Triggering condition for flight path

Due to time limitation, the triggering condition for flight path and max number of waypoints were summarized but not discussed. In RAN2#120 meeting, one contribution [2] raised to confirm whether the following cases are to be considered for flight path update:

- Case 1: Flight path update due to the changed waypoint and/or timestamp.

- Case 2: Flight path update due to the outdated (passed) waypoint.

Our point is the discussion of the above issue is out of RAN2 scope. Due to the use of flight path is in the scope of upper layer, we propose to leave this flight path update issue to upper layer.

Proposal 2: The update of flight path for NR UAV is requested by upper layer.

Max number of waypoints

In LTE, the maximum number of waypoints is defined as 20.

maxWayPoint-r15				INTEGER ::= 20	-- Maximum number of flight path information waypoints

In our understanding, the maximum number of waypoints depends on the need for NR UAV. Since the performance requirement (e.g. accuracy) for NR UAV seems the same as LTE UAV. It is nature to follow the LTE in the current release.

Proposal 3: RAN2 agree reuse the maximum number of waypoints in LTE UAV.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: The granularity of timestamp for LTE UAV is reused for NR UAV.

Proposal 2: The update of flight path for NR UAV is requested by upper layer.

Proposal 3: RAN2 agree reuse the maximum number of waypoints in LTE UAV.

- Reference

- TR 36.777 V15.0.0 Study on Enhanced LTE Support for Aerial Vehicles (Release 15)

- R2-2211766	 On measurement reporting enhancements in NR UAV Samsung Electronics Co., Ltd