3GPP TSG-RAN WG2 #121	R2-2301883

Athens, 27th February – 3rd March 2023

Source:                    	DENSO CORPORATION

Title:  	Consideration on flight path reporting of NR support for UAV

Document for:        	Discussion and decision

Agenda Item:         	8.8.3 	Flight path reporting

- Introduction

In the WID of NR support for UAV [1], it is mentioned that the following enhancements should be specified for Measurement Reporting.

- UE-triggered measurement report based on configured height thresholds - Reporting of height, location and speed in measurement report - Flight path reporting - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously

- A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355. - A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline. FFS on granularity - No requirements are placed on spatial distribution of waypoints - A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message. Flight path reporting uses at the UE Information request/response procedure as baseline. - UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report. - UAI message can also be used to indicate the UE has flight path availability. - FFS whether and what triggering conditions are specified for flight update. FFS The maximum number of waypoints within flight path plan is left FFS.

Then, based on the chair’s note for RAN2 #120 meeting [2], the following have been agreed about the flight path reporting.

In accordance with the above agreements including FFS, this paper attempts to investigate about the possible format for the flight path report and further enhancements.

- Discussion

  - Possible format for the flight path report

Based on the agreements made during last meeting, we assume that basic message structure of LTE flight path report could be reused for NR.

FlightPathInfoReport-r15 ::=		SEQUENCE {

flightPath-r15	SEQUENCE (SIZE (1..maxWayPoint-r15)) OF WayPointLocation-r15	OPTIONAL,

dummy							SEQUENCE {}							OPTIONAL

}

WayPointLocation-r15 ::=			SEQUENCE {

wayPointLocation-r15						LocationInfo-r10,

timeStamp-r15							AbsoluteTimeInfo-r10		OPTIONAL

}

Figure 1. Current LTE message structure of flight path report[3]

With the above assumption, updating the flight path would be one of consideration point on investigating the flight path format, as it was agreed to allow UE to update the flight path. We suppose that one of usecase to update flight path would be destination change by remote operation. In this case, entire flight path may be updated. As another example of usecase, time delay due to external condition (e.g. weather, congestion, etc.) would also be supposed. In such case, sending the entire flight path would be inefficient since the location information might not be updated but the timestamp should be updated.

Observation 1:	It would be beneficial to allow UE to update reported flight path partially but not only entire flight path.

FlightPathInfoReport-r18 ::=		SEQUENCE {

flightPath-r18					SEQUENCE (SIZE (1..maxWayPoint-r18)) OF WayPointLocation-r18,

nonCriticalExtension				SEQUENCE {}

}

WayPointLocation-r18 ::=	SEQUENCE {

index							INTEGER (0..XXXX),

wayPointLocation-r18				LocationInfo-r16             OPTIONAL,

timeStamp-r18						AbsoluteTimeInfo-r16         OPTIONAL

}

To update entire or partial flight path, introducing index for each waypoint would work. With the index, UE can specify the waypoint to be updated, and UE can send only difference from the last report if the UE intend to update the flight path partially. Therefore, the proposed format of flight path would be like below:

Figure 2. Proposed message structure of flight path

Proposal 1: RAN2 to introduce index for each waypoint so that UE can update entire or partial flight path. Additionally, define wayPointLocation as optional field to allow UE to update timeStamp only.

  - Triggering conditions for updating flight path

According to the chair’s note for RAN2 #120 meeting, it is mentioned that triggering condition for flight path update is FFS. The triggering condition could be left up to UE implementation, however, configuring the triggering condition would be beneficial to keep the effectiveness of the flight path information. For example, the network configures the UE to report updated flight path when UE detects significant difference between the latest estimated waypoints and the previous reported waypoints. In the case, the network may configure the tolerance of difference and the frequency of estimation for the waypoints. Additionally, the mechanism of ReportConfig for measurement report could be used for the triggering condition configuration (D1 event could be referred to as distance-based event).

Proposal 2:	RAN2 to introduce the triggering condition for flight path update to keep the effectiveness of the flight path information. For example, the network configures the UE to report updated flight path when UE detects significant difference between the latest estimated waypoints and the previous reported waypoints.

- Summary and proposal

This paper discussed the consideration on flight path reporting of NR support for UAV. In summary, the followings were observed and proposed:

Observation 1:	It would be beneficial to allow UE to update reported flight path partially but not only entire flight path.

Proposal 1: RAN2 to introduce index for each waypoint so that UE can update entire or partial flight path reported. Additionally, define wayPointLocation as optional field to allow UE to update timeStamp only.

Proposal 2:	RAN2 to introduce the triggering condition for flight path update to keep the effectiveness of the flight path information. For example, the network configures the UE to report updated flight path when UE detects significant difference between the latest estimated waypoints and the previous reported waypoints.

- References

[1] RP-223545, “WI on NR Support for UAV”

[2] R2-2213003, “Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning”

[3] TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification”