3GPP TSG-RAN WG2 Meeting #121                                           	R2-2301891

Athens, 26th Feb – 03rd March 2023

Source:	Telit Cinterion

Title:	DL Positioning

Agenda Item:	8.2.4

Document for:	Discussion and Decision

# Introduction

In the last RAN2 meeting an email discussion related to LPHAP upper layer enhancements was discussed. The discussion entitled various proposals and discussions, but this document specifically focuses on following agreements made in the meeting:

	Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4]

•	Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning.

	NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap.

•	NOTE: Inputs from RAN1 as necessary may be facilitated via LSs

	For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3].

•	SRS for positioning configurations in multiple cells [RAN2, RAN1].

	Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work.

•	Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3].

•	SRS for positioning activation/request procedure(s) [RAN2, RAN1].

	Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2].

	Specify solutions for alignment between eDRX and PRS configurations [RAN2].

	Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution we will present Telit point of view regarding which Alternative to use regarding candidate solutions to report measurements taken in RRC_IDLE.

# Discussion

This topic has received a lot of consideration and reporting of measurements in connected mode among two alternatives have been discussed in previous meetings as well.

Alt1: measurement is performed in IDLE and reported in CONNECTED

Alt2: measurement is performed in IDLE, and report is carried with initial access messages.

Telit would like to argue for Alt 1 which discusses about measurements performed in IDLE and then reported during connected mode which according to our understanding will be more efficient in terms of power saving.

It also depends on the situation and urgency of the UE. UE can do measurements in RRC_IDLE and then wait till it goes to connected mode and report. It can also immediately transit to RRC_Connected mode straight away to report measurements and sending the position depending on urgency.

If the UE is not moving it can wait and send measurements when it goes to connected mode next time. This way it saves battery for UE as well. Also as pointed out by some companies the measurements UE has to do for positioning are big and they will need loads of transmissions to report that which will require longer time and hence more power consumption. So UE can as argued above, depending upon the urgency can send the measurements report during connected mode which will make it efficient.

This alternative will have a very less RAN impact as well and we can continue to focus on other issues.

Otherwise we agree with Lenovo and can also ask RAN 1 for their opinion about the power saving evaluation for both alternatives.

Proposal 1: Measurement is performed in IDLE and reported in CONNECTED mode.

Proposal 2: Depending upon the urgency of report required i.e. UE can immediately transit to connected mode for urgent report.

Proposal 3: If UE is not moving it can wait till it goes to connected mode next time before reporting the measurements.

# Conclusion

Proposal 1: Measurement is performed in IDLE and reported in CONNECTED mode.

Proposal 2: Depending upon the urgency of report required i.e. UE can immediately transit to connected mode for urgent report.

Proposal 3: If UE is not moving it can wait till it goes to connected mode next time before reporting the measurements.

Proposal 4: We can ask RAN 1 to evaluate power saving benefits for both alternatives mentioned above.

# Reference

[1] R2-2210892	[Pre119bis-e][405] Summary of AI 8.2.3 on RAT-dependent integrity (Samsung

[2] RAN 2_120        Chair’s Report.