3GPP TSG-RAN WG2 Meeting #121	R2-2301914

# Athens, Greece, 27 February – 3 March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.323 | CR | 0118 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Order of performing integrity protection and setting PDCP SN in Transmit operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | By the current specification, for a PDCP SDU received from upper layers, the transmitting PDCP entity shall set the PDCP SN of the PDCP Data PDU after performing integrity protection. However, the “data unit that is integrity protected is the PDU header and the data part of the PDU before ciphering”, meaning that PDCP SN in the header needs to be set before performing integrity protection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The step of setting the PDCP SN of the PDCP Data PDU is moved before performing integrity protection. - Impact analysis Impacted functionality: Data-PDU generation in data-transmit operation Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE will not have a complete Data-PDU header by the time it needs to perform integrity protection (of also the header). - If the UE is implemented according to the CR and the network is not, the network will not have a complete Data-PDU header by the time it needs to perform integrity protection (of also the header). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The transmitting PDCP entity will not have a complete Data-PDU header by the time it needs to perform integrity protection (of also the header). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.2	Data transfer

### 5.2.1	Transmit operation

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	start the discardTimer associated with this PDCP SDU (if configured).

For a PDCP SDU received from upper layers, the transmitting PDCP entity shall:

-	associate the COUNT value corresponding to TX_NEXT to this PDCP SDU;

NOTE 1:	Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[pdcp-SN-SizeUL];

-	perform integrity protection, and ciphering using the TX_NEXT as specified in the clause 5.9 and 5.8, respectively;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[pdcp-SN-SizeUL];

-	increment TX_NEXT by one;

-	submit the resulting PDCP Data PDU to lower layer as specified below.

When submitting a PDCP PDU to lower layer, the transmitting PDCP entity shall:

-	if the transmitting PDCP entity is associated with one RLC entity:

-	submit the PDCP PDU to the associated RLC entity;

-	else, if the transmitting PDCP entity is associated with at least two RLC entities:

-	if the PDCP duplication is activated for the RB:

-	if the PDCP PDU is a PDCP Data PDU:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to the associated RLC entities activated for PDCP duplication;

-	else:

-	submit the PDCP Control PDU to the primary RLC entity;

-	else (i.e. the PDCP duplication is deactivated for the RB or the RB is a DAPS bearer):

-	if the split secondary RLC entity is configured; and

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the primary RLC entity and the split secondary RLC entity is equal to or larger than ul-DataSplitThreshold:

-	submit the PDCP PDU to either the primary RLC entity or the split secondary RLC entity;

-	else, if the transmitting PDCP entity is associated with the DAPS bearer:

-	if the uplink data switching has not been requested:

-	submit the PDCP PDU to the RLC entity associated with the source cell;

-	else:

-	if the PDCP PDU is a PDCP Data PDU:

-	submit the PDCP Data PDU to the RLC entity associated with the target cell;

-	else:

-	if the PDCP Control PDU is associated with source cell:

-	submit the PDCP Control PDU to the RLC entity associated with the source cell;

-	else:

-	submit the PDCP Control PDU to the RLC entity associated with the target cell;

-	else:

-	submit the PDCP PDU to the primary RLC entity.

NOTE 2:	If the transmitting PDCP entity is associated with two RLC entities, the UE should minimize the amount of PDCP PDUs submitted to lower layers before receiving request from lower layers and minimize the PDCP SN gap between PDCP PDUs submitted to two associated RLC entities to minimize PDCP reordering delay in the receiving PDCP entity.