3GPP TSG-RAN WG2 Meeting #121											R2-2301926

Athens, Greece, 27th Feb.– 3rd Mar, 2023

Agenda item:       5.2.3

Source: 	LG Electronics

Title: 	Summary on MAC CRs

Document for:	Discussion and Decision

Introduction

This document summarizes the contributions submitted to AI 5.2.3. Rapporteur’s recommendations on how to treat them are provided.

Correction CRs

The following correction CRs could be discussed together. The agreed changes for MAC spec could be merged into a Rapporteur’s correction CR and its Rel-17 mirror CR.

Table 1: Correction CRs

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2300834 | Correction on resource (re-)selection for NR sidelink | Xiaomi | Reason for change - During the resource selection procedure for a single MAC PDU, UE needs to select the time and frequency resources according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL-CSI reporting. However, during the resource selection procedure for multiple MAC PDU, the latency requirement of the triggered SL-CSI reporting is not considered. Proposal - In section 5.22.1.1, during resource selection procedure for multiple MAC PDU, add corresponding descriptions to consider the latency requirement of the triggered SL-CSI reporting. 1> if the MAC entity has selected to create a selected sidelink grant corresponding to transmissions of multiple MAC PDUs, and SL data is available in a logical channel: ~ 3> if transmission based on random selection is configured by upper layers: 4> randomly select the time and frequency resources for one transmission opportunity from the resources pool, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL-CSI reporting. 3> else: 4> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL-CSI reporting. | It is correct that the part considering the latency requirement of triggered SL-CSI reporting is missing from the multiple MAC PDU procedure. The correction can be merged into the Rapporteur CR. the correction can be discussed at the CR discussion. |
| R2-2300861 | Correction on the cast type indicator setting of MAC PDU only containing MAC CE | CATT | Reason for change: According to MAC spec, when setting the cast type in SCI for a MAC PDU, it stated that: 5> set the cast type indicator to one of broadcast, groupcast and unicast as indicated by upper layers;. But if the MAC PDU containing only MAC CE(s), considering the MAC CE is generated by MAC layer, upper layer will not indicate the cast type. Hence, the current MAC spec is not correct. In Rel-16, there is only one SL MAC CE transmitted in PC5, that is Sidelink CSI Reporting MAC CE, only unicast type is supported for it. Proposal: In subclause 5.22.1.3.1, clarify that the cast type indicator should be set as unicast for MAC PDU only containing SL MAC CE(s). | In release 17, there is no need to modify the current text because condition based IUC supports GC/BC. This correction can be discussed at the CR discussion. |
| R2-2301525 | Corrections on MAC reset regarding SL configured grant | ASUSTeK | Reason for change - In the current specification, when a UE performs MAC reset, the UE does not clear configured sidelink grant. This could lead to UE keep using NW configured sidelink resources after the serving cell changes or the NW releases the connection. Proposal - Added that the UE clears configured sidelink grant when performing MAC reset. |  |
|  |  |  | If a reset of the MAC entity is requested by upper layers, the MAC entity shall: ~ 1> clear, if any, configured sidelink grants; | Uplink configured grant is not cleared when a MAC reset occurs in Uu as well. Besides, RAN2 has never discussed or made an agreement on the correction. This correction can be discussed at the CR discussion. |

Proposal: Discuss the CRs listed in Table 1 offline, the agreed changes for MAC spec could be merged into Rapporteur’s correction CR.

Conclusions

The contributions submitted to AI 5.2.3 are summarized, Rapporteur’s recommendations are as follows.

Proposal: Discuss the CRs listed in Table 1 offline, the agreed changes for MAC spec could be merged into Rapporteur’s correction CR.