3GPP TSG-RAN WG2 Meeting #121											R2-2301927

Athens, Greece, 27th Feb.– 3rd Mar, 2023

Agenda item:       6.10.2

Source: 	LG Electronics

Title: 	Summary on user plan CRs

Document for:	Discussion and Decision

Introduction

This document summarizes the contributions submitted to AI 6.10.2. Rapporteur’s recommendations on how to treat them are provided.

Correction CRs

The following correction CRs could be discussed together. The agreed changes for MAC spec could be merged into a Rapporteur’s correction CR.

Table 1: Correction CRs

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2301724 | Discussion on L2 ID for GC/BC IUC | LG | RAN2 decides how L2 GC/BC ID for IUC GC/BC IUC information transmission is specified in MAC specification. - Option 1. The UE can use the GC/BC/UC-related L2 IDs that are already in use or interested in transmitting and receiving IUC messages, and which L2 ID to select from the GC/BC/UC-related L2 IDs that are already in use or interest is up to implementation. - Option 2. Define and use a dedicated L2 GC/BC ID for IUC GC/BC IUC information transmission. It can be discussed together with related submission (i.e., R2-2300130, R2-2300488, R2-2300757, R2-2300896, R2-2301473, R2-2300503, R2-2300838, R2-2300912) proposals. | From a MAC CR rapporteur point of view, I think clear text about supporting the IUC GC/BC should be specified in TS 38.321. Besides, the way to leave it to the UE implementation that can minimize the impact on the specification prefer. RAN2 can discuss this issue together with related submission. |
| R2-2300130 | Discussion on left issues on user plane procedure | OPPO | Proposal 1 For the GC/BC-based transmission of IUC for non-preferred resource set triggered by a condition, RAN2 confirm that it is supported if there is at least an on-going GC/BC communication, from which it is up to UE implementation to select the GC/BC L2 ID(s) for the transmission of IUC. Proposal 2 For the GC/BC-based transmission of IUC for non-preferred resource set triggered by a condition, limit the spec impact to stage-2, e.g., “For transmission of the non-preferred resource set triggered by a condition other than explicit request, groupcast/broadcast manner is also supported”. Proposal 3 RAN2 to discuss to align the priority derivation of IUC-info/IUC-request MAC CE in UL/SL prioritization as the priority derivation for SCI transmission, i.e., 1) rely on RRC parameter when configured, 2) rely on priority-field in the IUC-request (for IUC-info MAC-CE) or up to UE implementation (for IUC-request MAC-CE) when not configured. | Proposal 1/2 (IUC broadcast and groupcast) can be discussed with R2-2301724 and other related contribution. When it comes to the P3 (Priority of IUC/IUC Req MAC CE), UL/SL prioritization in MAC is not performed based on L1 priority. From rapporteur point of view, UL/SL prioritization of IUC MAC CE in MAC can be performed based on the priority value (“1”) of LCP and the MAC priority order below in 5.22.1.4.1.3. - Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - data from SCCH; - Sidelink CSI Reporting MAC CE; - Sidelink Inter-UE Coordination Request MAC CE and Sidelink Inter-UE Coordination Information MAC CE; - Sidelink DRX Command MAC CE; - data from any STCH. RAN2 can discuss the proposal 3 at the online/offline discussion. |
| R2-2300131 | Corrections on user plane for SL enhancement | OPPO | Correction 1 is related with P3 of R2-2300130. Correction 2. It has been agreed that the default SL DRX can be used DCR message. However, the current equation of the sl-drx-StartOffset and sl-drx-SlotOffset are limited to groupcast or broadcast, while unicast-based DCR message with default SL DRX is missing and should be added. Proposed change of correction 2: When the cast type is groupcast or broadcast as indicated by upper layer, or the cast type is unicast for the reception of Direct Link Establishment Request message [28] or ProSe Direct Link Establishment Request message [29] as indicated by upper layer, the sl-drx-StartOffset and sl-drx-SlotOffset are derived from the following equations: sl-drx-StartOffset (ms) = Destination Layer-2 ID modulo sl-DRX-GC-BC-Cycle (ms). sl-drx-SlotOffset (ms) = (Destination Layer-2 ID modulo the number of slots in one subframe) / (the number of slots in one subframe) (ms). | RAN2 can discuss the correction 1 with P3 of R2-2300130. When it comes to the correction 2, the proponent's observation is correct (unicast-based DCR message with default SL DRX is missing). RAN2 can discuss startoffset and slotoffset for Unicast DCR at the online/offline discussion. |
| R2-2300503 | discussion on IUC aspects in case of GC and BC | Ericsson | Proposal 1 Aim for minimal spec efforts to implement the support of GC and BC for IUC information triggered by a condition Proposal 2 Provisioning Destination L2 IDs in AS without involving SA2 for IUC information transmission in GC and BC, i.e., a set of Destination L2 IDs are configured/preconfigured in AS for IUC information transmission in GC and BC. Proposal 3 Up to UE-A implementation to determine a cast type and Destination L2 ID for IUC information which comprises non-preferred resource set and triggered by a condition. | Proposal 1/2/3 can be discussed with R2-2301724 (IUC broadcast and groupcast) and other related contribution. |
| R2-2300755/R2-2300756 | Discussion on the MAC layer procedure for non-preferred resource set | Apple | Proposal 1: TS 38.321 only specifies the generic UE behaviour of “passing non-preferred resource set to PHY” w/o exhausting all resource selection scenarios to handle non-preferred resource set. CR: Removed the handling of non-preferred resource set from 5.22.1.1 and 5.22.1.10.x Reception of IUC-Information Reporting Upon the reception of SL-IUC Information MAC CE from another UE, the MAC entity shall: - if sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is configured by RRC: 2> if the SL-IUC Information MAC CE contains a non-preferred resource set and if the UE is configured to use full sensing or parital sensing in the resource pool where the non-preferred resource set is located: 3> indicate the received non-preferred resource set to physical layer. 2> else if the SL-IUC Information MAC CE contains a preferred resource set: 3> use the preferred resource set in resource (re-)selection procedure as described in 5.22.1.1. | From a MAC CR rapporteur point of view, I agree with the direction of the correction. However, since preferred resource set related text has nothing to do with non-preferred resource set, the correction below should be deleted from the proposed correction. “2> else if the SL-IUC Information MAC CE contains a preferred resource set: 3> use the preferred resource set in resource (re-)selection procedure as described in 5.22.1.1.” The corredtion can be discussed at the online/offline discussioon. |
| R2-2300487 | Corrections on TS 38.321 for SL enhancements | Huawei | 1. RAN1 had confirmed it is a possible scenario in which UE B receives IUC Scheme 1 non-preferred resource set from UE A, but UE B does not perform sensing in the resource pool associated with the non-preferred resource set. However, in 5.22.1.1, the UE behaviour is not clear for this scenario. As RAN1 had not specified the UE behaviour in this scenario, it is better to leave it up to UE implementation. NOTE X: If sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is configured by RRC and when the UE does not have own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if only a non-preferred resource set is received from a UE, it is up to the UE implementation to determine either to perform random resource selection or to use the non-preferred resource set in its resource (re)selection for transmissions. 2. In RAN2#118e, it was agreed that “TX UE remains active for RRC reconfiguration complete/failure sidelink reception (only for initial RRC reconfiguration sidelink case). If TX UE already applies SL DRX configuration in the direction (RX UE -> TX UE), TX UE follows the current SL DRX configuration.”. However, the second half of this agreement is not covered by current spec, which should be added. NOTE: If the UE transmitting SL-SCH Data already applies SL DRX configuration in the direction from the UE receiving SL-SCH Data to the UE transmitting SL-SCH Data, the UE transmitting SL-SCH Data follows the current SL DRX configuration. | When it comes to the correction 1/2, rapporteur understands the proponent’s observation. We can hear from companie’s views whether to support normative text based correction (i.e., specify explict UE behaviour) or NOTE based correction (i.e, leave it to UE implementation) at the CR discussion. |
| R2-2300488 | GC and BC transmission for IUC information | Huawei | Proposal 1: If a GC/BC destination selected in the Logical Channel Prioritization procedure has the highest priority logical channel, this destination can be included for the GC/BC transmission of condition-based IUC information MAC CE with non-preferred resource set. Proposal 2: RAN2 to agree the TP to TS 38.321 on GC/BC transmission for IUC information. Correction: 1> else: 2> select a Destination associated to one of unicast, groupcast and broadcast, that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI: 3> SL data is available for transmission; and 3> SBj > 0, in case there is any logical channel having SBj > 0; and 3> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and 3> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and 3> sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI. 2> if a Destination associated to groupcast or broadcast is selected and is having the highest priority logical channel, this Destination can be used to transmit the condition-based IUC information MAC CE with non-preferred resource set. NOTE 1: If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority or if multiple Destinations have either the MAC CE and/or the logical channels satisfying all conditions above with the same priority as the MAC CE, which Destination is selected among them is up to UE implementation. 1> select the logical channels satisfying all the following conditions among the logical channels belonging to the selected Destination: 2> SL data is available for transmission; and | Rapporteur understand the motivation of the proposal, but we need to discuss whether the related text should specified in the LCP procedure. From a CR reporter's point of view, I prefer to leave it to the UE implementation to minimize the impact on the specification, and to specify it as a NOTE approach in 5.22.1.1. Proposal can be discussed with R2-2301724 and other related contribution. |
| R2-2300757 | Discussion on IUC broadcast and groupcast | Apple | Proposal 1 When UE A generating non-preferred resource set triggered by a condition other than explicit request, and the UE A has either sidelink GC or BC data to be sent, UE A piggyback the IUC MAC-CE with SL data in a groupcast or broadcast transmission. Proposal 2 When UE A generating non-preferred resource set triggered by a condition other than explicit request and the UE A has no SL GC/BC data, UE A transmits IUC-Information MAC-CE to every GC/BC L2 ID(s) configured. Proposal 3 Src L2 ID is self-chosen by UE A in IUC GC/BC if the IUC MAC CE is not piggybacked with other SL data. Proposal 4 RAN2 discuss whether to introduce UE capability for stand-alone transmission of IUC GC/BC. Proposal 5 RAN2 send a reply LS to RAN1 to inform the agreed RAN2 approach of IUC GC/BC support. | Proposal (IUC broadcast and groupcast) can be discussed with R2-2301724 and other related contribution. |
| R2-2300896 | Discussion on the cast type of IUC scheme 1 | CATT | Proposal 1: For all SL MAC CEs including the Inter-UE Coordination Information MAC CE triggered by condition in IUC scheme 1, only unicast is applied. Proposal 2: Send LS to RAN1 to clarify that for the Inter-UE Coordination Information MAC CE carrying the non-preferred resource set which is triggered by condition in IUC scheme 1, only unicast is applied. | IUC is a RAN1 leading item, and RAN1 decided to support GC/BC in condition based IUC. Therefore, RAN2 should specify RAN1 agreement into related RAN2 specifications. |
| R2-2301473 | BC/GC for IUC transmission | Samsung | [Proposal 1]: Cast type selection and L2 source/destination id selection for IUC transmission triggered by a condition in scheme 1 are up to UE implementation. | Proposal (IUC broadcast and groupcast) can be discussed with R2-2301724 and other related contribution. |
| R2-2300838/R2-2300839 | Discussion on the remaining issues for NR sidelink | Xiaomi | **R2-2300838** Proposal 1: RAN2 to further discuss the higher layer impacts to support GC/BC for IUC and determine whether to support GC/BC for IUC or not. Proposal 2: RAN2 agree to capture that UE performs random resource selection without considering non-preferred resource set when the UE does not have own sensing result and if only a non-preferred resource set is received. Proposal 3: RAN2 does not agree to separate the delivery of non-preferred resource set to PHY from the resource selection procedure. **R2-2300839 (CRs)** 1. The corresponding behaviour to stop, if any, ongoing Random Access procedure due to a pending SR for SL-DRX command under some conditions is missing in the current specification. The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for SL-BSR and/or SL-CSI reporting and/or SL-DRX command indication, which was initiated by the MAC entity prior to the sidelink MAC PDU assembly and which has no valid PUCCH resources configured, if: - a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes an SL-BSR MAC CE which contains buffer status up to (and including) the last event that triggered an SL-BSR (see clause 5.22.1.6) prior to the MAC PDU assembly; or - the SL grant(s) can accommodate all pending data available and/or SL-CSI reporting MAC CE and/or SL-DRX command indication for transmission. 2. RAN1 has confirmed the scenario where UE-B receives non-preferred resource set from UE-A but UE-B has no sensing result is valid. However, the corresponding UE behaviour for this case is still missing in the MAC specification and should be captured. 3> if sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is configured by RRC and when the UE does not have own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a non-preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity from the resources pool, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 3. In this release, UE can create a selected sidelink grant on the pool of resources based on random selection, or partial sensing, or full sensing or any combination(s). However the combination case is missing in NOTE 1. NOTE 1: If the MAC entity is configured with Sidelink resource allocation mode 2 to transmit using a pool of resources in a carrier as indicated in TS 38.331 [5] or TS 36.331 [21], the MAC entity can create a selected sidelink grant on the pool of resources based on random selection, or partial sensing, or full sensing or any combination(s) only after releasing configured sidelink grant(s), if any. | **R2-2300838** Proposal 1 (IUC broadcast and groupcast) can be discussed with R2-2301724 and other related contribution. When it comes to P2, RAN2 can discuss this proposal/correction with related contribution (R2-2300487_Huawei). When it comes to P3, RAN2 can discuss this with related contribution (e.g., R2-2300755_Apple) whether to agree separate the delivery of non-preferred resource set to PHY from the resource selection procedure or not. **R2-2300839 (CRs)** Proponent’s observation is correct. Correction 1 can be merged to Rapporteur’s CR during the CRs discussion. When it comes to correction 2, this correction can discuss with related contribution/correction (R2-2300487_Huawei) during the CR discussion. When it comes to correction 3, proponent’s observation is correct and proposed correction can be merged to rapporteur CR at the CR discussion. |
| R2-2300895 | Correction on SL IUC Information and Request MAC CE | CATT | In current RAN2 specification for SL Inter-UE Coordination Information MAC CE and Inter-UE Coordination Request MAC CE, some field, including RSL, LSIi, RCi and First resource locationi-1 for IUC Information MAC CE and RP, RSWL and Number of Subchannel for IUC Information Request MAC CE, are indicated as the codepoint value in SCI format. However,if the length of the field in SCI is shorter than the corresponding field length in MAC CE. In this case, the field in MAC CE will contain the field in SCI using the LSB bits, but how to set the other bits are not described in the spec. This should be clarified. #### 6.1.3.53 Inter-UE Coordination Information MAC CE The Inter-UE Coordination Information MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.4-1. The priority of the Inter-UE Coordination Information MAC CE is fixed to '1'. It has a variable size with following fields: - RT: This field indicates the resource set type, i.e., preferred resource set or non-preferred resource set, as the codepoint value of the SCI format 2-C resourceSetType field as specified in TS 38.212 [9]. - RSL: This field indicates the location of reference slot, as the codepoint value of the SCI format 2-C referenceSlotLocation field as specified in TS 38.212 [9]. The length of the field is 17 bits. If the length of referenceSlotLocation field in SCI format 2-C as specified in TS 38.212 [9] is shorter than 17 bit, this field contains referenceSlotLocation field using the LSB bits, the other bit(s) in this field is set to zero; | Rapporteur understood the motivation of the correction. The correction can be discussed at the CR discussion. |
| R2-2300912 | Miscellaneous Correction on MAC for IUC | ZTE | #### Correction 1. Correct the specification reference for IUC #### 5.22.1.9 IUC-Request transmission The Sidelink Inter-UE Coordination Request transmission procedure is used to trigger a peer UE to transmit Sidelink Inter-UE Coordination Information as specified in clause 8.4.1.3 of TS 38.212 [9]. #### Correction 2. Support GC/BC IUC #### 5.22.1.10 IUC-Information Reporting The Sidelink Inter-UE Coordination Information reporting procedure is used to provide a peer UE with inter-UE coordination information as specified in clause 8.1.4A of TS 38.214 [7]. The Sidelink Inter-UE Coordination reporting procedure can be triggered by Sidelink Inter-UE Coordination Request MAC CE or condition (See TS 38.331 [5] and TS 38.214 [7]).Unicast for preferred resource set and non-preferred resource set and Groupcast/Broadcast for non-preferred resource set are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception. | Correction 1 (i.e., fix the reference) can be merged into Rapporteur CR at the CR discussion. Correction 2 (IUC broadcast and groupcast) can be discussed with R2-2301724 and other related contribution. |
| R2-2300913 | Correction on restriction of using IUC information | ZTE | According to current spec, UE will take IUC information into account during selecting the transmission resource, i.e. select the intersection resource between preferred resource set and sensing result. As far as we know, IUC information does not include the resource pool information. Resource pool information of IUC information is indicated implicitly. According to following agreement, from UE-A’s perspective, the IUC information is transmitting on the resource pool on which the IUC information is located. **RAN1 agreement** • For inter-UE coordination information triggered by an explicit request in Scheme 1, − UE-A uses a TX resource pool used for UE-B’s request transmission to determine the set of resources and to transmit the set of resources to UE-B • For inter-UE coordination information triggered by a condition rather than request reception in Scheme 1, - UE-A transmitting in a resource pool provides inter-UE coordination information associated with the same resource pool Then, from UE-B’s perspective, the IUC information is only used for the resource pool from which the IUC information is received. In other words, the IUC information can only be used when the UE selects the resource from the resource pool where the IUC information is received. | Rapporteur understands the motivation of the correction. However, I'm not sure if this restriction should be explicitly specified in the MAC specification. We can hear from companie’s views during the CR discussion whether this correction is necessary. |
| R2-2301353 | IUC open issues | Nokia | #### Proposal 1: RAN2 to agree that the UE-B should be allowed to perform resource exclusion, even though no sensing results are available. #### Proposal 2: RAN2 to agree to split the current specification text into separate cases for UE B receiving a preferred or non-preferred resource set, in a backwards compatible way. #### Proposal 3: RAN2 to agree to capture the case when UE B has no sensing result and the non-preferred resource set has been received. #### Proposal 4: RAN2 to agree to include the first change from R2-2211694 along with a revision of the current specification text as suggested in proposal 2. | P1/P3 can be discussed with R2-2300487 and R2-2300838. When it comes to P2/P4, RAN2 can discuss this with related contribution (e.g., R2-2300755_Apple, handling the non-preferred resource set) whether to agree separate the delivery of non-preferred resource set to PHY from the resource selection procedure or not. |
| R2-2301375 | Clarification on IUC related transmission | vivo | #### Correction 1. #### Reason for change: The IUC-Request transmission procedure in 5.22.1.9 and the IUC-Information Reporting procedure in 5.22.1.10 are based on clause 8.1.4 of TS 38.214. however, 8.1.4 of TS 38.214 is not related to the IUC-Request transmission and IUC-Information Reporting. It is about how the UE determine the set of preferred or non-preferred resources after receiving IUC information reporting. #### Proposed change: #### The Sidelink Inter-UE Coordination Request transmission procedure is used to trigger a peer UE to transmit Sidelink Inter-UE Coordination Information. Correction 2. #### Reason for change: #### A RRC parameter sl-LatencyBoundIUC-Report has been introduced to indicate the latency bound of SL Inter-UE coordination report from the associated SL Inter-UE coordination explicit request triggering. However, the one-to-one mapping relationship between the request signalling and IUC signalling should is missed in the specification: - After sending a request signaling, UE-B shall not send a new request signaling within the latency bound of IUC signaling transmission corresponding to the prior request, until it receives the IUC signaling form UE-A. - After receiving a request signaling, UE-A shall not transmit an IUC signaling which is not corresponding to the request within the latency bound. Proposed change: #### 5.22.1.9 IUC-Request transmission The Sidelink Inter-UE Coordination Request transmission procedure is used to trigger a peer UE to transmit Sidelink Inter-UE Coordination Information. The Sidelink Inter-UE Coordination Request Information is transmitted when the condition as indicated by the higher layer parameter sl-TriggerConditionRequest is satisfied. The UE that sent an explicit inter-UE coordination request to a peer UE is not allowed to trigger another inter-UE coordination information request for the same peer UE within the latency bound as indicated by sl-LatencyBoundIUC-Report, until the reception of the inter-UE coordination information report from the same peer UE. #### 5.22.1.10 IUC-Information Reporting The Sidelink Inter-UE Coordination Information reporting procedure is used to provide a peer UE with inter-UE coordination information. The Sidelink Inter-UE Coordination reporting procedure can be triggered by Sidelink Inter-UE Coordination Request MAC CE or condition (See TS 38.331 [5] and TS 38.214 [7]). RRC configures the following parameter to control the SL-IUC Information reporting procedure: - sl-LatencyBoundIUC-Report, which is maintained for each PC5-RRC connection. If the UE is triggered to report sidelink inter-UE coordination information by an explicit request, the UE is not allowed to report another sidelink inter-UE coordination information based on a condition other than the explicit request reception from the same peer UE within the latency bound as indicated by sl-LatencyBoundIUC-Report. | Rapporteur has no objection to the correction 1. If there is no objection to the correction at the CR discussion, this correction can be merged into the rapporteur CR. Rapporteur has no objection to the correction 2. If there is no objection to the correction at the CR discussion, this correction can be merged into the rapporteur CR. |
| R2-2301531 | Correction on IUC request MAC CE | ASUSTeK | #### Reason for change: In MAC specification, how to set the priority field indicated in the IUC request MAC CE is not clear in the description. In RAN1#107 and 107bis, agreements were made that UE-B’s request includes Priority value, subchannels, and selection window to be used for PSCCH/PSSCH transmission: #### Therefore, it is necessary to specify how to set the priority field based on the RAN1 agreement, otherwise it would be unknown for the UE on how to set the field values, and similar changes should also be applied to description of RSWL and Number of Subchannel in the MAC CE. #### Proposed change: #### 6.1.3.54 Inter-UE Coordination Request MAC CE #### The Inter-UE Coordination request MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.4-1. The priority of the Inter-UE Coordination Request MAC CE is fixed to '1' for Logical Channel Prioritization (LCP) procedure. It has a fixed size of 48 bits with following fields: - Priority: This field indicates the highest priority of the logical channel with SL data available to be transmitted to a peer UE, as the codepoint value of the SCI format 2-C priority field as specified in TS 38.212 [9]. The length of the field is 3 bits; - RSWL: This field indicates resource selection window location for SL data available to be transmitted to a peer UE, as the codepoint value of the SCI format 2-C resourceSelectionWindowLocation field as specified in TS 38.212 [9]. The length of the field is 34 bits. If the length of resourceSelectionWindowLocation field in SCI format 2-C as specified in TS 38.212 [9] is shorter than 34 bit, this field contains resourceSelectionWindowLocation field using the LSB bits; - Number of Subchannel: This field indicates the number of subchannels for SL data available to be transmitted to a peer UE, as the codepoint value of the SCI format 2-C numberOfSubchannel field as specified in TS 38.212 [9]. The length of the field is 5 bits. If the length of numberOfSubchannel field in SCI format 2-C as specified in TS 38.212 [9] is shorter than 5 bit, this field contains numberOfSubchannel field using the LSB bits; - R: Reserved bit, set to 0. | The text of the current MAC specification reflects the RAN1 agreement well enough and is clear. I don't think any further corrections related to the proposal are necessary. RAN2 can discuss the correction at the CR discussion. |
| R2-2301620 | Correction on number of MAC CEs in a MAC PDU | Sharp | #### Reason for change: In Rel-16 only one MAC CE (i.e. Sidelink CSI Reporting MAC CE) was specified for transmission over NR sidelink, and, “a MAC CE” was used when stating whether the MAC CE is included in a MAC PDU. However, in Rel-17, more MAC CEs (e.g. IUC Information MAC CE) were specified for transmission over NR sidelink. Therefore, the above-mentioned wording needs to be updated accordingly. | Proponent’s observation is correct. Correction can be merged to Rapporteur’s CR at the CRs discussion. |
| R2-2301745 | User plane corrections on NR Sidelink enhancements | LG | #### *Correction 1* #### RAN2 should specify the RAN1 agreement for IUC GC/BC support in the relevant RAN2 specification (i.e., TS 38.321). #### Proposed change: #### NOTE 3B7: The UE can use a groupcast, broadcast and unicast related Destination Layer-2 ID that are already in use or interested in transmitting and receiving Inter-UE Coordination message, and which Destination Layer-2 ID to select from the groupcast, broadcast and unicast related Destination Layer-2 that are already in use or interested is up to UE implementation. #### *Correction 2* #### Reason for change: According to the NOTE below of the current MAC specification, only DST ID is considered in MAC filtering for UC-related first TB. #### - “NOTE: If this TB is associated to unicast and this TB is the first TB of a logical channel which associated LCID is equal to 0 or 1, and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, deliver the decoded MAC PDU to the disassembly and demultiplexing entity. Whether the TB is the first TB can be determined based on the Source Layer-2 ID and Destination Layer-2 ID pair.” #### However, in the DRX operation of the current MAC specification, even if RX UE receives UC-related first TB, it also checks SRC ID to start Inactivity timer. Therefore, a new NOTE statement can be added to match the MAC filtering procedure and the DRX timer procedure. #### Proposed change: 4> start or restart sl-drx-InactivityTimer for the corresponding Source Layer-2 ID and Destination Layer-2 ID pair in the first slot after SCI reception. #### NOTE: If a TB is associated to unicast and this TB is the first TB of a logical channel which associated LCID is equal to 0 or 1, start or restart sl-drx-InactivityTimer for the corresponding Source Layer-2 ID and Destination Layer-2 ID pair in the first slot after SCI reception. #### *Correction 3* #### Reason for change: According to the RAN1 agreement, when re-evaluation/pre-emption/conflict indicator (IUC scheme 2) based resource re-selection is triggered, the MAC layer should perform the following two behaviours on the MAC specification. #### - When resource re-selection is triggered, if there is a received preferred resource set, the MAC layer uses it to perform resource re-selection. #### - When resource re-selection is triggered, the MAC layer provides the received non-preferred resource set information to the physical layer. #### RAN1 made the agreements to apply to all common cases (i.e., Scenarios in which resource re-selection based on pre-emption/re-evaluation/conflict indication occurs) where resource re-selection occurs. In other words, above RAN 1 agreements are not limited to a specific case where resource re-selection occurs. #### So, normative text for IUC procedure (i.e., “IUC procedure when re-evaluation/pre-emption/conflict indicator (IUC scheme 2) based resource re-selection is triggered”) should be added in the section 5.22.1.2a/5.22.1.2b. #### *Correction 4* #### Reason for change: According to the RAN1 agreement when UE-B receives both a preferred resource set and a non-preferred resource set from the same or different UE-A, the resource selection procedure (i.e., Resource selection procedure when UE-B decide to use the preferred resource set) should be additionally captured in the MAC specification. #### So, normative text for IUC procedure (i.e., “Resource selection procedure when UE-B decide to use the preferred resource set when both preferred resource set and non-preferred resource are received from a UE or different UEs:”) should be added in the section 5.22.1.1. | Correction 1 (IUC broadcast and groupcast) can be discussed with R2-2301724 and other related contribution. Correction 2 is the text for a missing UE behaviour for SL DRX Inactivity timer. RAN2 can discuss the correction at the CR discussion. Correction 3/4 is the missing UE behaviours about IUC related RAN1 agreement. Correction 3/4 can be discussed at the CR discussion. |

Proposal: Discuss the proposals and CRs listed in Table 1 offline, the agreed changes for MAC spec could be merged into Rapporteur’s CR.

Conclusions

The contributions submitted to AI 6.10.2 are summarized, Rapporteur’s recommendations are as follows.

Proposal: Discuss the CRs listed in Table 1 offline, the agreed changes for MAC spec could be merged into Rapporteur’s CR.