3GPP TSG RAN WG2#121	R2-2301933

Athens, Greece, 27th February - 3rd March, 2023

3GPP TSG-SA WG2 Meeting #155	S2-2303381

February 20 – 24, 2023, Athens, Greece

Title:	Reply LS on Differentiation of Layer2 ID and Coexistence of U2N/U2U

Response to:	LS on Differentiation of Layer2 ID and Coexistence of U2N/U2U (S2-2302187/R2-2213328)

Release:	Rel-18

Work Item:	5G_ProSe_Ph2, NR_SL_relay_enh-Core

Source:	SA2

To:	RAN2

Cc:

Contact Person:

#### Name:	Qiang Deng

#### E-mail Address:	dengqiang1@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 would like to thank RAN2 on the LS on Differentiation of Layer2 ID and Coexistence of U2N/U2U (S2-2302187/R2-2213328).

SA2 has discussed the questions raised by RAN2 and would also like to provide the answers as below.

Question1:

Can a UE be involved in acting as both U2N (as U2N Relay or Remote UE) and U2U (as U2U Relay or Remote UE) at the same time?

Answer 1:

Yes. A UE can take any role as above based on its 5G ProSe Capability and configured/provisioned 5G ProSe policy/parameters.

Question2:

Can the L2ID used for U2U communication be the same as the L2ID for U2N services?

Answer 2:

No. The Source Layer-2 IDs used for 5G ProSe UE-to-UE Relay Communication would be different from the Source Layer-2 IDs used for 5G ProSe UE-to-Network Relay Discovery and Communication.

Question3a:

Whether the Layer2 ID of the relay UE can be the same for U2U Relay service and Non-relay service?

Answer 3a:

No. The Source Layer-2 ID of 5G ProSe UE-to-UE Relay used for 5G ProSe UE-to-UE Relay Discovery and Communication would be different from the Source Layer-2 IDs used for Non-relay discovery and communication.

Question3b:

Whether the Layer2 ID of the remote UE can be the same for U2U Relay service and Non-relay service?

Answer 3b:

No. The Source Layer-2 IDs of 5G ProSe End UE used for 5G ProSe UE-to-UE Relay Discovery and Communication would be different from the Source Layer-2 IDs used for Non-relay discovery and Communication.

Question4a:

Whether the Layer2 ID of the relay UE can be the same for U2U Relay communication and U2U Relay discovery transmission?

Answer 4a:

No. The Source Layer-2 ID used for 5G ProSe UE-to-UE Relay Discovery would be different from the Source Layer-2 ID used for 5G ProSe UE-to-UE Relay Communication for a 5G ProSe UE-to-UE Relay.

Question4b:

Whether the Layer2 ID of the remote UE can be the same for U2U Relay communication and U2U Relay discovery transmission?

Answer 4b:

No. The Source Layer-2 ID used for 5G ProSe UE-to-UE Relay Discovery would be different from the Source Layer-2 ID used for 5G ProSe UE-to-UE Relay Communication for a 5G ProSe End UE.

2. Actions:

To RAN2:

ACTION: 	SA2 kindly asks RAN2 to take the above answers into account.

3. Date of Next TSG-SA WG2 Meetings:

TSG SA WG2 Meeting #156	17 - 21 April 2023		E-meeting

TSG SA WG2 Meeting #157	22 - 26 May 2023		Berlin, DE