3GPP TSG-RAN WG2 Meeting # 121	R2-2301976

# Athens, Greece, 2023-02-27 – 2023-03-03                                          (rev.of R2-2301863)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 1381 | rev | 2 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | Χ | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction related to AS deactivation due to discontinuous coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom, Νokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, Thales, Vodafone, Telit, BT, Telstra, Telecom Italia, Turkcell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2023-03-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current statement in Section 23.21.3 Support of Discontinuous Coverage “When out of coverage, the UE is not required to perform AS functions.” is ambiguous and introduces an unintended impact on legacy behaviour related to TN case, wrongly implying UE out of NTN coverage would not be required to perform AS functions related to TN. Therefore, in order to solve this ambiguity and keep the long-lasting legacy UE behaviour in this regard, we see the need for a correction on this Stage 2 aspect to align with the Stage 3 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that in case of out of coverage, the UE is not required to perform AS functions related to NTN, while the long-lasting legacy UE behaviour for performing AS functions towards TN is kept unchanged. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-2 is ambiguous for UE behaviour in case of out of coverage (of NTN). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 23.21.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.304 CR0862 |  |  |
| affected: |  |  | χ | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | χ | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 23.21.3	Support of discontinuous coverage

As an NTN payload moves on a specified orbit, for example in case of a NGSO satellite, the NTN payload beam(s) coverage area may move and cover different portions of a geographical area due to the orbital movement of the NTN payload. As a consequence, a UE located in the concerned geographical area may experience a situation of discontinuous coverage, due to e.g., a sparse satellite constellation deployment.

To enable the UE, in RRC_IDLE, to save power during periods of no coverage, the network provides NTN payload assistance information (e.g., ephemeris parameters, the start-time of upcoming NTN payload coverage) to enable the UE to predict when coverage will be provided by upcoming NTN payloads. Predicting out of coverage and in coverage is up to UE implementation.